3GPP TSG-SA3 Meeting #100e 	S3-202645r1

- e-meeting, 17 - 28 August 2020													Revision of S3-202645

- Source:	Qualcomm Incorporated

- Title:	Some evaluation of solution #2.3 in TR 33.846

- Document for:	Approval

Agenda Item:	2.5

# 1	Decision/action requested

This contribution proposes some analysis for solution #2.3 in TS 33.846.

# 2	References

[1]			3GPP TS 33.846 v0.7.0

# 3	Rationale

The solution only protects against the linkability attacks based on recognising the difference between the different authentication responses (see key issue which discusses other types of linkability attack).

It is proposed to add this to the evaluation of the solution

# 4	Detailed proposal

It is proposed that SA3 include the below pCR for inclusion into the TR [1].

**** START OF CHANGES ****

- 6.2.3.3	Evaluation

This solution enables the network to mitigate one kind of  Linkability attacks which are based on recognising the different authentication responses.

When the UE receives the authentication request message, UE generates authentication response with unified format for successful authentication, MAC failure and sync failure. Successful authentication can be distinguished through verification of RES* in AMF. MAC failure and sync failure can be differentiated through verification of AUTS in UDM. This solution modifies the authentication response message.

The attackers are unable to get the failure type with the authentication response message. This solution impacts the visited network.

**** END OF CHANGES ****