- 3GPP TSG SA WG3 (Security) Meeting #100bis-e	draft_S3-202648-r1

- e-meeting, 12 – 16 October 2020	revision of S3-20xyzw

- Source:	Qualcomm Incorporated

- Title:	Solution for secure PC5 link establishment for UE-to-network relay

- Document for:	Approval

Agenda Item:	2.9

# 1	Decision/action requested

This contribution proposes a solution for PC5 link establishment for UE-to-network relay.

# 2	References

[1]	TR 33.847 v0.1.0

# 3	Rationale

This contribution proposes a solution for secure PC5 link establishment for UE-to-network relay to address the KI #3.

# 4	Detailed proposal

It is proposed that SA3 approve the below pCR for inclusion in the TR [1].

***** START OF CHANGES *****

## 6.Y	Solution #Y: PC5 link setup for UE-to-network relay

### 6.Y.1	Introduction

This solution addresses the KI #3. This solution provides a mechanism to setup a PC5 link between a remote UE and UE-to-network relay. This solution assumes 5G Prose function and Prose Key Management Function as in LTE Prose. This solution only describes the PC5 link setup procedure that is common for both L2 and L3 UE-to-network relay.

### 6.Y.2	Solution details

Figure 6.Y.2-1: Secure PC5 link establishment procedure for UE-to-network relay

NOTE: The Remote UE needs to be in coverage to perform steps 0a-d and 1a/b. How often steps 0a-d and 1a/b need to be performed is configured as part of the Prose security policy. The UE-to-Network relay is assumed to be in coverage for all steps of the procedure.

0a-d. The Remote UE and the UE-to-network (U2N) relay get the discovery parameters and Prose Key management function (PKMF) address from the 5G Prose function and the discovery security material from the PKMF respectively.

1a. The Remote UE sends a Prose Remote User Key (PRUK) Request message to the PKMF of the UE-to-network relay.

1b. The ProSe Key Management Function checks that the Remote UE is authorised to receive UE-to-network Relay service. If the Remote UE is authorised to receive the service, the PKMF sends a PRUK and PRUK ID to the Remote UE.

2. The discovery procedure is performed between the Remote UE and the UE-to-network Relay using the discovery parameters and discovery security material.

3. The Remote UE sends a Direct Communication Request that contain the PRUK ID, Relay Service Code (RSC) of the UE-to-network relay service and KNRP freshness parameter 1.

4a. The UE-to-network relay sends a Key Request message that contains PRUK ID, RSC and KNRP freshness parameter 1 to the PKMF.

4b. The PKMF generates KNRP freshness parameter 2 and derives KNRP using PRUK identified by PRUK ID, KNRP freshness parameter 1 and KNRP freshness parameter 2. sends a Key Response message that contains KNRP and KNRP freshness parameter 2 to the UE-to-network relay.

5a. The UE-to-network relay sends a Direct Security Mode Command message to the Remote UE (see 6.5.2.2). This message contains the KNRP Freshness Parameter 2 and protected based on the session key (KNRP-SESS) derived from KNRP.

5b. The Remote UE derives KNRP from its PRUK, RSC, KNRP Freshness Parameter 1 and the received KNRP Freshness Parameter 2. It then derives the session key (KNRP-SESS) in the same manner as the UE-to-network relay and processes the Direct Security Mode Command. Then, the Remote UE responds with a Direct Security Mode Complete message to the UE-to-network relay.

6. The remote UE and UE-to-network relay continues the rest of procedure for the relay service over the secure PC5 link.

NOTE: the rest of procedure is determined depending on the UE-to-network relay types (i.e., L2 or L3 relay).

## 6.Y.3	Solution Evaluation

TBD.

***** END OF CHANGES *****