3GPP TSG-SA3 Meeting #100bis-e 	S3-202676

- e-meeting, 12-16 October 2020													revision of S3-202306

- Source:	NCSC, Ericsson

- Title:	New Evaluation for Solution 3

- Document for:	Approval

Agenda Item:	2.6

# 1	Decision/action requested

It is requested that SA3 approves this evaluation of a solution for protecting the long term key during transfer out of the UDR

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

# 3	Rationale

Solution 3 evaluation.

# 4	Detailed proposal

START OF CHANGE 1

## 7.3	Solution 3: Protection of LTK over Nudr

### 7.3.1	Introduction

This solution addresses key issue 3 on "protection of long-term key during transfer out of UDR".

The solution is based on storing the long-term key encrypted in UDR and transferring it also encrypted over Nudr. As any other SBA reference point, Nudr is additionally protected using TLS as defined in 3GPP TS 33.501 [2].

This solution is based on capabilities defined or planned to already to be defined in 3GPP TSs and does not require any additional specification work.

### 7.3.2	Solution details

The OAuth 2.0 based authorization framework defined in 3GPP 33.501 [2] is being enhanced in Release 16 to allows the possibility to generate OAuth 2.0 tokens to retrieve AuthenticationSubscription data ONLY to UDM/ARPF NF type of service consumers.

This solution proposes that the protection of the long-term key while provided to the UDM/ARPF over Nudr is two-fold:

- In the first place, the long-term key is provisioned and stored in UDR in encrypted form. This solution does not cover actual mechanisms to perform such encryption (e.g. encryption algorithms, key length, etc …).

- Secondly, as any other SBA reference point, the Nudr is protected using TLS as defined in 3GPP TS 33.501 [2].

These two protection levels make impossible for any intermediate actor to eavesdrop or modify the long-term key while in transit over Nudr.

### 7.3.3	Evaluation

This solution addresses the requirements of the KI by protecting the transfer of the long-term key between the UDR and the UDM/ARPF in three ways:

-	transporting the long-term key in encrypted form during its transfer from UDR to UDM/APRF, and

-	additionally, protecting the transfer of the long-term key over Nudr based on secure encrypted transport mechanisms (such as HTTPS).

-	the OAuth tokens allow for the long-term key to only be retrieved by the UDM/ARPF

This solution requires that the UDM/ARPF stores the decryption key. The storage of the decryption key at the UDM/ARPF is subject to the same security requirements as if the ARPF would store the long-term keys. That is, the decryption key shall be protected from physical attacks and shall never leave the secure environment of the UDM/ARPF unprotected. This required security of the decryption key can be achieved as it is done in pre-5G networks (e.g. by using a Hardware Security Module in the UDM/ARPF). It may be desirable to export a protected copy of the decryption key to a backup location, to aid recovery if necessary.

END OF CHANGE 1