3GPP TSG-SA3 Meeting #100bis-e 	S3-202677

- e-meeting, 12-16 October 2020													revision of S3-202307

- Source:	NCSC

- Title:	New Evaluation for Solution 4

- Document for:	Approval

Agenda Item:	2.6

# 1	Decision/action requested

It is requested that SA3 approves this evaluation of a solution for protecting the long term key during storage in the UDR

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

# 3	Rationale

Solution 4 evaluation.

# 4	Detailed proposal

START OF CHANGE 1

## 7.4	Solution #4: Encrypted storage of the long-term key in the UDR

### 7.4.1	Introduction

This solution addresses key issue #2 on "protection of long-term key during storage in UDR".

In order to protect the long-term key during storage in the UDR, the long-term key is provisioned and stored in encrypted form in the UDR. During primary authentication, the UDM/ARPF retrieves the authentication subscription data for the UE which includes the encrypted long-term key as provisioned and stored in UDR. That is, the long-term key is never provided by the UDR in clear text and there is no need for the UDR to decrypt the long-term key.

At generation of a long-term key an encryption key is used that is shared with the UDM/ARPF where it is decrypted during primary authentication. The generation of a long-term key can be performed in the network of an operator or it can be performed at the facility where USIMs are being provisioned.

### 7.4.2	Solution details

The long-term key is provisioned and stored in encrypted form if stored in the UDR. The encryption algorithm used and the method for the handling of the encryption/decryption key(s) needs to conform to the security policy requirements of the operator. This solution recommends the use of NIST approved algorithms.

Editor’s note: the encryption algorithm used and the method for the handling of the encryption/decryption key(s) is FFS.

Editor's note: whether or not provisioning to ARPF or UDR is in 3GPP scope is FFS.

### 7.4.3	Evaluation

This solution addresses the requirements of the KI by protecting the long-term key in one way:

-	provisioning and storing the long-term key in encrypted in the UDR to UDM/APRF

This solution requires that the UDM/ARPF stores the decryption key. The storage of the decryption key at the UDM/ARPF is subject to the same security requirements as if the ARPF would store the long-term keys. That is, the decryption key shall be protected from physical attacks and shall never leave the secure environment of the UDM/ARPF unprotected. This required security of the decryption key can be achieved as it is done in pre-5G networks (e.g. by using a Hardware Security Module in the UDM/ARPF). It may be desirable to export a protected copy of the decryption key to a backup location, to aid recovery if necessary.

END OF CHANGE 1