3GPP TSG-SA3 Meeting #100bis-e 	S3-202680

- e-meeting, 12-16 October 2020													revision of S3-202307

- Source:	NCSC

- Title:	New solution for KI2: Storage of LTK in UDR

- Document for:	Approval

Agenda Item:	2.6

# 1	Decision/action requested

It is requested that SA3 approves this solution for protecting the storage of the long term key in the UDR

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

# 3	Rationale

A new solution is proposed for the protection of the long term key during storage in the UDR. The proposal is to trust OAuth 2.0 access tokens to restrict access to only NFs of type UDM/ARPF, and also the AuthenticationSubscription data type to restrict any modification to the long-term key to the sequenceNumber attribute. These restrictions would result in the long-term key being protected, and would avoid a bootstrap situation where the long-term key is encrypted with another key, which would itself then need to be protected to the same security level as the original long-term key, resulting in creating an equal problem to the one that we are trying to solve - it is bad practice to say a piece of data is encrypted without knowing how, as it gives the perception of security without providing any. This solution provides a simple option for protection of the long-term key that uses existing capabilities.

# 4	Detailed proposal

START OF CHANGE 1

## 7.x	Solution #<x>: Storage of the LTK in the UDR

### 7.x.1	Introduction

This solution addresses key issue 2, "protection of long-term key during storage in UDR ".

The solution trusts the access tokens created using the OAuth 2.0 based authorization framework to protect the long-term key from retrieval by unauthorised NFs, and that modification of the long-term key is restricted to the sequenceNumber attribute.

This solution is based on capabilities defined or planned to be defined in 3GPP TSs and does not require any additional specification work.

### 7.x.2	Solution details

The OAuth 2.0 based authorization framework defined in 3GPP 33.501, clause 13.4.1, is being enhanced in Release 16 to support the generation and validation of authorization tokens, including authorization at resource level. This allows the possibility of generating OAuth 2.0 access tokens to restrict retrieval of AuthenticationSubscription data to UDM/ARPF NF type service consumers only, preventing unauthorised access by another other NF type. Such access tokens can be required by the UDM to access the long-term key.

As defined in 3GPP TS 29.505, Clause 5.2.1, Table 5.2.1-1, any modification of the long term key with the AuthenticationSubscription data resource by the UDM/ARPF over Nudr is limited to the sequenceNumber attribute, and attempts to modify any other attribute shall be rejected by the UDR.

Editor’s Note: evaluation of this solution shall also consider the completeness of this solution against all threats to accessing the long-term key.

### 7.x.3	Evaluation

END OF CHANGE 1