3GPP TSG-SA3 Meeting #100bis	S3-202331

- e-meeting, 12 – 26 October 2020

- Source:	China Telecom

- Title:	key issue on security configuration differentiation in CU-UPs

- Document for:	Approval

Agenda Item:	2.13

# 1	Decision/action requested

This contribution proposes a proposal for the key issue of TR 33.840.

# 2	References

[1]	S3-201381, Study on security aspects of the Disaggregated gNB Architecture, China Telecom, China Unicom, CATT, Xiaomi, vivo

[2]	3GPP TR 38.823: "Study of Further Enhancement for Disaggregated gNB".

# 3	Rationale

It is proposed to add a new key issue of security configuration differentiation in gNB-CU-UPs.

# 4	Detailed proposal

*****Start of Change*****

X.X	Key Issue #X: Security configuration differentiation in gNB-CU-UPs

X.X.1 Issue details

Per TS33.501, the security requirements on gNB shall apply to all types of gNB. For CU-CP and CU-UP split scenario, each CU-UP in one gNB are not required to support same security capability/ algorithm priority. As described in TR38.823[xx], the CU-UPs could be configured with different security configuration irrespective of the location CU-UPs deployed. Based on the existing architecture for disaggregated gNB, the gNB-CU-CP selects the appropriate gNB-CU-UP(s) for the requested services for the UE. Therefore, it is need to support a case that the UE is connected to more than one gNB-CU-UPs providing different security algorithm or algorithm priority or security configuration simultaneously.

In addition, as specified in TS38.401[zz], one gNB consists of a gNB-CU and one or more gNB-DU(s) and one gNB-DU could be connected to multiple gNB-CU-UPs. Thus, this key issue assumes multiple gNB-CU-UPs can be mapped to one or more physical cells or group cells, i.e., MCG/SCG.

X.X.2 Security Threats

Not applicable.

X.X.3 Potential security requirements

5GS should support different user plane security protection options for the case when the same UE user plane is terminated in different gNB-CU-UPs.

Note 2: Potential solutions shall not compromise the requirement that the 5G RAN internal architecture (i.e., CP/UP separation and DU/CU split) is invisible to the UE.

*** END of CHANGE ***