3GPP TSG-SA3 Meeting #100e	S3-201702

- e-meeting, 17 – 28 August 2020

- Source:	China Telecom

- Title:	key issue on security policy differentiation in CU-UPs

- Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

This contribution proposes a proposal for the key issue of TR 33.840.

# 2	References

[1]	S3-201381, Study on security aspects of the Disaggregated gNB Architecture, China Telecom, China Unicom, CATT, Xiaomi, vivo

[2]	3GPP TR 38.823: "Study of Further Enhancement for Disaggregated gNB".

# 3	Rationale

It is proposed to add a new key issue of security differentiation in gNB-CU-UPs.

# 4	Detailed proposal

*****Start of Change*****

X.X	Key Issue #X: Security differentiation in gNB-CU-UPs

X.X.1 Issue details

In order to support vertical industry in 5G network, the operators and their customers/partners may adopt different security requirements for traffic protection for gNB-CU-UPs, e.g. different key, different security policy, different security algorithms. Therefore, it is need to support a case that the UE is connected to more than one gNB-CU-UPs providing different security algorithm or security policy simultaneously. And these gNB-CU-UPs may be deployed in different location or co-located. For example, the UEs connecting to the different gNB-CU-UPs with different security algorithms.

X.X.2 Security Threats

If the UE has activated the same security protection (e.g. the same algorithms) for user plane protection to all the gNB-CU-UPs, then if security context in one gNB-CU-UP is compromised, then this will compromise security contexts in other gNB-CU-UPs for the same UE.

X.X.3 Potential security requirements

5GS should support different security protection for user plane protection in different gNB-CU-UPs for the same UE.

*** END of CHANGE ***