3GPP TSG-SA3 Meeting #100e 	S3-201552

- e-meeting, 17 -28 August 2020

- Source:	KPN

- Title:	New Solution for KI3: Encrypted transfer of LTK between UDR and UDM/ARPF

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

It is requested that SA3 approves this solution based on encryption for protecting the long term key during transfer between the UDR and the UDM/ARPF

# 2	References

[1]	3GPP TS 33.845, Study on storage and transport of 5GC security parameters for ARPF authentication, v0.3.0

[2]	S3-201551, New solution for KI2: Encrypted storage of LTK in UDR, 3GPP SA3#100-e

# 3	Rationale

A new solution is proposed for the protection of the long term key during transfer from the UDR to the UDM/ARPF. The proposal is to use a an approved symmetric-key algorithm where the secret encryption/decryption key is shared between the UDM/ARPF and the source of the long term key (e.g. the party provisioning USIMs), based on the assumption that the long term key is already stored in encrypted form in the UDR (see new solution described in [2]). This would result a strong enough protection mechanism that does not require the storage of a decryption/encryption key in the UDR.

The solution allows the possibility of having multiple decryption/encryption keys by using a key identifier for identifying the appropriate key.

# 4	Detailed proposal

START OF CHANGE 1

## 7.x	Solution #<x>: Encrypted transfer of the long-term key between UDR and UDM/ARPF

### 7.x.1	Introduction

If stored in the UDR, the long-term key is always transported in encrypted form during its transfer from UDR to UDM/APRF.

### 7.x.2	Solution details

The long-term key is transferred in encrypted form between UDR and UDM/ARPF. The encryption algorithm used and the method for the handling of the encryption/decryption key(s) needs to conform to the security policy requirements of the operator. This solution recommends the use of NIST approved algorithms.

Editor’s note: the encryption algorithm used and the method for the handling of the encryption/decryption key(s) is FFS.

The transfer of the encrypted long-term key over the Nudr interface is protected at transport level using HTTPS as any other SBA interface.

During transfer, a key identifier is associated to the encrypted long-term key to enable identification of the decryption key in the UDM/ARPF.

7.x.3	Evaluation

Editor's note: to be provided.

END OF CHANGE 1