3GPP TSG-SA3 Meeting #100e 	S3-201615-r4

- e-meeting, 17 - 28 August 2020												  Merger of S3-201615 and S3-201631

- Source:	Nokia, Nokia Shanghai Bell, China Mobile

- Title:	Linking of UEs by SUCI replay attack

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

New key issue to address the potential linking of UEs by SUCI replay attack.

# 2	References

[1]	33.846

# 3	Rationale

It is proposed to introduce an additional key issue that addresses a linkage attack related to the SUCI. i.e. an attack by which it is possible to find out whether a UE observed at some location/time X is identical to a UE observed at some location/time Y.

# 4	Detailed proposal

****** START OF CHANGES

### 5.2.X	Key Issue #2.X: SUCI replay

#### 5.2.X.1	Key issue details

This key issue addresses SUCI replay attacks. A specific linkability linkage attack and a DoS attack related to SUCI replay are described in the following.

NOTE: KI#2.1 holds the related generic requirement for mitigation of linkability attacks, but this key issue holds specific requirements related to SUCI usage.

Linkability attack

The SUCI can be used for a linkability attack, i.e. an attack by which it is possible to find out whether a UE observed at some location/time X is identical to a UE observed at some location/time Y. For this, an attacker records a SUCI that is used over the radio interface by a UE_A. (As the UE uses a 5G-GUTI rather than a SUCI in most cases, the attacker may also execute an active attack, such as corrupting the 5G-GUTI when it is sent by the UE, which leads with a significant probability to an identity request, so the UE will send a SUCI subsequently in an identity response.)

When at some later time some UE_X makes a registration request to a false base station operated as a relay by the same attacker, the attacker can modify UE_X's registration request by exchanging the 5G-GUTI or SUCI used in this request by the previously captured SUCI of UE_A and forward the modified request to the network. Note, this is possible, because UE X's first message is unsecured pre-authentication traffic.

Subsequently, the attacker observes whether a successful AKA run is performed, and the registration request is accepted by the network. If so, then UE_X = UE_A.

Note that this attack cannot by mitigated by hiding only the content of the AKA response, because the attacker can detect from the subsequent messages (e.g. RRC SMC procedure, repetition of the AKA, or RRC release) whether the AKA run was successful or not. If the content of the AKA response is not hidden that could be used directly to determine the UE under attacke is present.

DoS attack

The current ECIES scheme is vulnerable to replay attacks because it does not have the mechanisms in the network side to justify whether the received SUCI was the previous one sent by the UE to the network or not.

If an attacker launches the replay attack multiple times, the UDM and the UE have to spend a lot of resources to process the replayed SUCI and the authentication request message respectively because these messages are legitimate. This raises a DoS attack on the UDM and the UE respectively.

#### 5.2.X.2	Potential threats

Linkability attack: An attacker observes whether a successful AKA run is performed with a replayed SUCI, i.e. if the registration request is accepted by the network. If so, the attacker can link a UE observed in one location with a UE observed in another location. If done at several location, even though UE may still be anonymous, the tracking profile of the same UE being in different location may compromise privacy.

Based on the response to the valid AUTN and RAND from the UE, the attacker could trace the UE whose SUPI is unknown to the attacker with some confidence.

DoS attack: Many times launches of the SUCI replay attack could lead to a DoS attack on the UDM and the UE respectively. A DoS attack on the UE may result in a decrease in the processing capability of the UE and a rapid consumption of the battery. A DoS attack on UDM will cause the processing power of the UDM to decrease and the response to the request of the legitimate UE to be slow.

#### 5.2.x.3		Potential security requirements

5G system shall provide mechanisms to mitigate the SUCI replay attacks.

****** END OF CHANGES