3GPP TSG SA WG3 Meeting #100e 	draft_S3-201714_r2

- e-meeting, 17-28 August 2020                                                         revision of S3-201714

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.536 | CR | 0011 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on processing NULL algorithms |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XARC |  |  |  |  |  |  | Date: |  | 2020-8-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to the 3GPP TS 38.323, the DRB, sidelink SRB and sidelink DRB shall not carry MAC-I if integrity protection is not activated. Furthermore, as specified in 3GPP TS 38.331, no security configurations are configured to PDCP layer if security protection is not activated. NULL security algorithm is included in the Chosen_algs parameter if security is not activated as specified in TS 33.536 v16.0.0. The content in PDCP packet needs clarification. |  |  |  |  |  |  |  |  |
| qu |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Carifies the content in the PDCP packet |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect PDCP contents in the PDCP packet |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.1.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

********** START OF 1st CHANGE **********

##### 5.3.3.1.5	Protection of the PC5 unicast link

5.3.3.1.5.1	General

Protection for the signalling and user plane data between the UEs is provided at the PDCP layer. As the security is not preserved through a drop of the connection, all signalling messages that need to be sent before security is established for a connection may be sent with no protection. The PC5-S signalling messages that can be sent and processed unprotected are given in TS 24.587 [8]. Once security is established for a connection all signalling messages for that connection are sent integrity protected and confidentiality protected with the chosen algorithms except the Direct Security Mode Command which is sent integrity protected only.

5.3.3.1.5.2	Integrity protection

UEs shall implement NIA0, 128-NIA1 and 128-NIA2 and may implement 128-NIA3 for integrity protection of the unicast link. The algorithm identifiers from clause 5.11.1.2 of TS 33.501 [6] are reused for PC5-S, PC5-RRC, and PC5-U.

These integrity algorithms are as specified in TS 33.501 [6] and are reused with the following modifications:

-	The key used is NRPIK;

-	Direction is set to 1 for direct link signalling transmitted by the UE that sent the Direct Security Mode Command for this security context and 0 otherwise;

-	Bearer[0] to Bearer[4] are set to 5 LSB of LCID;

-	COUNT[0] to COUNT[31] are filled with counter value (see clause 6.3.5 of TS 38.323 [9]).

NOTE:	The above input parameters do not apply to NIA0 as specified in Annex D.1 of TS 33.501 [6].

The receiving UE ensures that received protected signalling messages and user plane data that is integrity protected are not replayed.

5.3.3.1.5.3	Confidentiality protection

UEs shall implement NEA0, 128-NEA1 and 128-NEA2 and may implement 128-NEA3 for ciphering of the unicast link. The algorithm identifiers from clause 5.11.1.1 of TS 33.501 [6] are reused for PC5-S, PC5-RRC, and PC5-U.

These ciphering algorithms are as specified in TS 33.501 [6] and are used with the following modifications:

-	The key used in NRPEK;

-	Direction is set as for integrity protection (see 5.3.3.1.5.2);

-	Bearer[0] to Bearer[4] are set to 5 LSB of LCID;

-	COUNT[0] to COUNT[31] are filled with counter value.

NOTE:	The above input parameters do not apply to NIA0 as specified in Annex D.1 of TS 33.501 [6].

5.3.3.1.5.4	Content of the PDCP packet

The Key ID and least significant bits of the counter are carried in the PDCP header, along with any MAC that is needed for integrity protection if integrity protection is activated. The key ID is used to signal which security context is being used and shall be set to KNRP-sess ID. The payload field and MAC (if required) fields are ciphered if confidentiality protection is activated.

This is illustrated in Figure 5.3.3.1.5.4-1.

KNPR-sessID

LSBs of counter

payload Payload (Ciphered ciphered if confidentiality protection is activated)

Ciphered MAC (if required, ciphered if applicable)

Figure 5.3.3.1.5.4-1: Security parameters in the PDCP header for NR based PC5 unicast mode

********** END OF 1st CHANGE **********