- 3GPP TSG SA WG3 Meeting #100e                            	draft-S3-201735-r4

- e-meeting, 17 – 28 August 2020	                         revision S3-20abcd

- Source:	Huawei, Hisilicon

- Title:	System Information Protection using On-boarding Credential in NPN

- Document for:	Approval

Agenda Item:	5.2

# Decision/action requested

This contribution proposes a new solution for key issue 2 in TR 33.809

# References

# Rational

The contribution proposes a new solution for key issue #2 ”Security protection of system information”.

# Detailed proposal

*************** Start of 1st Change ****************

- 6.X	Solution #X: System Information Protection using On-boarding Credential in NPN

- 6.X.1	Introduction

This solution addresses key issue #2, and is only available for NPN scenario.

UE on-boarding and remote provisioning of NPN credentials is one of objective in FS_eNPN_SEC. The UE could use the method to obtain NPN credential. Since the NPN credential may include UE’s certificate and CA’s certificate, the RAN can sign the broadcast message with its private key relevant to RAN’s certificate signed by the CA’s private key, and the UE can verify the broadcast with the CA’s certificate.

- 6.X.2	Solution details

- 6.X.2.1	Credential provision

This solution is based on output of UE on-boarding solutions in FS_eNPN_SEC, if NPN credential is certificate based on PKI,

- The UE shall store CA certificate CAC via on-boarding method, and the CA belongs to a NPN.

- Each gNB in NPN should generate its private key, and provide related public key to CA in NPN to sign a RAN’s certificate RANC, e.g. using OAM via CMPv2 protocol.

- 6.X.2.2	Procedures

Broadcast protection in gNB side:

GNB broadcasts system informations, the broadcast includes broadcast content M1, signature and RANC. The signature is generated from the M1 and the private key relevant to RANC, the RANC contains the public key relevant to the private key and is signed by CA in NPN

Broadcast verification in UE side:

When the UE receives the broadcast, it shall first verify the validity of the RANC using the CAC. After successfully verifying the RANC, the UE uses public key in RANC to verify the signature. If the verification of signature succeed, the UE takes the broadcast genuine.

UE actions upon detection of invalid signature:

Each gNB used for NPN shall be provisioned with RANC.

The UE with CAC shall verify the broadcast, if the verification is failed, the UE drops the broadcast.

- 6.X.3	Assessment using Annex A.3

- 6.X.3.1a	UE aspects

UE needs to be on-boarding NPN credential, e.g. at least CA certificate. The on-boarding method and credential storage is based on output of FS_eNPN_SEC.

UE shall support verification of system information via CA certificate.

- 6.X.3.1b	UE actions upon detection of invalid signature

The action is depicted in 6.X.2.2.

- 6.X.3.2		Threats that are mitigated by signed SI messages

Key issue 2 are mitigated. All broadcast message in NPN can be signed using this solution after successful on-boarding.

- 6.X.3.3		Threats that are not mitigated by signed SI messages

- Editor’s Note: It is FFS whether or not this solution can sufficiently prevent replay attacks

- 6.X.3.4	Provisioning of keys

- Editor’s Note: Provisioning of keys is FFS

- 6.X.3.5	RAN aspects

The gNB in NPN shall support certificate provisioning method of NPN certificate, e.g. CMPv2. Since number of gNB in NPN is minimal, the configuration is not complicated.

The gNB in NPN shall support generation of the signature with the minimum SI broadcasted.

- 6.X.3.6	VPLMN aspects

For NPN, VPLMN and HPLMN are the same NPN.

The changes are based on on-boarding method based on output of FS_eNPN_SEC.

- 6.X.3.7	HPLMN aspects

For NPN, VPLMN and HPLMN are the same NPN.

The changes are based on on-boarding method based on output of FS_eNPN_SEC.

- 6.X.3.8	Network sharing aspects

gNB can broadcast a list of PLMN and NPN in the SIB message, if the gNB is shared gNB, it may carry more than one PLMN ID and NPN. In this case, the SI including NPN information is signed.

- 6.X.3.9	Roaming aspects

No consider in NPN.

- 6.X.3.10	Regulatory aspects

No consider in NPN.

- 6.X.3.11	Signature schemes

There could one or more signature schemes like:

-	null-scheme

-	ECDSA (recommended with named curves)

-	RSA

-	others

- 6.X.3.12	Signature length

ECDSA: 64 byte

RSA: 256 byte

- 6.X.3.13	Resistance against Quantum Computing

None identified as of now.

*************** End of 1st Change ****************