3GPP TSG-SA3 Meeting #100e 	S3-201758

- e-meeting, 17 – 28 August 2020														Merge of S3-201662,S3-202022, S3-202023, S3-201807

- Source:	Huawei, Hisilicon, LG Electronics, Ericsson, CATT, Interdigital

- Title:	New key issue on discovery message protection

- Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

NA

# 3	Rationale

Discovery message portion needs to be studied in SA3 due to the new 5G Prose architecture.

# 4	Detailed proposal

*************** Change 1***************

## X.Y	Key Issue #Y: Discovery message protection

### X.Y.1	Key issue details

The Open ProSe direct discovery procedure is used for a UE to discover or be discovered by other UE(s) in proximity over the PC5 interface. The UE can discover other UE(s) with interested application(s) and/or interested group(s) using the ProSe direct discovery procedure. In Open Discovery, a UE which wants to discover other UE’s does not require any explicit permission from the other UE’s in order to be allowed to discover them.

The Restricted ProSe direct discovery procedure is used for a UE to discover or be discovered by other UE(s) in proximity over the PC5 interface. In Restricted Discovery, a UE which wants to discover other UE’s requires an explicit permission from the other UE’s in order to be allowed to discover them.

There could be a case where a discoverer UE wantsintends to discover two different discoveree UEs (called discoveree UE A and discoveree UE B) at a time. With the existing ProSe architecture, the three UEs have to have the same security keys to support the case.

There riskis a vulnerability happens here with the model B architecture in this use case. Despite discoveree UE A allowing to be discovered by only the discoverer UE, discoveree UE B is also capable of detecting and decoding the discovery response message from discoveree UE A because it has the same keys, and thus discoverying discoveree UE A.

Hence it needs to be studied how to protect the discovery response message of a UE in a restricted direct discovery model B architecture mode from being discovered by other discoveree UE(s).

Editor’’s Note: Whether there is a case where a discoverer UE wants to discover more than one different discoveree UEs using same ProSe service at a time is FFS

- X.Y.2	Security threats

If the discovery message cannot be confidentiality protected, integrity protected, and replay protected, the Prose APP Code can be intercepted, modified, or replayed by an attacker. The announcing UE or discoveree UE may connect with a UE that is not interested in the that particular Prose service.

An attacker may impersonate the discoveree or the discovered UE.

A malicious application running on a discoveree UE can detect the response message from other discoveree UE(s) that uses the same security keys in restricted direct discovery model B architecture , so it canand may discover other discoveree UE(s), whichthat areis not supposed to be discoverable to it. This puts the privacy of other discoveree UE(s) at risk.

Editor’s Note: Whether this threat is valid is FFS.

### X.Y.3	Potential Security requirements

The discovery message in restricted discovery shall support confidentiality protection, integrity protection, and replay protection.

The entity which receives a restricted discovery message on the PC5 interface shall be able to verify the source authenticity.

Editor note: It is FFS in SA2 whether ProSe code or similar parameter will be used in discovery messages sent on PC5 interface. Any related privacy issues with the use of ProSe code or similar parameter needs to be further studied in restricted discovery.

The 5G System shall provide means to protect the discovery response message of a discoveree UE in the restricted direct discovery model B architecture from other discoveree UE(s).

Editor’s Note: In case that a discoverer UE wants to discover more than one different discoveree UEs using same ProSe service at a time, if security protection between different discoveree UEs is needed is FFS

********** End of Change **********