- 3GPP TSG-SA WG3 Meeting #100e 															  S3-201791

- e-meeting, 17 – 28 August 2020	                                                                 Revision of S3-20xxxx

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.535 | CR | 0022 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on AKMA Application Key derivation in the UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | AKMA |  |  |  |  |  |  | Date: |  | 2020-8-7 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The description in the current TS 33.535 is incomplete as lack of step and description to derive KAF in the UE. It is not conducive to the implementation of AKMA services. |  |  |  |  |  |  |  |  |
| qu |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the step of KAF generation in the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

********** START OF CHANGE **********

## 6.2	Deriving AKMA Application Key for a specific AF

Figure 6.2-1 shows the procedure used by the AF to request application function specific AKMA keys from 5GC directly, when the AF is located in the operator's network.

Figure 6.2-1: KAF generation from KAKMA

Before communication between the UE and the AKMA AF can start, the UE and the AKMA AF needs to know whether to use AKMA. This knowledge is implicit to the specific application on the UE and the AKMA AF.

1.	When the UE initiates communication with the AKMA AF, it shall include the derived A-KID in the Application Session Establishment request message (see clause 6.1).

2.	If the AF does not have an active context associated with the A-KID, then the AF sends a Naanf_AKMA_AFKey request to AAnF with the A-KID to request the AKMA Application Key for the UE. The AF also includes its identity (AF Id) in the request. The AAnF shall authorize AF. The AAnF shall check whether the AAnF can provide the service to the AF based on the configured local policy or based on the authorization information or policy provided by the NEF/NRF using the AF Id. If succeeds, the following procedures are executed. Otherwise, the AAnF shall reject the procedure.

The AAnF can check whether the subscriber is authorized to use AKMA by the presence of the AKMA anchor key K_AKMA that has been received from the AUSF.

If the AAnF is in possession of the AKMA Application Key (KAF), it responds to the AF with the KAF. If not, the AAnF shall check if it has the UE specific KAKMA key identified by the A-KID.

If KAKMA is available in AAnF, the AAnF shall continue with step3.

If KAKMA is not available, the AAnF shall continue with step 4 and send an error response.

3.	The AAnF derives the AKMA Application Key (KAF) from KAKMA.

The key derivation of KAF shall be performed using the key derivation function (KDF) specified in TS 33.220 [4]. KAF is computed (as per clause A.4) as KAF=KDF (KAKMA, AF_ID), where the AF_ID is constructed as follows: AF_ID = FQDN of the AF || Ua* security protocol identifier. The Ua* security protocol identifier is specified as Ua security protocol identifier in Annex H of TS 33.220 [4]. The key used for the derivation of KAF is KAKMA.

4.	The AAnF sends Naanf_AKMA_AFKey response to the AF with KAF and lifetime.

5.	The AF sends the Application Session Establishment reponse to the UE.

6. 	 The UE derives the AKMA Application Key (KAF) from KAKMA.

********** END OF CHANGE **********