3GPP TSG-SA3 Meeting #100e 	S3-201854

- e-meeting, 17-28 Augest 2020													Revision of S3-20xxxx

- Source:	Huawei, Hisilicon

- Title:	IMS SCAS: new test case on de-registration

- Document for:	Approval

Agenda Item:	4.15

# 1	Decision/action requested

This contribution proposes a new test case on the de-registration scenario.

# 2	References

[1]	3GPP TS 33.203	3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G security;Access security for IP-based services

[2]		S3-20xxxx, IMS SCAS: adding threats related to de-registration during the authentication

# 3	Rationale

As specified in TS 33.203 [1], clause 6.1.1, S-CSCF will not de-registration the UE when registered, if it fails an authentication. Otherwise, denial of service attack may be initiated by an attacker to deregister a legal user.

The threats analysis can be found in the S3-20xxxx [2].

# 4	Detailed proposal

It is suggested to approve the following change.

*************** Start of the changes ****************

#### 4.2.2.Y	Security functional requirements on the S-CSCF deriving from 3GPP specifications and related test cases

##### 4.2.2.Y.1	No de-registration during the authentication

Requirement Name: No de-registration during the authentication

Requirement Reference: TS 33.203 [XX], clause 6.1.1

Requirement Description:

"It should be noted that the UE initiated re-registration opens up a potential denial-of-service attack. That is, an attacker could try to register an already registered IMPU and respond with an incorrect authentication response in order to make the HN de-register the IMPU. For this reason a subscriber, when registered, shall not be de-registered if it fails an authentication.

"

as specified in TS 33.203 [XX], clause 6.1.1.

Threat References: TBD

Test case:

Test Name: TC_ NO_DE-REGISTRATION_AUTH_FAIL

Purpose:

Verify the S-CSCF shall not de-register the registered UE when it fails an authentication during re-registration.

Procedure and execution steps:

Pre-Conditions:

-	S-CSCF under test is connected in simulated/real network environment including P-CSCF and HSS.

-	The UE supporting IMS AKA has already been registered into the IMS network.

-	The tester shall have access to the Mw interface between the P-CSCF and S-CSCF.

-	The tester shall have access to the Cx interface between the HSS and S-CSCF.

Execution Steps

- During a new IMS AKA procedure in the UE ininited re-registration scenario, the tester sends a SM7 register message including the IMPI, and an incorrect authentication response.

- The S-CSCF under test retrieves the active XRES for that user and uses this to check the received authentication response

Expected Results:

The S-CSCF sends sends a 4xx Auth_Failure towards the UE indicating that authentication has failed.

The S-CSCF does not initiate de-registration procedure within the Registration expiration interval defined in TS 24.229 [YY], i.e. send either Cx-Put (Public User Identity, Private User Identity, clear S-CSCF name) or Cx-Put (Public User Identity, Private User Identity, keep S-CSCF name) to the HSS. Or, the IMPU status in the HSS is registered within the Registration expiration interval defined in TS 24.229 [YY].

Expected format of evidence:

Provide evidence of the check of the product documentation in plain text.

Save the logs and the communication flow in a .pcap file.

*************** End of the changes ****************