3GPP TSG-SA3 Meeting #100e 	S3-201860

- e-meeting, 17-28 Augest 2020													Revision of S3-20xxxx

- Source:	Huawei, Hisilicon

- Title:	IMS SCAS: new test case on unprotected register message

- Document for:	Approval

Agenda Item:	4.15

# 1	Decision/action requested

This contribution proposes a new test case on unprotected register message.

# 2	References

[1]	3GPP TS 33.203	3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G security;Access security for IP-based services

[2]		S3-201859

# 3	Rationale

As specified in TS 33.203 [1], clause 7.4.0, if the UE has an already active pair of security associations, and the UE sends unprotected REGISTER messages. In this case, the S-CSCF shall authenticate the user by means of the AKA protocol. Otherwise, the attacker without a legal certificates, or pre-shared key could be able to access the network. The data and resources stored in the network may be exposed to an attacker, making the system easily attacked and/or compromised.

The threats analysis can be found in the S3-201859 [2].

# 4	Detailed proposal

It is suggested to approve the following change.

*************** Start of the changes ****************

#### 4.2.2.X	Security functional requirements on the S-CSCF deriving from 3GPP specifications and related test cases

##### 4.2.2.X.1	Unprotected register message

Requirement Name: Unprotected register message

Requirement Reference: TS 33.203 [XX], clause 7.4.0

Requirement Description:

"If the UE has an already active pair of security associations, then it shall use this to protect the REGISTER message. If the S-CSCF is notified by the P-CSCF that the REGISTER message from the UE was integrity-protected it may decide not to authenticate the user by means of the AKA protocol. However, the UE may send unprotected REGISTER messages at any time. In this case, the S-CSCF shall authenticate the user by means of the AKA protocol. "

as specified in TS 33.203 [XX], clause 7.4.0.

Threat References: TBD

Test case:

Test Name: TC_UNPROTECTED_REGISTER_MESSAGE

Purpose:

Verify whether the S-CSCF authenticates the user by means of the AKA protocol, if the UE sends unprotected REGISTER messages, regardless whether the UE is already registered or not.

Procedure and execution steps:

Pre-Conditions:

-	S-CSCF network product are connected in simulated/real network environment.

-	The list of ordered integrity and encryption algorithms are configured on the P-CSCF under test.

-	The UE and the P-CSCF are simulated.

-	The UE supports a list of ordered integrity and encryption algorithms.

-	The tester has access to the Gm interface between the UE and P-CSCF.

-	The tester has access to the Mw interface between the P-CSCF and S-CSCF.

-	The UE has an already active pair of security associations.

Execution Steps

This test is performed in the Authenticated re-registration procedure, the UE has an already active pair of security associations.

1) The UE sends unprotected REGISTER messages (SM1) to the P-CSCF.

2) The P-CSCF sends unprotected REGISTER messages (SM2) to the S-CSCF under test.

3) The S-CSCF under test receives the SM2 from the P-CSCF.

4) The tester examines whether the S-CSCF under test sends SM4: Auth_Challenge to the P-CSCF to authenticate the user by means of the AKA protocol.

Expected Results:

The S-CSCF under test authenticates the user by means of the AKA protocol after.

Expected format of evidence:

Provide evidence of the check of the product documentation in plain text. Save the logs and the communication flow in a .pcap file.

*************** End of the changes ****************