3GPP TSG-SA3 Meeting #100e 	S3-201890

- e-meeting, 17 -28 August 2020													Revision of S3-20xxxx

- Source	:	Lenovo, Motorola Mobility

- Title	:	Key Issue on User Plane Security Compromise

- Document for	:	Approval

Agenda Item	:	5.14

# 1	Decision/action requested

This pCR proposes a Key Issue to TR 33.840 on User Plane Security Compromise

# 2	References

[1]	3GPP TS 38.401, ‘NG-RAN Architecture Description’, Rel-16

[2]	3GPP TR 38.823, ‘Study of further enhancement for disaggregated gNB’, Rel-16.

[3]	3GPP S3-200945r3 Study on security aspects of the Disaggregated gNB Architecture.

# 3	Rationale

According to TR 38.823 [2], the gNB-CU-CP selects the appropriate gNB-CU-UP(s) for the UE requested services. The TR 38.823 further discuss about the possible disaggregated gNB deployment where a gNB-CU-UP can be compromised and impacts the overall user plane security [2]. The current 5G systems assumes that multiple CU-UPs selected by the CU-CP for a UE requested service(s) belongs to same security domain [1]. Whereas, from the the RAN point of view, [2][3] discusses few scenarios where a UE connects to multiple gNB-CU-UPs among which Case 1 states that, ‘The gNB-CU-UPs connected to the same gNB-CU-CP can be distributed or centralized. Distributed gNB-CU-UP can provide local termination point for low-latency services, while centralized gNB-CU-UP is beneficial for cloud implementation.’.

In a scenario where different gNB-CU-UPs (one from distributed gNB-CU-UP and other(s) from centralized gNB-CU-UP) are selected for a UE, to provide different services, a user plane security compromise at a single distributed gNB-CU-UP will impact the over all user plane security of the UE (as all user planes for a UE share the same UP security keys). As the 5G system is expected to provide robust security for its end user services, SA3 should consider the disaggregated gNB deployment scenario and provide proactive measures to prevent the overall user plane security breach in 5GS.

# 4	Detailed proposal

SA3 is kindly requested to agree to the below pCR to TR 33.840.

*****Start of Change*****

## 5.X	Key Issue #X: User Plane Security Compromise

### 5.X.1	Key issue details

A gNB may consist of a gNB-CU-CP, multiple gNB-CU-UPs and multiple gNB-DUs. The gNB-CU-CP selects the appropriate gNB-CU-UP(s) for the requested services for the UE as defined in TS 38.401. The diaggregated gNB supports both distributed gNB-CU-UP(s) and centralized gNB-CU-UP(s) deployment options, where the distributed gNB-CU-UP(s) can provide local termination point for low latency services (example URLLC). To provide different set of services for a UE, a gNB-CU-CP can simultanely select one or more distributed gNB-CU-UP (for example a URLLC service) and other centralized gNB-CU-UP(s) (for example a eMBB service) and establish a common user plane security for all user planes irrespective of the user plane security termination point (i.e., gNB-CU-UP based on deployment location / supported slice). In a scenario where the user plane security termination point varies a potential security compromise at one termination point (i.e., a distributed gNB-CU-UP) will impact the over all user plane security of the UE. As the 5G system is expected to have a robust security by design, it is more relevant to address this key issue in Rel.17.

### 5.X.2	Security threats

In a disaggregated deployment, the compromise of one gNB-CU-UP can impact and breach the overall user plane security of UE(s) in the 5G system.

### 5.X.3	Potential security requirements

The 5G system shall provide means to prevent the overall user plane security breach in a disaggregated gNB deployment.

The 5G system shall enable sufficient cryptographic separation when required in the disaggregated gNB deployment to ensure the overall user plane security.

*****End of Change*****