3GPP TS 33.522 V0.2.0 (2020-08)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

5G Security Assurance Specification (SCAS);

Service Communication Proxy (SCP)

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

SCAS, 5G, SCP, assurance, security

3GPP

Postal address

3GPP support office address

650 Route des Lucioles – Sophia Antipolis

Valbonne – FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	SCP-specific security requirements and related test cases	6

4.1	Introduction	6

4.2	SCP-specific adaptations of security functional requirements and related test cases	6

4.2.1	Introduction	6

4.2.2	Security functional requirements on the SCP derived from 3GPP specifications and related test cases	6

4.2.2.1	Security functional requirements on the SCP derived from 3GPP specifications – general approach	6

4.2.2.2	Security functional requirements of SBI aspects	6

4.2.2.2.1	Introduction	6

4.2.2.2.2	Protection at the transport layer	7

4.2.2.2.3	Authorization of NF service access	7

4.2.3	Technical Baseline	7

4.2.4	Operating Systems	7

4.2.5	Web Servers	7

4.2.6	Network Devices	7

4.3	SCP-specific adaptations of hardening requirements and related test cases	7

4.3.1	Introduction	7

4.3.2	Technical Baseline	7

4.3.3	Operating Systems	7

4.3.4	Web Servers	7

4.3.5	Network Devices	7

4.3.6	Network Functions in service-based architecture	7

4.4	SCP-specific adaptations of basic vulnerability testing requirements and related test cases	7

Annex A (informative): Change history	8

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document contains objectives, requirements and test cases that are specific to the SCP network product class. It refers to the Catalogue of General Security Assurance Requirements and formulates specific adaptions of the requirements and test cases given there, as well as specifying requirements and test cases unique to the SCP network product class.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[3]	3GPP TS 33.501 “Security architecture and procedures for 5G system” (Release 16).

[4]	3GPP TR 23.501 " System architecture for the 5G System (5GS); Stage 2" (Release 16).

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

# 4	SCP-specific security requirements and related test cases

## 4.1	Introduction

The structure of the present document is aligned with TS 33.117 [2] such that the SCP-specific adaptation of a generic requirements in TS 33.117 [2] clause 4.a.b.c.d can always be found in TS 33.522 clause 4.a.b.c.d. The text on pre-requisites for testing in clause 4.1.1 of TS 33.117 [2] applies also to the present document.

## 4.2	SCP-specific adaptations of security functional requirements and related test cases

### 4.2.1	Introduction

The present clause describes the security functional requirements and the corresponding test cases for SCP network product class. The proposed security requirements are classified in two groups:

-	Security functional requirements derived from TS 33.501 [3] and detailed in clause 4.2.2

-	General security functional requirements which include requirements not already addressed in TS 33.501 [3] but whose support is also important to ensure that SCP conforms to a common security baseline detailed in clasue 4.2.3.

### 4.2.2	Security functional requirements on the SCP derived from 3GPP specifications and related test cases

#### 4.2.2.1	Security functional requirements on the SCP derived from 3GPP specifications – general approach

In addition to the requirements and test cases in TS 33.117 [2], clause 4.2.2, an SCP shall satisfy the following:

It is assumed for the purpose of the present SCAS that an SCP conforms to all mandatory security-related provisions pertaining to an SCP in:

-	3GPP TS 33.501: "Security architecture and procedures for 5G system";

-	other 3GPP specifications that make reference to TS 33.501 or are referred to from TS 33.501 (e.g. TS 23.501, TS 23.502, TS 29.500, etc.).

Security procedures pertaining to an SCP are typically embedded in NF/NF indirect communication, delegated discovery, message forwarding and routing, and are hence assumed to be tested together with them in interoperability testing at PLMN level, shared-slice level and slice-specific level.

#### 4.2.2.2	Security functional requirements of SBI aspects

##### 4.2.2.2.1	Introduction

According to TS 23.501 [4], although the SCP is not a Network Function instance and does not expose services itself, it still needs to support service-based interface. Therefore, the general baseline requirements supported by all Network Functions (NF) utilizing Service-Based Interfaces (SBI) as defined in TS 33.117 [2] clause 4.2.2.2 shall also be applicable to the SCP network product class. This sub-clause contains SCP-specific adaptations to the general SBI requirements and related test cases.

##### 4.2.2.2.2	Protection at the transport layer

There are no SCP-specific additions to clause 4.2.2.2.2 of TS 33.117 [2].

##### 4.2.2.2.3	Authorization of NF service access

As the SCP is not a network function instance and does not provide any services to any consumer NF, the SCP does not need to support authorization for NF service access. Therefore, the requirements and test cases in clause 4.2.2.2.3 of TS 33.117 [2] are not applicable to the SCP network product class.

Editor's note: 	It is to be updated when it is concluded in TS 33.501 [3] whether the SCP could perform OAuth based service access authorization on behalf of Network Functions or not.

### 4.2.3	Technical Baseline

### 4.2.4	Operating Systems

### 4.2.5	Web Servers

### 4.2.6	Network Devices

## 4.3	SCP-specific adaptations of hardening requirements and related test cases

### 4.3.1	Introduction

The requirements proposed hereafter (with the relative test cases) aim to securing SCP by reducing its surface of vulnerability. In particular, the identified requirements aim to ensure that all the default configurations of SCP (including operating system software, firmware and applications) are appropriately set.

### 4.3.2	Technical Baseline

### 4.3.3	Operating Systems

### 4.3.4	Web Servers

### 4.3.5	Network Devices

### 4.3.6	Network Functions in service-based architecture

## 4.4	SCP-specific adaptations of basic vulnerability testing requirements and related test cases

Editor’s Note: Take TS 33.501, as a starting point, and specify SCP-specific adaptations, if required. Note subclauses as “void” or “no adaptation needed” as appropriate.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-05 | SA3#99e | S3-201373 |  |  |  | Skeleton | 0.0.0 |
| 2020-05 | SA3#99e | S3-201379 |  |  |  | Implemented S3-201373, S3-201374 and S3-201375. | 0.1.0 |
| 2020-08 | SA3#100e | S3-202143 |  |  |  | Implemented S3-202138 | 0.2.0 |