3GPP TSG-SA3 Meeting #101-e	S3-202899

- e-meeting, 9 – 20 November 2020	Revision of S3-20xxxx

- Source:	CATT

- Title:	Update Solution #6 in TR 33.839

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

It is proposed to update solution#6. SA3 is kindly requested to approve this contribution.

# 2	References

[1]	3GPP TS 33.839, v0.2.0

# 3	Rationale

This contribution proposes to resolve the following editor’s notes.

Editor's note: details of authorization is FFS.

# 4	Detailed proposal

# *** START of CHANGE ***

### 6.6.2	Solution details

Figure 6.6.2-1 Authentication and Authorization between the EEC and the EES

The procedure assumes that the Edge Data Network is deployed by the ECSP. Both the ECS and the EES stores the mapping between the EEC ID and GPSI for each EEC. The the ECS will store the allowed EES list and the subscription expiration time.

1. The UE registers in the operator network and perform the primary authentication procedure.

2. When the UE trigger the edge service it sends the PDU session establishment request to the AMF to setup the PDU session for the services provided by Edge Data Network.

3-6. The following steps 3, 4, 5, 6 are the same as steps 5a-13 in clause 11.1.2 of TS 33.501[7]. The secondary authentication procedure is performed. The ECS is the authentication server of the Edge Data Network.

7. After the successful completion of the authentication procedure, the ECS sends EAP Success message to the SMF including the address information of Edge Enabler Server and the GPSI.

8. The SMF sends a Namf_Communication_N1N2MessageTransfer to the AMF with the received information.

9. The AMF forwards NAS SM PDU Session Establishment Response message along with EAP Success, the address information of suitable Edge Enabler Server and the GPSI to the UE/EEC.

10. The EEC sends Edge Enabler Client registration request to the EES.

11. The EES shall verify the mapping between the EEC ID and GPSI. Then the EES request to validate authorization of the EEC from the ECS with the EEC ID.The ECS will check whether the EEC has been authorized to access to the EES for edge computing service with GPSI corresponding to the EEC ID.

Editor’s Note: It is ffs how the EES verifies the binding between EEC ID and GPSI, and how the ECS obtains the binding between EEC ID and GPSI.

12. If the EEC is authoried, the ECS responses to the EES with the service authorization response message.

13. After successful service authorization verification, the EES sends Edge Enabler Client registration response to the EEC .

Editor's note: It is FFS how to proceed the authentication and authorization if the secondary authentication is not performed.