3GPP TSG-SA3 Meeting #101-e 	draft_S3-203225-r2

- e-meeting, 9th - 20th November 2020												Revision of S3-20xxxx

- Source:	Ericsson

- Title:	pCR to living document for TS 33.220: Resolving EN for HSS

- Document for:	Approval

Agenda Item:	4.14

# 1	Decision/action requested

It is proposed to remove EN for HSS.

# 2	References

[1]	S3-203219: "Living document for TS 33.220: SBA support for Zh and Zn".

[2]	3GPP TS 33.220: " Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)"

[3]	3GPP S3-202031: "

Discussion of HSS role in supporting SBA for GBA "

# 3	Rationale

In the living document for TS 33.220 [1], there is Editor's Note mentioning HSS as below:

Editor’s Note: For 5G, we only have the AUSF, UDM. Who will play the role of HSS is FFS.

S3-202031 [3] provides the reasons for choosing the HSS to be in the architecture of SBA for GBA and proposes to remove this Editor's Note from the living document of TS 33.220 [1].

This contribution proposes to remove the mentioned Editor's Note.

# 4	Detailed proposal

**** START OF CHANGES ****

### X.1.1 Architectural Support

Figure X.1.1-1 shows the non-roaming architecture to support SBA interactions in GBA. An SBI capable BSF, HSS and NAF shall implement the SBA interfaces specified in this Annex. An SBI capable NF can invoke SBA services provided by SBI capable NFs and may expose services itself. For this Annex an SBI capable BSF uses and provides SBA services, an SBI capable HSS provides SBA services while an SBI capable NAS only uses SBA services. The BSF, HSS and NAF reside in the home network.

Figure X.1.1-1: System Architecture to support SBA in GBA

Figure X.1.1-2 shows the architecture using the reference point representation. It should be observed that this annex addresses only the specification of the NG1 (between the BSF and HSS) and NG2 (between the NAF and BSF) reference point interfaces as SBA interfaces. The specification of Ua and Ub is not impacted by the introduction of the SBA interfaces between the NAF, BSF and HSS. Therefore, the UE interacts with the BSF and NAF as defined in the main body of this specification.

Figure X.1.1-2: System Architecture to support SBA in reference point representation

Editor’s Note: Roaming considerations are FFS.

Editor's Note: Additional service-based interface and reference point between BSF and UDM are FFS.

### X.1.2 Reference point to support SBA in GBA

The following reference points are realized by service-based interfaces in GBA:

NG1: Reference point between an SBI capable BSF and an SBI capable HSS.

NG2: Reference point between an SBI capable BSF and an SBI capable NAF.

Editor’s Note: New reference point names are to be confirmed with SA2.

If there is no HSS or if the HSS does not support NG1 reference point within the GBA architecture, then the BSF shall  be configured to use NG1’ reference point with UDM. If the NG1 reference point is available in the HSS, then it shall be used between the BSF and the HSS.

Editor's Note: NG1' reference point is between an SBI capable BSF and a UDM to provide authentication vectors and GUSS for the purposed of GBA. It is FFS how this interface is specified.

### X.1.3 Service based interface to support SBA in GBA

The following service-based interfaces are defined:

Nhss: Service-based interface exhibited by an SBI capable HSS.

Nbsp: Service-based interface exhibited by an SBI capable BSF.

Editor’s Note: New SBA interface name for BSF is to be confirmed with SA2.

Editor's Note: Additional service-based interface is FFS for NG1’.

These SBI services provide equivalent functionality to the Diameter Zh and Zn reference points.

To support co-existence of GBA nodes supporting SBA services and GBA nodes not supporting SBA services SBI capable GBA nodes may support both SBI and non-SBI interfaces.

## X.2 GAA/GBA SBA Services

Editor’s Note: More details to be specified.

Editor’s Note: Whether GBA AKA is still application in 5G-GBA is FFS.

**** END OF CHANGES ****