3GPP TSG-SA3 Meeting #102Bis-e 	draft_S3-210843-r1

- e-meeting, 1 – 5 March 2021													Revision of S3-210843

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Assuring SUCI generation by legitimiate SUPI owner using KSUCI

- Document for:	Approval

Agenda Item:	2.5

# 1	Decision/action requested

Assuring SUCI generation by legitimate SUPI owner using KSUCI  solution approval request

# 2	References

[1]	TR 33.846

# 3	Rationale

Solution on Assuring SUCI generation by legitimate SUPI owner for Key Issue #2.2: SUCI based attacks (Linkability by generation of different SUCIs).

# 4	Detailed proposal

************ START OF CHANGES

- 6.0	Mapping of solutions to key issues

Table 6.0-1: Mapping of solutions to key issues

|  | Key Issues |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1.X | #2.1 | #2.2 | #3.1 | #3.2 | #4.1 |  |  |
| Solutions for anchor keys security |  |  |  |  |  |  |  |  |
| No solution so far |  |  |  |  |  |  |  |  |
| Solutions for resilience against identifier linkability |  |  |  |  |  |  |  |  |
| #2.1: Handling of Sync failure by AUTS encryption |  | x |  |  |  | x |  |  |
| #2.2: Encryption of authentication failure message types by UE with new keys derived from K_AUSF |  | x |  |  |  | x |  |  |
| #2.3: Unified authentication response message by UE |  | x |  |  |  |  |  |  |
| #2.4: MAC-S based solution |  | x |  |  |  | x |  |  |
| #2.5: Encryption of authentication failure message with SUCI method |  |  | x |  |  |  |  |  |
| #2.6: Certificate based encryption of unicast NAS message |  | x |  |  |  | x |  |  |
| #2.X: Assuring SUCI generation by legitimiate SUPI owner using KSUCI |  |  | x |  |  |  |  |  |
| Solutions for availability aspects of SUCI usage |  |  |  |  |  |  |  |  |
| No solution so far |  |  |  |  |  |  |  |  |
| Solutions on re-synchronisation in AKA |  |  |  |  |  |  |  |  |
| #4.1: Using MACS as freshness in the calculation of AK |  |  |  |  |  | x |  |  |
| #4.2: Using symmetric encryption function to protect SQN during a re-synchronisation procedure in AKA |  |  |  |  |  | x |  |  |
| #4.3: SQN protection by concealment with SUPI in USIM |  | x | x |  |  | x |  |  |
| #4.4: SQN protection during re-synchronisation procedure in AKA |  |  |  |  |  | x |  |  |
| #4.5: AUTS SQNMS solution for 5GS |  |  |  |  |  | x |  |  |
| #4.7: SQN protection by concealment with SUPI with f5* |  | x | x |  |  | x |  |  |

### ************ NEXT CHANGE

### 6.2.X	Solution #2.X: Assuring SUCI generation by legitimiate SUPI owner using KSUCI

#### 6.2.X.1	Introduction

This solution addresses Key Issue #2.2: SUCI based attacks (Linkability by generation of different SUCIs).

This solution proposed to use a specific key, called KSUCI when generating the SUCI. By this it is assured that the generated SUCI can only be created by the user to whom the long-term key K belongs to.

#### 6.2.X.2	Solution details

The solution follows the SUCI generation scheme, but the MAC tag is generated out of a new MAC key, which is the result of an additionally introduced one-time function taking the Ephemeral MAC key and KSUCI as input. The rest of the known SUCI generation scheme is not touched.

KSUCI is generated by considering long term key K, FC and newly generated RANDSUCI. UE can share the scheme output of SUCI to UDM as "Final output = Eph. public key || RANDSUCI || Ciphertext || MAC tag [|| any other parameter]".

During SUCI generation, after the Ephemeral MAC key has been generated (according to the scheme described in Annex A of TS 33.501), a new one-way function f is applied. f is using the generated KSUCI as additional input to the ephemeral MAC key. The result is used as the new MAC key: MAC key  f (KSUCI, Ephemeral MAC key).

With the new MAC key, the computation of the SUCI is continued as specified in 3GPP TS 33.501 [V].

Figure 6.2.X.2-1 SUCI derivation using KSUCI

#### 6.2.X.3	Evaluation

This solution works for calculation of SUCI at both USIM and ME.

This new MAC key cannot be generated by an attacker, unless he would be in possession of the individual long-term key K and its derivative KSUCI of the subscriber.Thus, it is assured that only the UE/UICC where the long-term key K is stored can create SUCI, which renders the attack of linkability creating different SUCIs out of a known SUPI.

NOTE: Derivations of long-term key K are used in the authentication process, e.g. for RES being derived from CK/IK, with RES being sent over the air in clear. Similarly, there is no security issue foreseen in using a new derivation of K called KSUCI for creating the new MAC key.

Editor Notes: The security risk to use a derivation of long term key for any new purposes (example: during SUCI derivation) different from the ones already specified for AKA procedure needs to be evaluated by ETSI SAGE.

************ END OF CHANGES