3GPP TSG-SA3 Meeting #102-e	S3-210755

# e-meeting, 18 - 29 January 2021, Online

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.513 | CR | Draft CR | rev | 1 | Current version: | 16.2.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New test case on traffic filtering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | SCAS_5G_IPUPS |  |  |  |  |  | Date: |  |  | 2020-12-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It has been specified in TS 33.501 clause 5.9.3.4 that The IPUPS shall only forward GTP-U packets that contain an F-TEID that belongs to an active PDU session and discard all others. The corresponding threat (S3-202136) that has been added in TR 33.926 is also approved |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new test case on how IPUPS handle traffic that is not within an active PDU session. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 4.2.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Change 1 *************************

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501 (Release 15): "Security architecture and procedures for 5G system".

[3]	3GPP TS 33.117: "Catalogue of general security assurance requirements".

[4]	3GPP TS 23.501: "System Architecture for 5G system".

[5]	3GPP TS 29.281: "General Packet Radio System (GPRS) Tunnelling Protocol User Plane (GTPv1-U) ".

[6]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[7]	3GPP TR 33.926: "Security Assurance Specification (SCAS) threats and critical assets in 3GPP network product classes".

[X]	3GPP TS 33.501 (Release 16): "Security architecture and procedures for 5G system".

*************************** End of Change 1 ***************************

******************* Start of Change 1****************************

4.2.2.Y	IPUPS

Requirement Name:IPUPS packeting handling

Requirement Reference: TS 33.501[X], clause 5.9.3.4

Requirement Description:

"The IPUPS shall only forward GTP-U packets that contain an F-TEID that belongs to an active PDU session and discard all others. "

as specified in TS 33.501[5], clause 5.9.3.4.

Threat Reference: TR 33.926 [7], Clause L.2.W, "invalid user plane data forwarding"

TEST CASE:

NOTE Z:	This test case is only applicable to UPF supporting IPUPS.

Test Name: TC_IPUPS_PACKET_HANDLING

Purpose:

Verify that the packets not belonging to an active PDU session is discarded.

Pre-Conditions:

Test environment is set up with a V-SMF,an H-SMF, an H-UPF and a gNB which may be simulated.

Execution Steps:

- The V-SMF requests the UPF with IPUPS functionality under test to establish a N4 session for a PDU session in home-routing roaming. The UPF with IPUPS functionality under test responds to the SMF with the F-TEID for the N9 tunnel towards the H-UPF, and the F-TEID for the N3 tunnel towards the gNB.

- The V-SMF requests the H-SMF to establish a PDU session providing the received F-TEID for the N9 tunnel.

- The H-SMF requests the H-UPF to establish an N4 session providing the received F-TEID for the N9 tunnel. H-UPF in the response provides its F-TEID for the N9 tunnel. The H-SMF provides the received F-TEID from the H-UPF to the V-SMF

- The V-SMF requests the gNB to allocate resource for the PDU session providing the F-TEID for the N3 tunnel received at step 1.  The gNB replies with its F-TEID for the N3 tunnel to the V-SMF

- The V-SMF provides the UPF with IPUPS functionality under test with the received F-TEID assigned by the gNB for the N3 tunnel and the received F-TEID assigned by the H-UPF for the N9 tunnel

- The H-UPF is triggered to send GTP-U packets using the F-TEID assigned by the V-UPF for the N9 tunnel.

- The H-UPF is triggered to send GTP-U packets using a F-TEID different than the one assigned by V-UPF for N9 tunnel.

Expected Results:

When the H-UPF is triggered to send GTP-U packets using the F-TEID assigned by the V-UPF for the N9 tunnel (step 6 in the execution steps), GTP-U packets are witnessed over the N3 tunnel.

When the H-UPF is triggered to send GTP-U packets using an F-TEID different than the one assigned by the V-UPF (step 7 in the execution steps), no GTP-U packets are witnessed over the N3 tunnel.

Expected format of evidence:

Files recording the the GTP packets captured (e.g. pcap trace).

************************ End of Change 1 ************************