3GPP TSG-SA3 Meeting #103-e 	S3-212065

# e-meeting, 17th - 28th May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.203 | CR | 0258 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps | X | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Security updates for algorithms and protocols in 33.203 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCryptPr |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - HTTP Digest (RFC 2617 [12]) mandates use of MD5. It has been obsoleted by RFC 7235, RFC 7615, and RFC 7616 which mandates support of SHA-256. - ICE, STUN, TURN (RFC 5245 [59], RFC 5389 [69], RFC 5766 [61]) mandates use of SHA-1. They have been obsoleted by RFC 8445, RFC 8839, RFC 8489, and RFC 8656 which mandates support of SHA-256. - IPv6 privacy (RFC 3041 [18]) mandates use of MD5. It has been obsoleted by RFC 8981 which mandates support of SHA-256. - No reference to TS 33.401 [55] - RFC 2817 should be [63] not [62] - Annex H and I specify use of SHA-1. As GMAC has been mandatory to support since Rel-13. SHA-1 can be removed. Without SHA-1 there is no need for CBC mode. - Annex S (Intra-domain Domain Security) mandates support of IKEv1, SHA-1, 3DES, and MD5. IKEv1 (RFC 2407 [47], RFC 2408 [48], RFC 2409 [49]) has been obsoleted by IKEv2 (RFC 7296). IKEv1 has not been allowed in 3GPP since Rel-11. - Annex O (TLS based access security) mandates the 3GPP profile but talks about TLS 1.0, TLS 1.1 and NULL cipher suites, which are all forbidden by the 3GPP TLS profile. VoLTE and VoNR are essntial 5G mechanisms based on IMS. MD5, SHA-1, 3DES, TLS 1.0, TLS 1.1, are very algorithms that may offer a false sense of security. They should not be used in any system, especially not critical infrastructure. Critical 3GPP infrastructure like 5G should use only use recommended best practice algorithms. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - HTTP Digest (RFC 2617 [12]) replaced with RFC 7235, RFC 7615, and RFC 7616. Both SHA-256 and MD5 is mandatory to support. - ICE (RFC 5245 [59]), STUN (RFC 5389 [69]), and TURN (RFC 5766 [61]) are voided and replaced with RFC 8445, RFC 8839, RFC 8489, and RFC 8656. - IPv6 privacy (RFC 3041 [18]) is voided and replaced with RFC 8981. - TS 33.401 [55] is voided. - Reference to RFC 2817 is changed from [62] not [63] - SHA-1 and CBC is removed from Annex H and I. - IKEv1, SHA-1, 3DES, and MD5 are removed from Annex S and replaced with the 3GPP IKEv2 and ESP profiles. - TLS 1.0, TLS 1.1, and NULL are removed from Annex O. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - MD5, SHA-1, 3DES, TLS 1.0, TLS 1.1, are very algorithms that may offer a false sense of security. They should not be used in any system, especially not critical infrastructure. - Referencing obsolete RFCs makes 3GPP 5G standards look old. There is also a security risk as obsolete RFCs often have obsolete security and privacy guidelines. - Implementors might believe that support of TLS 1.0 and 1.1 are mandatory to support. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.5, Annex H, Annex I, Annex N.1, Annex N.2.1, Annex O.2.1, Annex S.5.3, Annex S.5.4.2.1, Annex S.5.4.2.2, Annex W.3.1.2, Annex W.3.1.3, Annex W.3.3, Annex X.2.2.3, Annex X.5.1, Annex X.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** BEGIN CHANGES ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 33.102: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Security Architecture".

[2]	Void.

[3]	3GPP TS 23.228: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia (IM) Subsystem".

[4]	Void.

[5]	3GPP TS 33.210: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; 3G Security; Network domain security; IP network layer security".

[6]	IETF RFC 3261 "SIP: Session Initiation Protocol".

[7]	3GPP TS 21.905: "3rd Generation Partnership Project: Technical Specification Group Services and System Aspects; Vocabulary for 3GPP specifications".

[8]	3GPP TS 24.229: "3rd Generation Partnership Project: Technical Specification Group Core Network; IP Multimedia Call Control Protocol based on SIP and SDP".

[9]	3GPP TS 23.002: "3rd Generation Partnership Project: Technical Specification Group Services and System Aspects, Network Architecture".

[10]	3GPP TS 23.060: "3rd Generation Partnership Project: Technical Specification Group Services and System Aspects, General Packet Radio Service (GPRS); Service Description".

[11]	3GPP TS 24.228: "3rd Generation Partnership Project: Technical Specification Group Core Network; Signalling flows for the IP multimedia call control based on SIP and SDP".

[12]	Void.

[13]-[16]	Void.

[17]	IETF RFC 3310 (2002): "HTTP Digest Authentication Using AKA". April, 2002.

[18]	Void.

[19]	Void.

[20]	Void.

[21]	IETF RFC 3329 (2003): "Security Mechanism Agreement for the Session Initiation Protocol (SIP)".

[22]	Void.

[23]	IETF RFC 3263 (2002): "Session Initiation Protocol (SIP): Locating SIP Servers".

[24]	3GPP TS 33.310: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Network Domain Security (NDS); Authentication Framework (AF)".

[25]	Void.

[26]	ETSI ES 282 001: "TISPAN - Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); NGN Functional Architecture for NGN Release 1".

[27]	IETF RFC 3947 (2005): "Negotiation of NAT-Traversal in the IKE".

[28]	IETF RFC 3948 (2005): "UDP Encapsulation of IPsec ESP Packets".

[29]	IETF RFC 3323 (2002): "A Privacy Mechanism for the Session Initiation Protocol (SIP)".

[30]	IETF RFC 3325 (2002): "Private Extensions to the Session Initiation Protocol (SIP) for Asserted Identity within Trusted Network".

[31]	3GPP TS 23.167: "3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; IP Multimedia Subsystem (IMS) emergency sessions”.

[32]	IETF RFC 5626 (2009): "Managing Client Initiated Connections in the Session Initiation Protocol (SIP)".

[33]	Void.

[34]	Void.

[35]	Void.

[36]	ETSI ES 282 004: “NGN Functional Architecture; Network Attachment Sub-System (NASS)”

[37]	ETSI TS 187 001: " Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); NGN SECurity (SEC); Requirements"

[38]	Void.

[39]	3GPP TS 29.228: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; IP Multimedia (IM) Subsystem Cx and Dx interfaces; Signalling flows and message contents".

[40]	3GPP2 X.S0011: "cdma2000  Wireless IP Network Standard".

[41]	3GPP2 C.S0023: "Removable User Identity Module for Spread Spectrum Systems".

[42]	Void.

[43]	3GPP2 S.S0055: "Enhanced Cryptographic Algorithms".

[44]	3GPP2 S.S0078: "Common Security Algorithms".

[45]	3GPP2 C.S0065: "cdma2000 Application on UICC for Spread Spectrum Systems".

[46]	3GPP TS 23.003: "3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Numbering, addressing and identification".

[47]	Void.

[48]	Void.

[49]	Void.

[50]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) Centralized Services; Stage 2".

[51]	3GPP TS 31.103: "3rd Generation Partnership Project: Technical Specification Group Core Network and Terminals; Characteristics of the IP Multimedia Services Identity Module (ISIM) application".

[52]	IETF RFC 5280: "Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile".

[53]	IETF RFC 4301: "Security Architecture for the Internet Protocol".

[54]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[55]	Void.

[56]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[57]	ETSI TS 187 003 v3.4.1: "Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); NGN Security; Security Architecture".

[58]	Void.

[59]	Void.

[60]	IETF RFC 6544: "TCP Candidates with Interactive Connectivity Establishment (ICE) ".

[61]	Void.

[62]	IETF RFC 6062: "Traversal Using Relays around NAT (TURN) Extensions for TCP Allocations".

[63]	IETF RFC 2817: "Upgrading to TLS Within HTTP/1.1".

[64]	IETF RFC 6623: "Indication of Support for Keep-Alive".

[65]	IETF RFC 4169: "Hypertext Transfer Protocol (HTTP) Digest Authentication Using Authentication and Key Agreement (AKA) Version-2”.

[66]	3GPP TS 33.220: "Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)".

[67]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[68]	IETF RFC 7376: "Problems with Session Traversal Utilities for NAT (STUN) Long-Term Authentication for Traversal Using Relays around NAT (TURN)".

[69]	Void.

[70] 	IETF RFC 7635: "Session Traversal Utilities for NAT (STUN) Extension for Third Party Authorization".

[71]	Void

[72]	IETF RFC 6749: "The OAuth 2.0 Authorization framework".

[73]	IETF RFC 4106: "The Use of Galois/Counter Mode (GCM) in IPsec Encapsulating Security Payload (ESP)".

[74]	IETF RFC 4543: "The Use of Galois Message Authentication Code (GMAC) in IPsec ESP and AH".

[75]	IETF RFC 7800: "Proof-of-Possession Key Semantics for JSON Web Tokens (JWTs)".

[Z1]	IETF RFC 8489: "Session Traversal Utilities for NAT (STUN)".

[Z2]	IETF RFC 8656: " Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)".

[Z3]	IETF RFC 8445: "Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal".

[Z4]	IETF RFC 8839: "Session Description Protocol (SDP) Offer/Answer Procedures for Interactive Connectivity Establishment (ICE)".

[Z5]	IETF RFC 8981: "Temporary Address Extensions for Stateless Address Autoconfiguration in IPv6".

[Z6]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[Z7]	IETF RFC 7235: "Hypertext Transfer Protocol (HTTP/1.1): Authentication".

[Z8]	IETF RFC 7615: " HTTP Authentication-Info and Proxy-Authentication-Info Response Header Fields".

[Z9]	IETF RFC 7616: "HTTP Digest Access Authentication".

*** NEXT CHANGE ***

## 7.5	Rules for security association handling when the UE changes IP address

When a UE changes its IP address, e.g. by using the method described in RFC 8981 [Z5], then the UE shall delete the existing SA's and initiate an unprotected registration procedure using the new IP address as the source IP address in the packets carrying the REGISTER messages.

*** NEXT CHANGE ***

Annex H (normative):
The use of "Security Mechanism Agreement for SIP Sessions" [21] for security mode set-up

The BNF syntax of RFC 3329 [21] is defined for negotiating security associations for semi-manually keyed IPsec or TLS in the following way:

security-client		= "Security-Client" HCOLON sec-mechanism *(COMMA sec-mechanism)

security-server		= "Security-Server" HCOLON sec-mechanism *(COMMA sec-mechanism)

security-verify		= "Security-Verify" HCOLON sec-mechanism *(COMMA sec-mechanism)

sec-mechanism		= mechanism-name *(SEMI mech-parameters)

mechanism-name		= "ipsec-3gpp" / "tls"

mech-parameters		= ( preference / algorithm / protocol / mode / encrypt-algorithm / spi-c / spi-s / port-c / port-s )

preference				= "q" EQUAL qvalue

qvalue					= ( "0" [ "." 0*3DIGIT ] ) / ( "1" [ "." 0*3("0") ] )

algorithm				= "alg" EQUAL ("aes-gmac" / "null" )

protocol				= "prot" EQUAL ( "ah" / "esp" )

mode					= "mod" EQUAL ( "trans" / "tun" / "UDP-enc-tun"  )

encrypt-algorithm	= "ealg" EQUAL ("aes-gcm" / "null" )

spi-c					= "spi-c" EQUAL spivalue

spi-s					= "spi-s" EQUAL spivalue

spivalue				= 10DIGIT; 0 to 4294967295

port-c					= "port-c" EQUAL port

port-s					= "port-s" EQUAL port

port					= 1*DIGIT

The changes compared to RFC 3329 [21] are:

"alg" parameter: Addition of "aes-gmac" and "null". Removal of "hmac-md5-96" and "hmac-sha-1-96"

"ealg" parameter: Addition of "aes-gcm". Removal of "des-ede3-cbc"

"mod" parameter: Addition of "UDP-enc-tun"

The use of security association parameters is specified in clauses 7.1, 7.2, M.7.1 and M.7.2 of the present document. The parameters described by the BNF above have the following semantics:

Mechanism-name: For manually keyed IPsec, this field includes the value "ipsec-3gpp". "ipsec-3gpp" mechanism extends the general negotiation procedure of RFC 3329 [21] in the following way:

1	The server shall store the Security-Client header received in the request before sending the response with the Security-Server header.

2	The client shall include the Security-Client header in the first protected request. In other words, the first protected request shall include both Security-Verify and Security-Client header fields.

3	The server shall check that the content of Security-Client headers received in previous steps (1 and 2) are the same.

Mech-parameters: Of the mech-parameters, only preference is relevant when the mechanism-name has the value "tls".

Preference: As defined in RFC 3329 [21].

Algorithm: Defines the authentication algorithm. The algorithm parameter is mandatory. The value "aes-gmac" refers to the authentication algorithm ENCR_NULL_AUTH_AES_GMAC defined in IETF RFC 4543 [74]. The value "null" shall only be used with encryption algorithm "aes-gcm".

Protocol: Defines the IPsec protocol. May have a value "ah" or "esp". If no Protocol parameter is present, the value will be "esp".

NOTE 1:	According to clause 6 only "esp" (RFC 4303 [54]) is allowed for use in IMS.

Mode: Defines the mode in which the IPsec protocol is used. May have a value "trans" for transport mode, and value "tun" for tunneling mode. If no Mode parameter is present, the value will be "trans".

NOTE 2:	Void.

Encrypt-algorithm: If present, defines the encryption algorithm. The value "aes-gcm" refers to the encryption algorithm AES-GCM with a 16 octet ICV defined in IETF RFC 4106 [73]. If no Encrypt-algorithm parameter is present, the algorithm will be "null". The value "aes-gcm" shall only be used with authentication algorithm equal to "null".

Spi-c: Defines the SPI number of the inbound SA at the protected client port.

Spi-s: Defines the SPI number of the inbound SA at the protected server port.

Port-c: Defines the protected client port.

Port-s: Defines the protected server port.

It is assumed that the underlying IPsec implementation supports selectors that allow all transport protocols supported by SIP to be protected with a single SA.

*** NEXT CHANGE ***

Annex I (normative):
Key expansion functions for IPsec ESP

Integrity Keys:

-

If selected authentication algorithm is AES-GMAC as specified in RFC 4543 [74] with 128 bit key then IKESP = IKIM.

The salt value specified in Section 3.2 of RFC 4543 [74] shall be derived using the key derivation function KDF defined in Annex B of TS 33.220 [66]. The input Key to the KDF function shall be equal to the concatenation of CKIM and IKIM: CKIM || IKIM. The input S to the KDF function shall be formed from the following parameters:

-	FC = 0x58.

-	P0 = "AES_GMAC_SALT" .

-	L0 = length of the string “AES_GMAC_SALT” (i.e. 0x00 0x0D).

The salt value shall consist of the 32 least significant bits of the 256 bits of the KDF output.

Encryption Keys:

-

If selected encryption algorithm is AES-GCM as specified in RFC 4106 [73] with 128 bit key then CKESP = CKIM. The salt value specified in Section 4 of RFC 4106 [73] shall be derived using the key derivation function KDF defined in Annex B of TS 33.220 [66]. The input Key to the KDF function shall be equal to the concatenation of CKIM and IKIM: CKIM || IKIM. The input S to the KDF function shall be formed from the following parameters:

-	FC = 0x59

-	P0 = “AES_GCM_SALT”

-	L0 = length of the string “AES_GCM_SALT” (i.e. 0x00 0x0C)

The salt value shall consist of the 32 least significant bits of the 256 bits of the KDF output.

*** NEXT CHANGE ***

# N.1	SIP Digest

SIP Digest authentication and the requirements in this Annex shall not apply to access networks defined in 3GPP specifications. The P-CSCF can enforce this condition by identifying REGISTER requests relating to SIP Digest according to the rules in Annex P.3 of the present document and discarding them when received over an access network defined in 3GPP specifications.

The provisions in Annex N are optional for implementation. The provisions in Annex N are optional for use. However, the use of one of the authentication mechanisms in the present document is mandated.

SIP Digest shall not be used in conjunction with IPsec.

NOTE 1: The use of SIP Digest in conjunction with IPsec, as specified in the main body and in Annex N of this specification, is technically impossible because SIP Digest does not generate session keys for use with IPsec security associations.

An additional scheme for authentication is SIP Digest as specified in RFC 3261 [6]. SIP Digest achieves mutual authentication between the UE and the HN, and is based on HTTP Digest as specified in RFC 7235 [Z7] and RFC 7616 [Z9]. The identity used for authenticating a subscriber is the private identity, IMPI, which has the form of a NAI. The HSS and the UE share a preset secret (e.g., a password) associated with the IMPI. The generation of the authentication challenge shall be done in the same way as specified in RFC 7235 [Z7] and RFC 7616 [Z9] and the present document.

NOTE 1x:	The use of MD5 in SIP Digest is not recommended and only supported for interop with Rel-16 and older releases.

It is the policy of the HN that decides if an authentication shall take place for the registration of an additional IMPU that is not part of the already registered set of IMPUs associated with the same IMPI.

If a UE supports SIP Digest as well as further authentication methods, the UE shall proceed as follows:

-	If the access network is of a type defined in 3GPP specifications then the UE shall not select SIP Digest, in accordance with the requirement at the start of this clause.

NOTE 2: The rules listed in Annex T of this specification say how a UE can select between IMS AKA and GIBA.

-	If the access network is of a type not defined  in 3GPP specifications then

-	if both the UE and network support IMS AKA according to the main body or Annex M of this specification, as determined by the use of sip-sec-agree RFC 3329 [21], the authentication method shall be IMS AKA;

-	otherwise the authentication method shall be SIP Digest as specified in Annex N of this specification.

*** NEXT CHANGE ***

## N.2.1	Authentication Requirements

### N.2.1.1	Authentication Requirements for Registrations

For the purposes of this subclause, the name "authentication" is used synonymously with "entity authentication".

Before a user can get access to the IM services at least one IMPU needs to be registered and the IMPI authenticated in the IMS at application level. In order to get registered the UE sends a SIP REGISTER message towards the SIP registrar, i.e. the S-CSCF, cf. figure N.1, which will perform the authentication of the user. The message flows are the same regardless of whether the user has an IMPU already registered or not.

Figure N.1: The IMS Authentication using SIP Digest for an unregistered IM subscriber and successful mutual authentication

The detailed registration procedures are defined in TS 24.229 [8].

The NAT traversal procedures in RFC 5626  [32] and in TS 24.229 [8] clause K.4 shall apply.

NOTE 1: It is recognized that RFC 5626 [32] can be useful for capabilities beyond NAT traversal (e.g. multiple registrations) however this annex does not consider such capabilities at this time.

The UE should include an indication of support for managing client-initiated connections as defined in RFC 5626 [32] in all REGISTER requests. Per RFC 5626 [32], the P-CSCF shall be able to accept registration request with or without an indication of support for managing client-initiated connections. However, the P-CSCF should only accept a register request without support for managing client-initiated connections if it can determine that no NAT is present in the signaling path between the UE and the P-CSCF.

NOTE 2: It is left to stage 3 specifications how a P-CSCF can determine whether the conditions in the preceding paragraph are met. An operator may configure all UEs and P-CSCFs in his network not to use support for managing client-initiated connections (provided there is no roaming). Cf. also the implications of the indication of support for managing client-initiated connections for the P-CSCF procedures after receiving SM11.

SMn stands for SIP Message n and CMm stands for Cx message m which has a relation to the authentication process:

SM1: REGISTER(IMPI*, IMPU)

“IMPI*” in SM1 means that the inclusion of the IMPI is optional in SM1.

NOTE 2a:	When a registering UE omits the IMPI from the REGISTER request, the IMPI for the registration is derived from the registering IMPU. Since there can be only one registered instance of an IMPI at any point in time, the registering IMPU in this case cannot be shared across multiple UEs.

In SM2 and SM3 the P-CSCF and the I-CSCF respectively forwards the SIP REGISTER towards the S-CSCF. If SM1 does not contain an IMPI, the P-CSCF shall behave according to Annex P.3 and forward the message as SM2 to the I-CSCF.

NOTE 2b: Annex P.3 formulates conditions depending on the presence of an Authorization header. Note that, if SM1 does not contain an IMPI, then SM1 does not contain an Authorization header.

The I-CSCF queries the HSS to find the address of the S-CSCF. If SM2 does not contain an IMPI the I-CSCF shall derive the IMPI from the IMPU in the REGISTER request as described in 3GPP TS 24.229 [8]. Then the I-CSCF forwards the message as SM3 to the S-CSCF.

After receiving SM3, if the IMPU is not currently registered at the S-CSCF, the S-CSCF needs to set the registration flag at the HSS to initial registration pending. This is done in order to handle UE terminated calls while the initial registration is in progress and not successfully completed. The registration flag is stored in the HSS together with the S-CSCF name and user identity, and is used to indicate whether a particular IMPU of the user is unregistered or registered at a particular S-CSCF or if the initial registration at a particular S-CSCF is pending. The registration flag is set by the S-CSCF sending a Cx-Put to the HSS. If the IMPU is currently registered, the S-CSCF shall leave the registration flag set to registered. At this stage the HSS has performed a check that the IMPI and the IMPU belong to the same user.

The S-CSCF shall determine the type of authentication based on the rules in Annex P. If SM3 does not contain an IMPI the S-CSCF shall derive the IMPI from the IMPU in the REGISTER request as described in 3GPP TS 24.229 [8]. If the IMS registration request is related to SIP Digest, then the procedures below apply.

Upon receiving the SIP REGISTER the S-CSCF shall use a SIP Digest Authentication Vector (SD-AV) for authenticating the user. If the S-CSCF has no valid SD-AV for the specific IMPI, then the S-CSCF shall send a request for SD-AV(s) to the HSS in CM1 where the number m of SD-AVs wanted is equal to 1.

| CM1: Cx-AV-Req(IMPI, m) |  |
| --- | --- |

Upon receipt of a request from the S-CSCF, the HSS sends one SD-AV to the S-CSCF using CM2. The SD-AV consists of the qop (quality of protection) value, the authentication algorithm, realm, and a hash, called H(A1), of the IMPI, realm, and password. Refer to RFC 7235 [Z7] and RFC 7616 [Z9] for additional information on the values in the authentication vector for SIP Digest based authentication.

The qop value shall be set to "auth" since SIP Digest, as used in IMS, can only provide authentication, not message integrity.

| CM2: Cx-AV-Req-Resp(IMPI, realm, algorithm, qop, H(A1) ) |  |
| --- | --- |

The S-CSCF generates a random nonce, stores H(A1) and the nonce against the IMPI, and then sends a SIP 401 Auth_Challenge i.e., an authentication challenge towards the UE including the nonce in SM4. It also includes the realm, qop and algorithm parameters. RFC 7235 [Z7] and RFC 7616 [Z9] specifies how to populate the parameters of a 401 Auth_Challenge.

SM4: 401 Auth_Challenge(IMPI, realm, nonce, qop, algorithm)

The I-CSCF forwards the SIP 4xx Auth_Challenge message towards the P-CSCF as SM5.

When the P-CSCF receives SM5 it shall forward the message to the UE.

SM6: 401 Auth_Challenge(IMPI, realm, nonce, qop, algorithm)

Upon receiving the challenge, SM6, the UE generates a cnonce. It then uses the cnonce as well as parameters provided in the SM6 such as nonce and qop to calculate an authentication response according to RFC 7235 [Z7] and RFC 7616 [Z9].  This response and other parameters are put into the Authorization header and sent back towards the network in SM7. The inclusion of the IMPI and an Authorization header in SM7 are mandatory.

SM7: REGISTER(IMPI, realm, nonce, response, cnonce, qop, nonce-count, algorithm, digest-uri)

NOTE 3: As specified in RFC 3261 [6], when  the P-CSCF receives a SIP request from the UE, the P-CSCF checks the IP address in the "sent-by" parameter of the Via header field provided by the UE. If the "sent-by" parameter contains a domain name, or if it contains an IP address that differs from the packet source IP address, the P-CSCF adds a "received" parameter to that Via header field value. This parameter contains the source IP address from which the packet was received.

The P-CSCF forwards the authentication response in SM8 to the I-CSCF, which queries the HSS to find the address of the S-CSCF. In SM9 the I-CSCF forwards the authentication response to the S-CSCF.

Upon receiving SM9 containing the response, the S-CSCF calculates the expected response using the previously stored H(A1) and stored nonce together with other parameters contained in SM9 (e.g.,  cnonce, nonce-count, qop, as specified in RFC 7235 [Z7] and RFC 7616 [Z9]) and uses this to check against the response sent by the UE. If the check is successful then the user has been authenticated and the IMPU is registered in the S-CSCF. If the IMPU was not currently registered, the S-CSCF shall send a Cx-Put to update the registration-flag to registered. If the IMPU was currently registered the registration-flag is not altered.

NOTE 4: Depending on its local security policy, the S-CSCF may delete H(A1) immediately after checking the Digest response, but this may then lead to an increased exposure of H(A1) on the Cx-interface as H(A1) would then have to be fetched from the HSS more often.

It shall be possible to implicitly register IMPU(s) (see clause 4.3.3.4 in TS 23.228 [3]). All the IMPU(s) being implicitly registered shall be delivered by the HSS to the S-CSCF and subsequently to the P-CSCF. The S-CSCF shall regard all implicitly registered IMPU(s) as registered IMPU(s).

When an IMPU has been registered this registration will be valid for some period of time. Both the UE and the S-CSCF will keep track of a timer for this purpose but the expiration time in the UE is smaller than the one in the S-CSCF in order to make it possible for the UE to be registered and reachable without interruptions. A successful registration of a previously registered IMPU (including implicitly registered IMPUs) means the expiry time of the registration is refreshed.

If the user has been successfully authenticated, the S-CSCF sends a SM10 SIP 2xx Auth_OK message to the I-CSCF indicating that the registration was successful. The 2xx Auth_OK message contains the Authentication-Info header with a response digest as specified in RFC 7235 [Z7] and RFC 7616 [Z9]. The response digest allows the UE to authenticate the HN.

In SM11 the I-CSCF forwards the SIP 2xx Auth_OK towards the P-CSCF.

The P-CSCF associates the UE's packet source IP address along with the "sent-by" parameter of the Via header, cf. RFC 3261 [6], of the REGISTER message with the IMPI and all the successfully registered IMPUs related to that IMPI. If managing of client-initiated connections as defined in RFC 5626 [32] is used then the P-CSCF shall also include the UE's packet source port of the REGISTER message as part of the association. The P-CSCF stores the associated parameters in an IP address check table. If managing of client-initiated connections is not used then the P-CSCF shall overwrite any existing entry in the IP address check table which has the same IP address, but a different IMPI. If managing of client-initiated connections is used then the P-CSCF shall overwrite any existing entry in the IP address check table which has the same (IP address, port) pair, but a different IMPI.

The P-CSCF forwards the SIP 2xx AUTH_OK towards the UE.

NOTE 5: If a P-CSCF associated the port with the IMPI even when managing of client-initiated connections was not used then the UE would be unnecessarily restricted in opening new connections during a registration. The restriction is unavoidable in the presence of NAT.

Upon receiving SM12, the UE shall calculate the expected response from the HN as described in RFC 7235 [Z7] and RFC 7616 [Z9]. To authenticate the HN, the UE shall compare its expected response to the response provided by the HN. If the comparison fails the UE shall abort the communication.

### N.2.1.2	Authentication Requirements for Non-registration Messages

For the purposes of this subsection, the name "authentication" is used synonymously with "message origin authentication".

The IP address check table (cf. subclause N.2.1.1) shall be used by the P-CSCF to identify the initiator of subsequent requests as follows: one of the public user identities associated with the packet IP address (and port if applicable) is selected and asserted to the S-CSCF according to the rules in TS 24.229 [8], subclause 5.2.6.3.

In addition, subsequent requests (e.g. INVITE) may be authenticated with SIP Digest, as described in the following:

NOTE 1: The assertion of IMPUs based on checks of IP address (and ports if applicable) provides a reasonable level of security only in environments where the risk from source IP address and port spoofing or from IP address re-assignment unnoticed by the SIP application is sufficiently low. If the environment does not fulfill this condition then it is recommended to use SIP Digest in conjunction with either TLS, as specified in Annex O of this specification, or with the SIP Digest proxy authentication mechanism as specified in this subclause. It is not part of this specification to determine which environments fulfill the conditions in this NOTE. This is left to specifications, possibly maintained by standardization bodies other than 3GPP, describing these environments. More details on the usage of the authentication mechanisms for non-registration messages are provided in Annex Q (informative).

When the S-CSCF receives a SIP request with a method other than the REGISTER method from the UE, the S-CSCF may perform authentication on the SIP request according to the operator's policy and according to the following procedures.

- If the request does not contain a Proxy-Authorization header or the Proxy-Authorization header does not contain a digest response the S-CSCF shall send a 407 (Proxy Authentication Required) response to challenge the UE. The 407 response shall contain digest challenge parameters in a Proxy-Authenticate header as defined by RFC 7615 [Z9]. The challenge parameters, with the exception of the nonce, shall be taken from the same SD-AV as used for the last successful registration or re-registration message of the UE. The nonce shall be generated freshly by the S-CSCF. Upon receiving the challenge the UE  shall extract digest challenge parameters from the Proxy-Authenticate header field and calculate a digest response as indicated in RFC 7235 [Z7] and RFC 7616 [Z9]. The UE should store the received digest challenge. The UE then sends a new request to the network containing a Proxy-Authorization header in which the header fields are populated as described in RFC 7235 [Z7] and RFC 7616 [Z9] using the calculated digest response. Upon receiving the new request which contains a digest response, the S-CSCF verifies the user’s identity by validating the digest response information (e.g. the nonce-count) contained in the Proxy-Authorization header field against the expected information based on the same SD-AV as used for generating the challenge;

NOTE 1a: Authorization (used for registration messages, cf. sub-clause N.2.1.1) and Proxy-Authorization (used for non-registration messages, this sub-clause) are handled by logically separated protocol engines and thus each mechanism has its own nonce, cnonce and nonce-count parameters.

NOTE 1b: The usage of the same SD-AV for authentication of non-registration messages and of registration messages requires the storage of the SD-AV in S-CSCF during the authentication of registration messages (cf. subclause N.2.1.1), as retrieval of AVs from HSS is only specified for handling of registration messages. In case of dynamic password change (cf. clause N.2.5), the SD-AV (or SD-AVs) used for generating the challenge(s) are specified in clause N.2.5.

- If the check is successful then the request has been authenticated, and the S-CSCF sends a 2xx AUTH_OK towards the UE;

- If the check fails, based on local policy the S-CSCF may choose to re-challenge the user by using the same procedure described in this subclause, or reject the request by sending a 403 response.

When the UE is to send a non-REGISTER SIP request it should first check whether it has a digest challenge stored which was previously received in a Proxy-Authenticate header. If such a digest challenge is available in the UE the UE should use it together with the nonce-count mechanism as specified in RFC 7235 [Z7] and RFC 7616 [Z9] to calculate a digest response, include the digest response in a Proxy-Authorization header and send this header together with the non-REGISTER SIP request.

NOTE 2: According to RFC 7235 [Z7] and RFC 7616 [Z9], the S-CSCF may send a 407 (Proxy Authentication Required) as a response to any non-REGISTER request, indicating that the nonce is stale and the digest response shall be recomputed using the fresh challenge sent in the same 407 message.

When the S-CSCF has successfully used the SIP Digest proxy authentication mechanism it shall check if the public user identity asserted by the P-CSCF belongs to the implicit registration set (i.e. the public user identities associated with the authenticated user). If the check is not successful the S-CSCF shall reject the non-registration request.

NOTE 3: Such a rejection may occur when one of the conditions mentioned in NOTE 1 is not fulfilled.

NOTE 4: When TLS according to Annex O is used, or when IPsec according to the main body or Annex M is used, then the failure conditions mentioned in NOTE 1 and Annex Q.3 cannot occur, and the public user identity asserted by the P-CSCF is reliable.

*** NEXT CHANGE ***

## O.2.1	TLS Profile for TLS based access security

When the UE and the P-CSCF implement and use TLS as specified in the present Annex O, TLS shall be implemented and used according to the TLS profile specified in TS 33.310 [24], Annex E. For all TLS versions the provisions on ciphersuites given in TS 33.310 [24], Annex E, shall apply.

-

-	Authentication of the P-CSCF

-	The P-CSCF shall be authenticated by the UE by presenting a valid server certificate.  The P-CSCF certificate profile shall be based on TLS certificates as presented in clause O.5.1.

-	Authentication of the UE

-	The P-CSCF shall not request a certificate in a Server Hello Message from the UE.  The HN shall authenticate the UE as specified in Annex N of this specification.

-	Verification of the TLS session endpoints

-	In order for the UE to be able to trust the TLS session endpoint, the P-CSCF certificate shall be used during the authentication procedure.

-	In order for the P-CSCF to be able to trust that the UE, which was authenticated according to Annex N, is the TLS session endpoint, the P-CSCF shall use the mechanism for associating the TLS Session ID with registration parameters IP address, port, IMPI, IMPU(s), specified in clause O.2.2, and shall have assurance that man-in-the-middle attacks can be mitigated, e.g. by following the rules in the NOTE in clause O.1.1.

-	TLS session parameters

- The TLS Handshake Protocol negotiates a session, which is identified by a Session ID.

-	The lifetime of a Session ID is subject to local policies of the UE and the P-CSCF. A recommended lifetime is one hour (or at least more than the re-REGISTRATION time out). The procedure for TLS session re-negotiation in IMS is specified in clauses O.4.1 and O.4.2.

-	Ports

-	The P-CSCF shall be prepared to accept TLS session requests on port 5061 or on a port published by the operator.

-	Forwarding requests

-	The procedures for forwarding requests by the edge proxy in RFC 5626 [32] shall apply to the P-CSCF when managing TLS connections.

NOTE 1: The use of RFC 5626 [32] in conjunction with TLS is needed so that terminating requests can re-use an existing TLS connection.

*** NEXT CHANGE ***

## S.5.3	Intra-domain Domain Security

The interface labeled 5 in Figure S.1 is between SIP-capable nodes in the same network security domain.  The interface labeled 3 in Figure S.1 is between the I-CSCF/S-CSCF and the HSS. There may be other interfaces to nodes inside the Home Network, which are also intended to be covered by this clause.  As these interfaces exist entirely within one network security domain, the administrative authority may choose any mechanism to secure this interface, including physical security where appropriate. Cryptographic methods of security, if applied, shall include both privacy and integrity protection, and be at least as strong as the IPsec (RFC 4301 [53], RFC 7296 [Z6]) profile defined in clause S.5.4.

*** NEXT CHANGE ***

#### S.5.4.2.1	General

For the interfaces security protection between IMS network elements, this clause specifies the protection using IPsec as specified in RFC 4301 [53]. The key management and distribution architecture is based on the IPsec IKE (RFC 4301 [53], RFC 7296 [Z6]) protocols. IKEv2 shall follow the 3GPP IKEv2 profile as defined in clause 5.4 of TS 33.210 [5] and clause 6.2.1b of TS 33.310 [24].

The security services provided by network domain security are:

-	data integrity;

-	data origin authentication;

-	anti-replay protection;

-	confidentiality (optional);

-	limited protection against traffic flow analysis when confidentiality is applied.

The IPsec security protocol shall always be ESP. Integrity protection/message authentication together with anti-replay protection shall always be used. IPsec ESP should be used with both encryption and integrity protection for all SIP signaling traversing inter-security domain boundaries.

IPsec offers a set of security services, which is determined by the negotiated IPsec security associations. That is, the IPsec SA defines which security protocol to be used, the mode, and the endpoints of the SA.

*** NEXT CHANGE ***

#### S.5.4.2.2	Support of ESP authentication and encryption

For IMS signaling traffic, ESP shall always be used to provide data integrity, data origin authentication, and anti-replay protection services, thus the ESP_NULL authentication algorithm shall not be allowed for use. ESP shall follow the 3GPP ESP profile as defined in clause 5.3 of TS 33.210 [5].

*** NEXT CHANGE ***

### W.3.1.2	Firewall traversal for IMS control plane using SIP over TLS/TCP

Firewall traversal for IMS control plane is accomplished by running SIP over TLS and using port 443 (HTTPS) instead of the standard port 5061 (SIPS). This makes the SIP signalling appear as HTTPS traffic to any firewall that is present along the signalling path.

In order to ensure that the firewall pinholes are maintained, the IMS client -shall apply the keep-alive mechanism specified in RFC 5626 [32]. The keep-alive mechanism is negotiated by the IMS client and the P-CSCF at IMS registration using the method described in RFC 6223 [64]. Note that RFC 5626 defines two keep-alive techniques: a technique based on STUN for connection-less transports and a technique based on SIP (called CRLF) for connection-oriented transports. Since TCP is used as transport between the IMS client and the P-CSCF, the CRLF keep-alive technique must be used.

In case the IMS client is configured to use an HTTP proxy, the IMS client uses the HTTP CONNECT method (see RFC 2817 [63]) to request the proxy to establish a TCP connection with the P-CSCF on its behalf.  Once the client has received a positive reply from the proxy that the TCP connection has been established, the client initates the TLS handshake with the P-CSCF and establishes the TLS tunnel. Note that the use of the HTTP CONNECT method is completely transparent to P-CSCF.

Editor’s note: It needs to be verified that this does not interfere with the HTTP proxy settings on the UE.

*** NEXT CHANGE ***

### W.3.1.3	Firewall traversal for IMS media plane using ICE and TURN

Firewall traversal for IMS media plane is accomplished by using the ICE protocol together with an enhanced version of TURN. ICE is defined in RFC 8445 [Z3] and RFC 8839 [Z4] and is a protocol for performing NAT traversal of UDP based media streams. ICE in turn makes use of TURN, defined in RFC 5766 [60], which is a protocol for relaying media through a relay server. An IMS client that supports ICE will allocate relayed candidates at the TURN server and include the candidate information in the SDP offer/answer sent to the peer. The relayed candidates will be used as a last resort when the client and peer fail to establish a direct communication path. The communication between the client and the TURN server (this includes both the relayed media and the control information needed to setup the relayed candidates) can occur over UDP, TCP or TCP/TLS. By using TCP/TLS on port 443 (HTTPS) or TCP on port 80 (HTTP) the communication will appear as HTTP(S) to firewalls and will (typically) be allowed through. Using TCP instead of TLS/TCP reduces the overhead but will fail when the firewall performs DPI or if an HTTP proxy is present. An IMS client may be configured to use both TURN over TCP/80 and TURN over TLS/443, in such case, the client should prefer to use TURN over TCP/80 to avoid TLS overhead.

ICE and TURN have later on been extended to also support TCP based media. ICE TCP is defined in RFC 8445 [Z3] and RFC 8839 [Z4] and TURN TCP is defined in RFC 8656 [Z2]. One of the changes introduced in TURN TCP is that the multiple TCP connections are established between the client and TURN server: one for exchange of control information and one for each relayed TCP based media stream. All UDP based media streams are relayed over the same TCP connection that is used for the control information, just as in the original TURN protocol. The TURN server will use TCP/TLS on port 443 (HTTPS) or TCP over port 80 (HTTP) for all the connections. In order to reduce the TLS setup time when several TCP connections are established, the IMS client and TURN server may use the TLS session resumption feature.

An IMS client that is configured to use an HTTP proxy uses the HTTP CONNECT method (see RFC 2817 [63]) to request the proxy to establish a TCP connection with the TURN server. Once the client has received a positive reply from the proxy that the TCP connection has been established, the client initates the TLS handshake with the TURN server and establishes the TLS tunnel. This procedure is repeated once for every TCP connection the client establishes with the TURN server. Note that the use of the HTTP CONNECT method is completely transparent to TURN server.

Using ICE for firewall traversal is particularly suitable for IMS clients that already implement ICE for NAT traversal, since in this case only minimal changes are required to the client. Usage of ICE for IMS clients is specified in TS 23.228 [3] and TS 24.229 [8].

Note that there is no need to specify any keep-alive mechanism since this functionality is already included in ICE. The IMS client will send regular STUN keep-alives which ensures that the firewall pinholes are maintained.

Editor's note: ICE TCP is required for TCP based media (e.g. MSRP) but is not yet supported in TS 23.228 [3] and TS 24.229 [8]. These specifications need to be updated.

Editor's note: How the client is authenticated and authorized by the TURN server is ffs. One possibility is to use the SIP Digest credentials and the normal TURN authentication procedure. However, this would require an additional interface between the TURN server and the HSS. Another possibility is to use GBA but this would perhaps be unnecessarily complex considering that the only attack we need to protect against is DoS.

*** NEXT CHANGE ***

## W.3.3	Required functions of the UE

For firewall traversal of IMS control plane, the IMS client shall implement the following functionality:

-	support SIP over TLS/TCP on the non-standard port 443 (HTTPS);

-	support the SIP Digest authentication method according to Annex N;

-	support the CRLF keep-alive technique defined in RFC 5626 [32] together with the negotiation mechanism defined in RFC 6223 [64];

-	support the HTTP CONNECT method in RFC 2817 [63] for establishing the TLS tunnel with the P-CSCF when the IMS client is configured with an HTTP proxy.

For firewall traversal of IMS media plane, the IMS client shall implement the following functionality:

-	support ICE for UDP and TCP based media streams according to Annex G of TS 23.228 [3];

-	support TLS/TCP on non-standard port 443 and TCP on non-standard port 80 for communication with TURN server;

-	support the HTTP CONNECT method in RFC 2817 [63] for establishing TLS tunnels with the TURN server when the IMS client is configured with an HTTP proxy.

Note that the HTTP CONNECT method is only used when the IMS client is configured with an HTTP proxy for outgoing HTTP(S) requests. The way in which the IMS client obtains the proxy address and port is out of scope.

*** NEXT CHANGE ***

### X.2.2.3	Procedures

Figure X.2.3-1 shows the registration flow. In this figure SIP over secure WebSocket is used between the WIC and the eP-CSCF. Other protocols (e.g. HTTP RESTful or JSON over WebSocket) can also be used as long as it is able to relay the digest challenge, challenge-response, and auth-info values.

Solution 1.1 requires that the IMPU and SIP Digest password are made available to the JavaScript in the WIC. The IMPI can be omitted from the initial SIP Register request, and if that is the case the S-CSCF will try to determine its value from the registering IMPU. This requires that IMPUs are not shared between IMS users (see Annex N).

NOTE 1:	It is assumed that the credentials are entered by the user via the web GUI or retrieved from the WWSF over HTTPS. Note that the latter option requires that WWSF has authenticated the user previously.

NOTE 2:	Unless the SIP Digest password or the intermediate hash value H(A1) (see RFC 7235 [Z7] and RFC 7616 [Z9]) is stored in the WIC, the password needs to be re-obtained each time a re-registration is performed. If the password is entered manually and if re-registrations occur often, this will result in a negative user experience.  This can be avoided by storing the SIP Digest password or H(A1) in the WIC after the initial registration procedure. Ensuring the confidentiality of the SIP Digest password or H(A1) during storage is at the discretion of the implementation and is outside the scope of 3GPP. The use of MD5 in HTTP Digest is not recommended and only supported for interop with Rel-16 and older releases.

NOTE 3:	It is recommended that the user does not enter his SIP Digest credentials into the WIC, except possibly once before the initial registration.

Figure X.2.2.3-1: WebRTC IMS Client authentication using SIP Digest

The details of the signalling flows are as follows:

1)	Web page download from WWSF

From within a WebRTC-enabled browser, the user accesses a URI to the WWSF to initiate an HTTPS connection to the WWSF. The TLS connection provides one-way authentication of the server based on the server certificate. The browser downloads and initializes the WIC from the WWSF.

2)	Establishment of secure Web socket connection between WIC and eP-CSCF

The WIC opens a WSS (secure Web Socket) connection to the eP-CSCF. The TLS connection provides one-way authentication of the server based on the server certificate. The eP-CSCF verifies in this step that the WIC establishing the signalling connection comes from a trusted domain.

NOTE 3:	The protection mechanism works under the assumption that the browser is not under the attacker's control.

3-10)	SIP Digest message flow

The SIP Digest messages exchanged between the WIC and eP-CSCF and between the eP-CSCF and the I/S-CSCF are as defined in Annex N of this document.

*** NEXT CHANGE ***

## X.5.1	Introduction

TURN RFC 8656 [Z2] specifies that TURN servers and clients MUST implement "Long-Term Credential Mechanism" as specified in clause 10.2 of RFC 8489 [Z1]. In this mechanism, the client and server share a pre-provisioned username and password that remains in the system till the user is using the system.The TURN server uses these credentials to authenticate the client by performing digest challenge/response.

In IMS_WebRTC, the browser plays the role of a TURN client. The WIC (i.e. the Javascript code) controls the execution of the browser via the W3C defined RTCPeerConnection API. Through this API, the WIC provides TURN credentials to the browser. These credentials should therefore be made available to the WIC.

There are two known gaps that need to be addressed before TURN can be used in IMS_WebRTC:

1)	At present, the provisioning of TURN long-term credentials in the WIC is un-defined.

2)	Moreover, as indicated in RFC 7376 [68], ensuring secrecy of these credentials in a web-based application such as the WIC is difficult. Once these credentials are exposed to a Javascript script, it could lead to various security issues such as leak of the credentials, privacy leakage etc.

A solution is needed to dynamically configure the TURN credentials in the WIC while ensuring that security gaps identified by RFC 7376 [68] are addressed.

Two solutions are presented in this annex for TURN credential provisioning and authentication:a)	eP-CSCF based dynamic provisioning of credentials in the WIC and TURN server

b)	 TURN client authentication based OAuth 2.0 access tokens.

Both solutions are optional for implementation.

*** NEXT CHANGE ***

## X.5.2	Solution 1: TURN credential provisioning and authentication using eP-CSCF

### X.5.2.1	Overview

This solution reuses the TURN long-term credential method defined in RFC 8656 [Z2] , but the credential is dynamically provisioned by eP-CSCF via the signalling channel.  When WIC registers to IMS, WIC requests the IMS networks to provision a credential for TURN authentication using a 3GPP extension header. If the request is authenticated and authorized, eP-CSCF generates a TURN credential, including user id, password, expiration, etc,  and sends the credential to WIC in the response message. Since the signaling messages between WIC and eP-CSCF are protected by the secure protocols, e.g. secure Websocket, the TURN credential is securely transferred to WIC. The WIC retrieves the credential and uses it in subsequent TURN allocation requests. The WIC may request  TURN credential for every registration, or use the credential until it expires. WIC can also use other signaling messages such as OPTION to request a new credential at anytime before re-registration.

This method requires some enhancement of WIC and eP-CSCF.  WIC needs to be enhanced to use the 3GPP extension header to request TURN credential from eP-CSCF via signaling messages. The eP-CSCF needs to be enhanced to process TURN credential request and generate TURN user name and password using a preshared key with the TURN server. The TURN server also needs to be enhanced to re-generate TURN password from username in TURN request and the preshared key with P-CSCF.

This solution provides a way to provision TURN credential in large scale with minor change to existing functions.  It addresses the security issues in 8489 [Z1] by dynamically generating TURN user name and password.  This solution is optional to support.  When to use this solution depends on WebRTC deployment scenario and operator’s policy.  For example, if a deployment does not have WAF, or if the WAF or TURN server does not support TURN access token, the eP-CSCF based approach may be used for TURN credential provision and authentication since the alternative solution requires the use of WAF and support of TURN access token by TURN server, WAF.

### X.5.2.2	Procedures

The procedure of TURN credential provision via eP-CSCF is shown in figure X.5.2.2.1. To use this solution, a shared secret key Km should be configured between eP-CSCF and TURN server using out of band method not defined in this solution.

1)	WIC establishes secure websocket with eP-CSCF

2)	WIC sends REGISTER request to eP-CSCF with 3GPP extended header (3gpp-ext-turn-cred) for TURN credential request.

3)	eP-CSCF authenticate and authorized the request then

1)	 Generate a random user ID Tid and credential expiration time Texp based on its policy, Tid and Texp should be encoded as string type.

2)	 Generate TURN password using Km,  Tpwd = HASH (Km , Tid : Texp )

4)	eP-CSCF sends REGISTER response with generated TURN credential using the 3gpp extended header.   The valud of the header  = (Tid : Texp : Tpwd),  which is the concatenation of Tid, Texp, Tpwd  separated by semi-colon sign.

5)	IC extracts the TURN credential from the TURN credential header.  The WIC uses (Tid : Texp) as TURN USERNAME and uses Kpwd to compute the MESSAGE-INTEGRITY value  of TURN Allocate request as defined in IETF RFC 8656 [Z2].

6)	TURN server re-generates TURN password using the USERNAME attribute in request and the preshared key with eP-CSCF

7)	TURN server validates that the credential has not expired and verifies the integrity of the TURN request using the password generated in step 6.

8)	TURN server sends Allocate response with allocated relay address to WIC

The procedure above uses REGISTER as example to explain how to request TURN credential from eP-CSCF by signaling messages. WIC may use other signaling messages, such as OPTIONAL, to request TURN credential.

Figure X.5.2.2-1: TURN Credential provisioned by eP-CSCF

*** END OF CHANGES ***