3GPP TSG-SA3 Meeting #104e-Ad-hoc 	S3-213400

- e-meeting, 27-30 Sept 2021  											                     Revision of S3-20xxxx

- Source:	Huawei; HiSilicon; KPN

- Title:	Add Conclusion to Key Issue #12 about unicast communication and key refresh

- Document for:	Approval

Agenda Item:	5.2

# 1	Decision/action requested

Kindly Request this contribution on TR 33.847 can be accepted by the group

# 2	References

N/A

# 3	Rationale

This contribution aims to update the conclusion of Key issue #12.

# 4	Detailed proposal

**** 1st change****

## 7.12	Key Issue #12: Security of one-to-one communication over PC5

For Key Issue #12, the followings are taken as conclusions:

-	It is concluded that the security mechanism defined in TS 33.536 [8] is taken as the baseline for normative work with the following additional change:

-	It is concluded for the PC5 signalling security, that the security mechanism in Solution #7 is taken as the basis for normative work.

NOTE: Under what scenario the PC5 requires unprotected signalling shall be defined during normative phase.

NOTE: Privacy protection of UE IDs will be defined in the normative phase?

Further enhancements can be discussed and added during normative phase.

-	The security mechanism in Solution #23 is taken as basis for normative work to provide means for secure refresh of UE security context: during the direct discovery with 5G DDNMF for direct communication, the UE gets the keys for PC5 connection and the associated key validity time.

NOTE: Further details for secure refresh of UE security context will be finished during the normative phase.

**** End of changes****