- 3GPP TSG-SA3 Meeting #104e-Ad-hoc	draft_S3-213472-r2

- e-meeting, 27 – 30 September 2021	revision of S3-21xabc

- Source:	CATT

- Title:	pCR to 5G ProSe draft TS Clause 4-Overview

- Document for:	Approval

Agenda Item:	4.8

# 1	Decision/action requested

Approve the pCR below

# 2		Rationale

This contribution proposes text of clause 4 of the 5G ProSe draft TS.

# 3	Detailed proposal

#### *** BEGIN CHANGES ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[i]	3GPP TS 23.304: " Proximity based Services (ProSe) in the 5G System (5GS)".

*** NEXT CHANGE ***

# 4	Overview

Editor’s Notes: This clause contains the overview of 5G ProSe security and links to other specifications, reference points and functional entities, etc.

## 4.1	General

The overall architecture for 5G ProSe is given in TS 23.304 [i]. 5G ProSe includes several features that may be deployed independently of each other. For this reason, no overall security architecture is provided and each feature describes its own architecture.

Security for the 5G ProSe common procedures is described in clause 5, while the overall security of the 5G ProSe features is described in clause 6.

## 4.2	Reference points and functional entities

PCx:	The reference point between the UE and the 5G ProSe Key Management Function. PCx relies on 5GC user plane for transport (i.e. an "over IP" reference point). It is used to transport security material to UEs for Groupcast mode 5G ProSe Direct Communication and 5G ProSe UE-to-Network Relay Communication.

#### *** END OF CHANGES ***