3GPP TSG-SA3 Meeting #104-e ad-hoc 	draft_S3-213575-r1

- e-meeting, 27 - 30 September 2021											Revision of S3-20xxxx

- Source:	Ericsson

- Title:	Conclusions for KI#2 on secure provisioning of PNI-NPN credentials

- Document for:	Approval

Agenda Item:		5.4

# 1	Decision/action requested

Appove the pCR below with conclusions for KI#2 (Provisioning of Credentials).

# 2	References

[1]	3GPP TR 33.857 "Study on enhanced security support for Non-Public Networks (NPN)"

[2]	3GPP TS 23.501: "System Architecture for the 5G System"

# 3	Rationale

This contribution is a re-submission of S3-211770 "Conclusions for KI#2 on secure provisioning of PNI-NPN credentials" from SA3#103-e, taking into account the new baseline in TR 33.857 v0.7.0.

It is related to clause 5.39.1 in TS 23.501 [2]: "The UE and the HPLMN may provide functionalities to provision or update the credentials used for NSSAA or credentials for secondary authentication/authorization to the UE. The provisioning via UE Parameters Update procedure as defined in clause 4.20 of TS 23.502 [3] and via User Plane are both supported."

UPU (UE Parameters Update) itself only provides integrity protection, not confidentiality protection. However, secured packet on top of UPU provides confidentiality protection as well.

# 4	Detailed proposal

***	BEGINNING OF CHANGES ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.501: "Security architecture and procedures for 5G System"

[3]	3GPP TR 23.700-07: "Study on enhanced support of non-public networks (Release 17)"

[4]		3GPP TS 23.501: "System Architecture for the 5G System"

[5]	IETF RFC 5281: "Extensible Authentication Protocol Tunneled Transport Layer Security             	Authenticated Protocol Version 0 (EAP-TTLSv0)"

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[7] 	IETF RFC 5216: "The EAP-TLS Authentication Protocol".

[x]	3GPP TS 31.115: "Secured packet structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications"

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

***	NEXT CHANGE ***

## 7.2	Conclusions on KI#2: Provisioning of Credentials

For secure control plane provisioning of PNI-NPN credentials of type symmetric key or password, secured packet as specified in TS 31.115 [x] on top of UPU (UE Parameter Update via UDM Control Plane Procedure, as specified in TS 23.502 [6]) can be used.

***	END OF CHANGES ***