3GPP TSG-SA3 Meeting #104-e 	S3-212592

# e-meeting, 16 - 27 August 2021											Revision of S3-20xxxx

# Source:	Huawei, HiSilicon

# Title:	A solution decoupling NSAC and NSSAA statuses

# Document for:	Approval

Agenda Item:		5.21 FS_eNS2_SEC

# 1	Decision/action requested

Include the solution in TR33.874

# 2	References

[1]

# 3	Rationale

This contribution proposes a new solution to address the KI on DoS to NSAC procedure.

# 4	Detailed proposal

pCR

***	BEGINNING OF CHANGES (all text are lnew) ***

## 6.Y	Solution #Y: Decouple NSAC and NSSAA statuses

### 5.1.1	Introduction

NSAC and NSSAA are two independent procedures, the solution proposes the outcomes of these two procedures are kept independent withouta negative impact to the efficiency to the NSSAA procedure.

### 5.1.2	Solution details

1. The AMF triggers the number of UEs per network slice availability check and update procedure as specified in the clause 4.2.11.2 in TS 23.501 [x].

2. As part of UE Registration procedure, the AMF sends a Registration Accept message to the UE in which the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI list for which the NSACF has indicated that the maximum number of UEs per network slice has been reached, and for each rejected S-NSSAI the AMF includes a reject cause set to 'maximum number of UEs per network slice reached' and optionally a back-off timer.

The AMF stores the reject cause, together with the NSSAA outcome if available. When the UE sends a registration request again, the AMF may skip the NSSAA if the NSSAA was successful.

### 5.1.3	Evaluation

TBC

***	END OF CHANGES	***