3GPP TSG-SA3 Meeting #105e	draft_S3-213911-r1

- e-meeting, 8 - 19 November 2021	Revision of S3-21xxxx

- Source:	InterDigital, LG Electronics

- Title:	TR 33.847 Conclusion for NSSAA support with L3 U2N

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

This contribution proposes conclusion text for support for NSSAA with L3 U2N in TR 33.847

# 2	References

[1]	TR 33.847 v0.8.0

# 3	Rationale

This document presents conclusions for the support for NSSAA via L3 UE2NW relay. There are two potential solutions for the support of NSSAA [1]: Solution #24 and Solution #34.

Sol#24:

- A new NSSAA procedure via the L3 U2N relay is introduced:

  - Relay UE's AMF triggers the NSSAA based on the Remote UE's NSSAI subscription.

  - The existing EAP authentication messages for NSSAA are expected to be reused as much as possible.

  - The Remote UE and the Relay UE need to support EAP authentication procedure over PC5 interface.

- CP based Remote UE authorization is required.

Sol#34:

- The existing NSSAA authentication mechanism via N3IWF is reused.

- N3IWF support is required.

Proposal:

- It is proposed to support NSSAA of the Remote UE via L3 UE-to-Network Relay as it may be required for the S-NSSAI associated with the RSC.

- It is proposed to select solutions #24 and #34 for the support for NSSAA, respectively without and with N3IWF support.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.847 [1].

***	BEGIN OF CHANGES	***

## 7.4	Key issue #4: Authorization in the UE-to-Network relay scenario

The solutions for U2N Relay authorization and security can be classified as user-plane (UP) or controlled-plane (CP) based solutions. The UP based solutions use a UP connection to a PKMF while CP based solutions use the primary authentication for PC5 keys establishment.

The following text is taken as conclusions for the UE-to-Network Relay solution:

-	For the control plane solution:

-	Baseline solution for Authorization for Remote UE/Relay is based on primary authentication (CP based approach, e.g., sol#1, #10, #15, #30) and using PCF based service authorization and provisioning as defined in [16] TS 23.304 clause 5.1.4.

-	Performing primary authentication during PC5 link establishment is supported (e.g. Sol#1, Sol#10, Sol#30).

NOTE 1:	The detailed procedure to enable authorization for Remote UE/Relay will be determined accordingly during normative phase. Additional support for Remote UE using its 5G-GUTI in DCR will be determined during normative phase.

-	For the user-plane solution, based on the conclusions in KI #3, it is concluded that the user-plane solutions including Solution #18, Solution #21 and Solution #29 are selected as the basis of normative work.

For the support of NSSAA it is concluded that both solutions without and with N3IWF are supported for L3 U2N relay. Solution #24 (without N3IWF) and Solution #34 (with N3IWF) are selected as the basis of normative work.

NOTE 2: The detailed procedure to enable NSSAA for Remote UE without N3IWF will be determined during normative phase with coordination with SA2.

Editor's note:	Final conclusion for secondary A&A is FFS.

***	END OF CHANGES	***