3GPP TSG-SA3 Meeting #105-e 	S3-214239-r1

e-meeting, 8 - 19 November 2021

- Source:	Samsung, Huawei, Nokia, Nokia Shanghai Bell

- Title:	New Solution for Key Issue #3: Authorization of notification endpoint

- Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

This contribution proposes to add a new solution for non-delegated subscribe-notification key issue.

# 2	References

[1]	TR 33.875 v040 Study on enhanced security aspects of the 5G Service Based Architecture (SBA)

# 3	Rationale

This contribution would like to add a new solution for service access authorization in “subscribe-notifiy” scenarios.

According to sub clause 13.4.1 in TS33.501, current OAuth2.0 based authorization framework verifies whether NF Service Consumer can be authorized to use specific service operations and/or resource data sets within service operations per NF Service Consumer type or NF instance ID. But, current solution is lack of functionality checking whether NF indicated by “Notification URI” is the authorized NF to be served by NF Service Producer. If a NF indicated by “Notification URI” is same with NF Service Consumer, it can be assured the NF is authorized by access token but if the NF is different from the NF Service Consumer, access token cannot assure the NF is authorized. It is because in Service Request, NF Service Consumer uses a “Notification URI” as a destination to the receive the Notification service and the NF Service Producer provides the service via “Notification URI” without further verification even though the owner of “Notification URI” may be an unauthorized NF which is different from NF Service Consumer.

Therefore, there shall be a solution to verify a NF which owns “Notification URI” for “subscribe-notify” service scenario.

# 4	Detailed proposal

It is suggested to approve the following changes.

***** Start of Changes*****

## 6.X	Solution #X: Authorization of notification endpoint in “Subscribe-Notify” scenarios

### 6.X.1	Introduction

This solution addresses key issue #3: Service access authorization in the “Subscribe-Notify” scenarios by including “notification URI” in the token request.

This solution proposes to include “notification URI” and CCA of the NF which owns the “notification URI” at the access token request. It will enable NRF to verify the access token request and authorize it.

### 6.X.2 Solution details

According to subclause 5.2.7 in TS23.502, NF shall register its own FQDN or IP address in NRF during NF Management NF register service operation.

During access token request process, NF Service Consumer include “notification URI” in Nnrf_Access Token_Get Request when operation semantics of the requested service type is “Subscribe/Notify”. NRF may verify whether the input parameters in the access token request match the corresponding information in the public key certificate of the NF Service Consumer or those in the NF profile of the NF Service Consumer. In addition, when “notification URI”(s) are included in the access token request, NRF may verify whether the “notification URI”(s) locate its location(s) with the same address(es) which are indicated by its own FQDN or IP address as registered in NRF. The NRF checks whether the NF Service Consumer is authorized to access the requested service(s).

When the NF Service Consumer is authorized, the NRF shall then generate an access token with appropriate claims which may include “notification URI” as requested by NF Service Consumer.

Figure 6.X.2.-1: Access token request for “Subscribe-Notify” NF Service illustration 1

A NF Service Consumer(NF_A) may request to subscribe to NF Service offered by a NF Service Producer(NF_B) on behalf of NF_C according to the Service Request from NF_C (e.g. Figure 4.15.3.2.2-1: Nudm_EventExposure_Subscribe, Unsubscribe and Notify operation in TS23.502). The Figure 6.X.2-2 describe the solution to verify the service request from NF_A including Notification URI of NF_C.

Figure 6.X.2-2: Access token request for “Subscribe-Notify” NF Service illustration 2

Step 0.	The NF_C sends a Service Request to the NF_A for notification service on an event with an access token including notification URI and CCA of the NF_C.

Step 1.	The NF_A decides to subscribe a service of the NF_B associated to the Service Request received at step 0 and sends an Access Token Request to the NRF for a Service Request toward the NF_B including notification URI and CCA of the NF_C.

Step 2.	The NRF verifies whether the NF_A is authorized and whether the NF_C identified by the appended CCA is a valid Network Function and authorized to receive the requested NF Service from the NF_B.

Editor’s Note : It is ffs how the NRF determines whether the notification URI of the NF_C is authorized to receive the requeted service from NF_B, since not all consumers register a profile at the NRF.

Step 3.	Upon successful verification, the NRF publishes an access token for a Service Request of the NF_A toward the NF_B. The access token includes notification URI of the NF_C in the claims as requested in step 2.

Step 4.	The NF_A sends a Service Request for a notification service toward NF_B including the access token received at step 3 and CCA of the NF_A.

Step 5.	The NF_B verifies the access token and CCA of the NF_A. During the verification, the NF_B can check whether notification URI in the Service Request are included in the access token. If the verification is successful, the NF_B may respond the NF_A and provide the NF_C with the notification service as requested by the NF_A.

### 6.X.3 Evaluation

This solution provides a way to authorize the owner NF of “Notification URI” by NRF and NF Service Producer and avoid accessing resource by unauthorized UE in “Subscribe/Notify” scenario.

This solution updates procedures of access token publishing and verification which includes authorization of NF which owns a URI identified by “notification URI”.