3GPP TSG-SA3 Meeting #107Adhoc-e 	draft_S3-221333-r1

- e-meeting, 27th June – 1st July, 2022

- Source:	Interdigital

- Title:	Key Issue on Direct C2 Security

- Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

This document proposes to add key issue on Direct C2 Security in TR 33.891

# 2	References

# 3	Rationale

This contribution proposes a new key issue for Direct C2 Security.

# 4	Detailed proposal

**** START OF CHANGE ****

## 5.X	Key issue #X: Direct C2 Security

### 5.X.1	Key issue details

TS 23.256 [xx] describes how the UAV pairing with a UAV-C is authorized by a USS over a PDU Session for C2 communications performed over Uu.

TS 33.256 [yy] describes how the 3GPP system shall provide means for the USS/UTM to transport security information to the UE to secure C2 communications.

However, aspects for direct C2 communication over PC5 are currently not covered in the above.

In TR 23.700-58 [zz], key issue #1 focuses on the transport of C2 communications over PC5 in the 3GPP system, while considering the following aspects:

"- how is the C2 communication over PC5 between a UAV and UAV controller established;

-	how is the UAV authorized for setting up direct C2 communication over PC5 with a UAV controller, both for in-coverage and out of coverage scenarios, and how is the authorization revoked;

-	whether the UAV needs to discover the UAV controller, or vice versa and if so, how?"

### 5.X.2	Security threats

The lack of C2 communication security over PC5 between UAV and UAV-C may let the attackers to eavesdrop and control the UAV operations thereby leading to UAV hijack and misoperations.

### 5.X.3	Potential security requirement

The 3GPP system shall provide means for UAV and UAV-C to establish secure PC5 link used for C2 communication .