- 3GPP TSG-SA3 Meeting #107e AdHoc	S3-221438-r1

- e-meeting,  27 June - 1 July 2022

- Source:	ZTE

- Title:	ECS/EES authentication method information provisioning solution on Key issue #2.2

- Document for:	Approval

Agenda Item:	5.9

# 1	Decision/action requested

This document proposes to add a new solution on key issue #2.2: Authentication mechanism selection between EEC and ECS/EES. SA3 is kindly requested to approve this doc.

# 2	References

[1]	3GPP TR 33.739 v0.1.0

# 3	Rationale

It is concluded for multiple authentication methods (TLS with AKMA, TLS with GBA and other TLS authentication methods) between EEC and ECS, EEC and EES. However, with these multiple authentication methods, how to select which authentication mechanism to use between the EEC and EES, EEC and ECS is not addressed. This new solution supports ECS/EES authentication method information provisioning to help authentication mechanism selection. During the PDU session establishment procedure, UE that hosts EEC(s) receives ECS/EES authentication method information (e.g. via PCO) and determines which to use.

# 4	Detailed proposal

**** START OF 1st CHANGE ****

## 6.X	Solution #X: ECS/EES authentication method information provisioning

### 6.X.1	Solution overview

This solution addresses KI#2.2: Authentication mechanism selection between EEC and ECS/EES.

During the PDU session establishment procedure, UE that hosts EEC(s) receives ECS/EES authentication method information (e.g. via PCO) and determines which to use.

### 6.X.2	Solution details

If the UE hosts an EEC and supports transferring the ECS/EES authentication method information from the 5GC to the EEC, the UE indicates in the PCO at PDU Session establishment that it supports the ability to receive ECS address(es) via NAS and to transfer the ECS Address(es) to the EEC(s) (see TS 23.502 [xx]). As described in TS 23.502 [xx], if the UE supports the ability to receive ECS/EES authentication method information via NAS and to transfer the ECS/EES authentication method information to the EEC(s), the UE may receive ECS/EES authentication method information from the SMF via PCO during PDU Session Establishment and/or during PDU Session Modification procedures.

The SMF may receive ECS/EES authentication method information from the UDM together with SM subscription information. The UDM in the HPLMN may provide the SMF with ECS/EES authentication method information. Or the ECS/EES authentication method information is preconfigured in the SMF, for example, in H-SMF for HR case or V-SMF for LBO cases.

The SMF determines the ECS/EES authentication method information to be sent to the UE based on UE subscription information received from UDM(as described in clause 4.15.6.3d-2 of TS 23.502 [xx]).

- According to the ECS/EES authentication method information received from SMF (via PCO), UE selects TLS authentication methods both supported by EEC and ECS/EES. If there is no authentication methods supported by both sides, it returns failure.

### 6.X.3	Solution evaluation

Editor’s Note: Evaluation is FFS.

**** START OF 2nd CHANGE ****

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-48: "5G System Enhancements for Edge Computing; Phase 2".

[3]	3GPP TR 23.700-98: "Study on Enhanced architecture for enabling Edge Applications ".

[4]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications".

[5]	3GPP TS 33.839: "Study on security aspects of enhancement of support for edge computing in the 5G Core (5GC)".

[xx]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

**** END OF CHANGE ****