3GPP TSG-SA3 Meeting #108e 	S3-221897-r2

- e-meeting, 22 - 26 August 2022

- Source:	Samsung, Nokia, Nokia Shanghai Bell

- Title:	Resolving EN for solution#11

- Document for:	Approval

Agenda Item:	5.24

# 1	Decision/action requested

It is requested to approve this proposal.

# 2	References

NA

# 3	Rationale

This contribution proposes to resolve the following EN under solution#11.

Editor’s Note: Definition of Slice Resource is FFS.

- This solution does not define any “slice resources”. The solution talks about only the resources provided by the NF producer for NF-Consumers which are allowed to access it and perform read or write operation. Hence, it is proposed to convert the EN into Note.

Editor’s Note: Further evaluation is FFS.

Additional evaluation to justify the solution is added as follows:

In this solution, the NF Service Producer is still dependent on NRF to include the validated S-NSSAIs, therefore how NRF validates the NF Service Consumers is not covered in this solution.

# 4	Detailed proposal

******Start of Change******

## 6.11	Solution #11: Registered NF Profile changes for Inter-Slice Access

### 6.11.1	Introduction

This solution proposes to address following security requirement of KI #9:

- 	The 5GS should provide a mechanism that allows the NF Service Producer not to provide service to NF consumers, which are not authorized to access a slice.

This solution gives the option for NF Service Producers to indicate which consumer’s S-NSSAIs are allowed to access which producer’s S-NSSAIs, and by which operations in its profile.

### 6.11.1	Solution Details

Using individual or combination of IEs like allowedNfTypes and allowedNssais, NRF can already filter the requests from NF Service Consumers not allowed to access any services of an NF Service Producer.

To address the issue of static configurations at NRF for restricted Inter-slice access, it is proposed that:

-	When an NF Service Producer registers its profile in NRF, it includes a new information element, “allowedSnssaisPerSnssai”, which specifies, for each NF-Service Consumer’s S-NSSAI, the list of NF-Service Producer’s S-NSSAIs it is allowed to access.

-	When an NF Service Producer registers its profile in NRF, it includes a new information element, “allowedOperationsPerSnssai”, which specifies allowed operations on its resources, for NF Consumers belonging to the given S-NSSAI.

These attributes can then be used by NRF to determine the “producerSnssaiList” to be included in the AccessTokenClaims. Additionally, NRF can determine whether a given resource/operation-level scope can be granted to an NF Service Consumer that requested an Oauth2 access token with a specific scope, or limit the scope or authorization irrespective of the request.

Additionally, to allow NF Service Producers (themselves) validate the “Inter-Slice” access requests from the NF Service consumers, it is proposed to include “Requester-NSSAI” in the access-grant, indicating the NRF validated NF-Consumer’s S-NSSAIs.

The NF Service Producer can thus check the decision of the NRF to assign an access token, but is still dependent on the NRF validating the consumer’s S-NSSAI and including it in the access token.

NOTE: Slice resource is not defined in this solution.

6.11.3	Evaluation

This solution proposes to address the following security requirement of KI #9:

- 	The 5GS should provide a mechanism that allows the NF Service Producer not to provide service to NF consumers, which are not authorized to access a slice.

1. NFp validating NFc:

The solution gives the option for NF Service Producers to indicate which consumer’s S-NSSAIs are allowed to access which producer’s S-NSSAIs, and by which operations, in its profile.

2. NRF validating NFc:

In this solution, the NF Service Producer is still dependent on NRF to include the validated S-NSSAIs, therefore how NRF validates the NF Service Ponsumers is not covered in this solution.

******End of Change******