3GPP TSG-SA3 Meeting #108e 	S3-222223-r1

- e-meeting, 22 - 26 August 2022										            		    Revision of S3-22xxxx

- Source:	Xiaomi

- Title:	New solution on User Consent for UE Data Exposure to HPLMN in the Roaming case

- Document for:	Approval

Agenda Item:	5.22

# 1	Decision/action requested

This contribution proposes to add a new solution in the FS_UC3S_Ph2 study TR 33.896

# 2	References

[1]	3GPP TR 33.896 v0.1.0: "Study of Security Aspects on User Consent for 3GPP Services Phase 2"

[2]	3GPP TR 23.700-81 v0.3.0: "Study of Enablers for Network Automation for 5G System (5GS); Phase 3"

[3]	3GPP TS 33.501 v17.4.2: "Security architecture and procedures for 5G system"

# 3	Rationale

Key issue #1 in TR 33.896 [1] addresses the checking of user consent for roaming case in eNA. As per TR 23.700-81 [2], both PLMNs (VPLMN, HPLMN) need the ability to control the amount of UE data exposed based on user consent, operator policy, regulatory policy and/or roaming agreements. If the consumer NF in the HPLMN requests the data collection or data analytics from the VPLMN, the H-NWDAF will perform the role of enforcement point and determine whether the request can be sent to the V-NWDAF. As the enforcement point, the H-NWDAF interacts with the UDM to retrieve the user consent parameters, which is consistent with the current user consent framework in Annex V in 3GPP TS 33.501 [3].

Therefore, this pCR proposes a solution to address Key issue #1 User consent for roaming case in eNA in TR 33.896 [1] for FS_UC3S_Ph2.

# 4	Detailed proposal

## 6.Y	Solution #Y: User Consent for UE Data Exposure to HPLMN in the Roaming case

## 6.Y.1	Introduction

The solution addresses Key Issue #1: User consent for roaming case in eNA. It aims to meet one of the potential requirements in Key issue #1 on check of user consent for the roaming scenario in eNA.

As per TR 23.700-81 [2], both PLMNs (VPLMN, HPLMN) need the ability to control the UE data exposed based on user consent, operator policy, regulatory policy and/or roaming agreements. If the consumer NF in the HPLMN requests the data collection or data analytics from the VPLMN, the H-NWDAF will take the role of enforcement point and determine whether the request can be sent to the V-NWDAF. As the enforcement point, the H-NWDAF interacts with the UDM to retrieve the user consent parameters, which is consistent with the current user consent framework in Annex V in 3GPP TS 33.501 [3].

### 6.Y.2	Solution details

0.	The UDM stores the user consent information, operator policy or regulatory policy. The user consent information is associated with SUPI, and stored in UDM as subscription data. The regulatory policy is specific to the particular country or region, can be used in the roaming scenario.

1.	The Consumer NF in HPLMN requests/subscribes analytics to the H-NWDAF. The message includes Target of Analytics Reporting that is set to a group of UE(s) or any UE. In the roaming case, the message may also include an indication for roaming UE and the PLMN ID of VPLMN.

Editor's note: Usage of roaming indication is FFS.

2.	If there is no user consent info in the H-NWDAF's UE context, the H-NWDAF sends the Nudm_SDM_GetRequest message to the UDM to check the user consent information.

3.	The UDM retrieves user consent info and sends Nudm_SDM_Get Response message to the H-NWDAF.

4.	Once receiving the response message, the H-NWDAF stores the user consent info in the H-NWDAF’s UE context. Based on the operator policy, regulatory policy, roaming agreements of VPLMN and user consent, the H-NWDAF determines whether it is allowed to request the collected data or analytics from the VPLMN to be provided to the consumer NF in the HPLMN. If the user consent info indicates that Consumer NF in the HPLMN is allowed to request the collected data or analytics from the VPLMN, the H-NWDAF discovers the V-NWDAF and sends analytics request to the V-NWDAF.

Note X: The regulatory policy in the H-NWDAF is specific to the country or region where UE is registered.

5.	If the regulation policy configured in/provisioned to the V-NWDAF indicates that Consumer NF in the HPLMN is allowed to request the collected data or analytics, the V-NWDAF provides the collected data or analytics to the H-NWDAF.

Note Y: The regulation policy in the V-NWDAF is specific to the country or region where UE is currently visiting.

6.	The H-NWDAF obtains the collected data or analytics and sends the Nnwdaf_AnalyticsSubscription_Notify message to Consumer NF in HPLMN.

### 6.Y.3	Evaluation

TBD

*************** End of the Change ****************