3GPP TSG-SA3 Meeting #108e 	draft_S3-222232-r4

- e-meeting, 22 - 26 August 2022

- Source:	Xiaomi

- Title: 	KI1, New Sol Authentication mechanism for untrusted non-3GPP Access in NPN scenarios

- Document for:	Approval

Agenda Item:	5.16

# 1	Decision/action requested

It is requested to approve the new solution for KI #1 of 3GPP TR 33.858 v0.1.0.

# 2	References

[1]	3GPP TR 33.858: "Study on security aspects of enhanced support of Non-Public Networks (NPN) phase 2"

# 3	Rationale

This pCR proposes a solution to address secure non-3GPP access issue of key issue #1 in TR 33.858 [1].

# 4	Detailed proposal

*************** Start of 1st Change ****************

## 6.A	Solution #A: < Authentication mechanism for untrusted non-3GPP Access in SNPN scenarios >

### 6.A.1	Introduction

This solution addresses key issue #1.

TR 23.700-08 [2] studies "Key Issue #2: Support of Non-3GPP access for SNPN". Clause 5.2.1 of TR 23.700-08 [2] states: "Currently the 3GPP specifications do not support direct connection to SNPN via non-3GPP access networks" and "One objective of this key issue is to enable the 5GS to support direct connection of non-3GPP access networks to the SNPN's 5GC."

To access to SNPN via no-3GPP access, on the one hand, the UE may need to handle the new identity (e.g., onboarding SUCI/onboarding SUPI) and the new Registration Type (i.e.  SNPN Onboarding) to N3IWF. On the other hand, the UE may use anonymous value SUCI during the registration procedure, which will result in N3IWF failing to locate KN3IWF to authenticate the identity of UE.

This solution is proposed to address the aforementioned problem and enables the UE to access SNPN via an untrusted non-3GPP access network.

6.A.2	Solution details

Figure 6.A.2-x: Authentication for untrusted non-3GPP access

1-4. Step 1 to step 4 are identical to step 1 to step 4 in clause 7.2.1 of TS 33.501 [X]

5.	The UE shall send an IKE_AUTH request which includes an EAP-Response/5G-NAS packet that contains a Registration Request message containing UE security capabilities and the SUCI/onboarding SUCI/ anonymous value SUCI. Moreover, in SNPN scenarios, if construction of SUCI as described in clause 6.12 of TS 33.501 cannot be used and if the EAP method supports SUPI privacy, then the UE may send an anonymous value SUCI based on configuration. And SNPN identifier, which includes PLMN ID and NID, should be included in AN parameter.

For SNPN Onboarding registration, the UE shall use Onboarding SUCI as specified in clause I.2 of TS 33.501[X].The AN parameters that is sent from UE to N3IWF shall include Onboarding indication if the UE is accessing 5GS for Onboarding. The Registration Type should be set as "SNPN Onboarding", which indicates that the UE wants to perform SNPN Onboarding Registration (i.e. allows the UE to access an ON-SNPN for the purpose of provisioning the UE with SO-SNPN credentials).

The remaining part of step 5 is identical to step 5 in clause 7.2.1 of TS 33.501 [X].

6.

When the AMF receives a NAS Registration Request with the 5GS Registration Type set to "SNPN Onboarding", the AMF applies locally configured AMF Configuration Data for Onboarding in order to restrict UE network usage to only onboarding and stores in the UE Context in AMF an indication that the UE is registered for onboarding.

Based on ON-SNPN policies, the AMF may start an implementation specific deregistration timer configured for UE Onboarding as described in TS 23.501 [Y].

7.	 According to clause I.2.2 of TS 33.501 [X], AUSF can obtain SUPI/onboarding SUPI of UE after the authentication procedure. And AUSF can generate SUCI/onboarding SUCI based on SUPI/onboarding SUPI.

If UE utilizes anonymous value SUCI in step 5, to help N3IWF to locate KN3IWF, AUSF should send SUCI/onboarding SUCI to AMF/SEAF.

In SNPN scenarios, AMF can choose 5G AKA, EAP-AKA', or any other key-generating EAP authentication method to authenticate UE as described in clause 6.1.3 or clause I.2.2 of TS 33.501 [X]. In this case, the AMF shall send a key request to the AUSF. The AUSF may initiate an authentication procedure as specified in clause 6.1.3 or clause I.2.2 of TS 33.501 [X].

If EAP-AKA' or key-generating EAP authentication method is used for authentication as described in clause 6.1.3.1 and clause I.2.2 of TS 33.501 [X], then the AUSF shall include the EAP-Success.

The remaining part of step 7 is identical to step 7 in clause 7.2.1 of TS 33.501 [X].

Editor’s Note: in step 7: “Whether SUCI/Onboarding SUCI is needed by the AMF is FFS”

8-11. Step 8 to step 11 are identical to step 8 to step 11 in clause 7.2.1 of TS 33.501 [X].

12. If UE utilizes anonymous value SUCI in step 5, to help N3IWF to locate KN3IWF, AMF should send SUCI/onboarding SUCI to N3IWF along with KN3IWF.

The remaining part of step 12 is identical to step 12 in clause 7.2.1 of TS 33.501 [X].

Editor’s Note: in step 12: “Whether SUCI/Onboarding SUCI is needed by the N3IWF is FFS”

13. Step 13 is identical to step 13 in clause 7.2.1 of TS 33.501 [X].

14.	To help the N3IWF locate KN3IWF, the UE should send SUCI/onboarding SUCI to the N3IWF when the UE utilizes anonymous value SUCI in step 5.

The remaining part of step 14 is identical to step 14 in clause 7.2.1 of TS 33.501 [X].

Editor’s Note: in step 14: “Whether SUCI/Onboarding SUCI is needed by the N3IWF is FFS”

NOTE: in step 14: “UE can only provide SUCI/Onboarding SUCI in case it is configured with SUPI privacy parameters”

15-17. Step 15 to step 17 are identical to step 15 to step 17 in clause 7.2.1 of TS 33.501 [X].

Editor’s Note: Onboarding over untrusted access is pending SA2 decision.

### 6.A.3		System impact

TBD

### 6.A.4	Evaluation

TBD

*************** End of 1st Change ****************

*************** Start of 2nd Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-08: "Study on enhanced support of Non-Public Networks; Phase 2".

[3]	3GPP TS 22.261: "Service requirements for the 5G system".

…

[X]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[Y]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

*************** End of 2nd Change ****************