3GPP TSG-SA3 Meeting #109AdHoc-e 	S3-230055-r1

- Electronic meeting, 16 - 20 January 2023

- Source:	Nokia, Nokia Shanghai Bell, Lenovo

- Title:	pCR on solution 6 update

- Document for:	Approval

Agenda Item:	5.13

# 1	Decision/action requested

Approve the changes in clause 4 for inclusion in 33.887.

# 2	References

[1] 3GPP TR 33.887 Study on Security aspects for 5WWC Phase 2

# 3	Rationale

Solution 6 is updated.

Evaluation is added

# 4	Detailed proposal

**** START OF CHANGE ****

## 6.6	Solution #6: TNAP mobility solution with count

### 6.6.1	Introduction

This solution addresses the security issue due to  TNAP mobility defined in KI#4.

### 6.6.2	Solution details

- 6.6.2.1	Procedure

Figure 6.6.2-1 TNAP mobility procedure

- UE is connected to TNAP#1 via the procedure defined in TS 33.501 figure 7A.2.1-1. Once authenticated, TNGF sends the reauth Id to UE over the protected interface. Reauth Id can be a generated as <PLMNID><TNGF_ID> <Temp Id>.

- Note: TNGF Id could be TNGF address (like fqdn)already defined by SA2.

2, 3.	UE decides to move from TNAP#1 to TNAP#2 and creates an L2 connection with TNAP#2.

4, 5, 6.		TNAP#2 sends the L2 EAP-Request for Identity towards the UE and the UE responds back with an L2 EAP-Response with Identity and a TNAP_Mobility_Indication flag. The TNAP2 forwards the EAP response with reauth Id and the TNAP_Mobility_Indication flag towards TNGF..

7,8. 	 Based on the reauth Id, TNFG identifies the UE and retrieves the context and  checks if the stored context in step 1 is valid and then derives the TNAP’ keys as described in section 6.6.2.2 of this document. The TNGF responds back to TNAP#2 with the generated key. In TNAP#2, the newly received TNAP key is considered as Pairwise Master Key (PMK).

9, 10,11.	The TNAP#2 sends an EAP-notification back to the UE with the update flag. Based on the flag, UE updates the counter and derives the keys. A 4-way handshake is executed (see IEEE 802.11) which establishes a security context between the WLAN AP and the UE that is used to protect unicast and multicast traffic over the air.

Once the procedure is complete, the TNGF sends the new reauth Id to UE over the secure interface that UE can use for the next interaction.

NOTE: If the UE gets the new IP configurations from TNAP2, then the UE updates the SA address using IKE informational request "UPDATE_SA_ADDRESS" to TNGF for further communications.

- 6.6.2.2	Key derivation

Derivation of KTNAP' from KTNGF during mobility use the following input parameters.

-	FC = 0xWX

-	P1 = COUNT,

-	L1 = length of COUNT (i.e. 0x00 0x04)

The input key KEY shall be KTNGF.

In this case, the COUNT shall be the downlink NAS COUNT of the Non-3GPP access.

### 6.6.3	Evaluation

TNGF and UE: Generate new keys based on the count value and handle the reauth Id.

TNAP: use updated keys for the mobility scenario

Editor's Notes: The count desynchronization issue and further evaluation are FFS.

**** END OF CHANGE ****