3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230064-r2

- Electronic meeting, 16 - 20 January 2023

- Source:	Lenovo

- Title:	Update to Solution #9 in eNA

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This pCR propose to update Solution #9 in TR 33.738

# 2	References

1]	3GPP TR 33.738, ‘Study on security aspects of enablers for Network Automation for 5G - phase 3’, (Release 18).

[2]	3GPP TS 23.288, ‘Architecture enhancements for 5G System (5GS) to support network data analytics services’, (Release 17).

# 3	Rationale

This pCR updates Solution #9 to clarify the following aspects.

According to TS 23.288, Clause 6.2.2 Data Collection from NFs, Clause 6.2.2.1 General, it states the following:

‘The Data Collection from NFs is used by NWDAF to subscribe/unsubscribe at any 5GC NF to be notified for data on a set of events.

The Data Collection from NFs is based on the services of AMF, SMF, UDM, PCF, NRF, NSACF and AF (possibly via NEF):

-	Event Exposure Service offered by each NF as defined in TS 23.502 [3] clause 4.15 and clause 5.2.’.

Therefore, this pCR clarifies that data collection based on specific events as in the existing system can be reused to the extent possible. Unless there is a threat identified over the architecture defined in 23.288 for data collection, no new approach is required for data collection.

 According to the key issue details in TR 33.738, Clause 5.4 Anomalous NF behaviour detection by NWDAF, it states the following, which clarifies that the NFs may behave as errorneously or anomalous, but NWDAF behaviour is not challenged and NWDAF is assumed to be reliable as it is expected to support the monitoring in this Key Issue description.

‘it is imperative that an analytics function such as NWDAF supports the monitoring of the behaviour of all NFs and ensures that the NFs behave as defined/specified. If the NFs behave erroneously, it should be possible to detect the anomaly, so that appropriate steps can be taken, e.g., by an operator to control the potentially damaging behaviour.’.

Based on the above clarifications, Solution #9 has been updated and additional clarifications on the relationship between Inputs and outputs as well as the evaluation was provided.

# 4	Detailed proposal

SA3 is kindly requested to agree on the pCR below to TR 33.738.

*****Start of Change 1*****

## 6.9	Solution #9: Anomalous NF behaviour event related data collection and anomalous NF detection

### 6.9.1	Introduction

The solution addresses KI#4 on Anomalous NF behaviour detection by NWDAF.

The solution describes various anomalous NF behaviour event related data collection and the related anomalous NF behaviour analytics exposure procedure.

### 6.9.2	Solution details

A service consumer (i.e., OAM/AF/NF) can subscribe to the NWDAF and get notified about the anomalous NF behaviour related analytics using the procedure shown in Figure 6.9.2-1.

Figure 6.9.2-1: Anomalous NF behaviour event related network data analytics exposure procedure

The steps shown in figure 6.9.2-1 is described as follows:

1.	The service consumer subscribes to the anomalous NF behaviour related analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation message, with the analytics ID (set to the anomalous NF behaviour), list of event ID(s) (related to received messages (e.g., malformed messages) violating predefined service operation input or output formats, message requests exceeding configured limits, unintended or unrecognized configuration change/operational change, any errors notification), target of analytics (indicates NF ID(s)/AF ID(s), NF type) and may include other information for the analytics according to TS 23.288 Clause 6.1.3.

If the service consumer is already subscribed to the anomalous NF behaviour related analytics information, the service consumer requests analytics information by invoking Nnwdaf_AnalyticsInfo_Request service operation with the respective analytics ID and the target of analytics.

2.	When a request for analytics information is received, the NWDAF determines whether triggering new data collection is needed.

3. If the NWDAF determine to perform data collection, it need to subscribe to 5GC NFs or AFs related event exposure services (i.e., via NEF if located externally) to be notified for data on a set of events as described in TS 23.288 Clause 6.2.2.1.

The NWDAF subscribes to set of event IDs (related to the service consumer’s anomalous NF behaviour subscription which includes event IDs indicating messages (e.g., malformed messages) violating predefined service operation input or output formats, message requests exceeding configured limits, unintended or unrecognized configuration change/operational change, any error notification) by invoking the Nnf_EventExposure_Subscribe service operation.

4. If the NWDAF subscribes to the set of event IDs (as in step 3), the NF manages the inference data which includes all event related information.

5. The NF notifies the NWDAF (e.g., event specific inference data) by invoking the Nnf_EventExposure_Notify service operation.

6. The NWDAF peforms the requested analytics using the data collected from the NF.

NOTE 1: The Anomalous NF behaviour analytics related implementation logic can be upto Operator’s implementation and it is out of 3GPP scope.

NOTE 2: If the data need to be collected from an AF, then in steps 3-6, the target AF will be involved instead of NF.

7. If the service consumer is subscribed to analytics information, the NWDAF notifies the service consumer (i.e., based on the request from the service consumer using Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response service operation), the Analytics Reporting Parameters includes the event specific anomalous NF behaviour analytics information. as shown in the table 6.Y.2-1 and 6.Y.2-2 respectively.

NOTE 3: If the NF specific resource usage and load information are required additionally to process anomalous NF behaviour related analytics, then steps 2, 3a and 3b (as described in Solution 6) can be additionally perfomed.

Table 6.9.2-1: Inputs collected by NWDAF from target of analytics (i.e., NFs) for other anomalous NF behaviour detection

| Information | Description |
| --- | --- |
| Target of analytics NF Identifier | The NF identifier indicates the NF from which the data is collected to identify if any anomalous NF behaviour is experienced by the target NF. |
| Predefined service operation violation | Information on messages violating predefined service operation input/output format specified including malformed messages (if any). |
| Message load Information | Information on service requests exceeding configured limit at a time instance or in a period or specific to a slice. |
| Configuration Information | Information related to alerts/triggers associated with unintended configuration or operational change based on operator configuration. |
| NF service consumer identifier(s) | Information on the list of NF service consumers of the target NF. |
| NF service producer identifier(s) | Information on the list of NF service producers from which the target NF consumes services. |

Table 6.9.2-2: Anamolous NF behaviour Statistics

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of observed exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Scalar value indicating the severity of the abnormal behaviour |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > Cause | Indicates the cause for the exception and alerts such as configuration issues, type of attack (e.g., cyber attack or anyother)/threat, malfunction, overload, or software issues accordingly. |
| > List of anamolous NFs | NF Identification information (i.e., NF IDs) related to NFs that are identified as behaving anomolously |
| > List of impacted NFs | NF Identification information (i.e., NF IDs) for all the impacted NFs associated due to the anamolous NF behaviour. |
| > Amount | Estimated number of NFs affected by the Exception |

Table 6.9.2-3: Anamolous NF behaviour Predictions

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of predicted exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Scalar value indicating the severity of the abnormal behaviour |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > Cause | Indicates the cause for the exception and alerts such as configuration issues, type of attack (e.g., cyber attack or anyother)/threat, malfunction, overload, or software issues accordingly. |
| > List of anamolous NFs | NF Identification information (i.e., NF IDs) related to NFs that are identified as behaving anomolously |
| > List of impacted NFs | NF Identification information (i.e., NF IDs) for all the impacted NFs associated due to the anamolous NF behaviour. |
| > Amount | Estimated number of NFs affected by the Exception |
| > Confidence | Confidence of this prediction |

NOTE 4: The solution uses the existing method specified in TS 23.288 Clause 6.2.2.1 for data collection from NFs which involves notification of data on a set of events. The events do not indicate threats instead it is related to collection of information related to violations of expected/specified behaviours.

NOTE 5a: This solution proposes to extend event exposure to NF related information, like malformed messages.

Editor’s Note: It is ffs whether standardizing events is a sufficiently dynamic approach to react to new evolving

NOTE 5b: Key Issue #4, covers the scenario where the NFs can behave erroneously or anomalously, but the NWDAF is expected to support monitoring of NFs behaviour to ensure that, NFs behave as defined/specified. In the scope of the key issue #4, the solution assumes that the NWDAF can be trusted (e.g., aspects based on what a NWDAF can be trusted is upto operator implementation).

Editor’s Note: Whether the inputs are sufficient and feasible is FFS.

NOTE 6: The list of input data and the output analytics (i.e., related to anamolous NF behaviour Statistics and predictions) are shown in Table 6.9.2-1, 6.9.2-2, and 6.9.2-3. The input-based analytics output generation will depend on the algorithms or intelligence logic used and it is outside scope of 3GPP.

Editor’s Note: What’s the relationship between inputs and output is FFS.

### 6.9.3	Evaluation

The solution addresses Key Issue #4 and has the following impacts:

This Solution assumes that NWDAF can be trusted. This solution extends event exposure to NF related information, like malformed messages.

The NWDAF need to support anomalous NF behaviour analytics subscription process, where the NWDAF need to collect data related to new events such as violation of predefined service operation message, message request load information, and security alerts in NFs. Further the NWDAF need to provide output related to anamolous NF behaviour statistics and predictions to detect any anamolous NF(s) and associated risk (if exists) to the subscribed anomalous NF behaviour analytics consumer(s).

To minimize the impact with usage of NF Event Exposure APIs, it is workable to consider Solution 6 as baseline for input data collection as defined in Clause 6.6.2.3. Furthermore, the inputs and output parameters defined in the solution 9 can be merged with the ones proposed in solution 6 to have a comprehensive detection mechanism.

The solution does not provide evidence for abnormal NF detection precision, false positive rate, false negative rate. Without that, the administrator may take wrong action for the wrong detection so that the network may face unsecure configuration risk.

Editor’s Note: Further evaluation is FFS.

*****End of Change 1*****