3GPP TSG-SA3 Meeting #109AdHoc-e 	draft_S3-230065-r2

- Electronic meeting, 16 - 20 January 2023

- Source:	Lenovo

- Title:	Cyber attack detection

- Document for:	Approval

Agenda Item:	5.8

# 1	Decision/action requested

This pCR proposes a solution to KI#6 in TR 33.738.

# 2	References

[1]	3GPP TR 33.738, ‘Study on security aspects of enablers for Network Automation for 5G - phase 3’.

[2]	3GPP TS 23.288, ‘Architecture enhancements for 5G System (5GS) to support network data analytics services’.

# 3	Rationale

This pCR provides a solution to KI#6 on Cyber-attack detection.

# 4	Detailed proposal

SA3 is kindly requested to agree on the pCR below to TR 33.738

*****Start of Change 1*****

## 6.Y	Solution #Y: Cyber attack detection using an analytics function

### 6.Y.1	Introduction

The solution addresses KI#6 on Cyber-attack detection.

KI#6 describes that, to achieve cyber-attacks detection, the NWDAF can collaborate with UE and any other NFs to collect related data as inputs and provide alerts of anomaly events as outputs to OAM and other NFs which have subscribed to them. Further KI#6 also considered two example cases such as (i) fraudulent relay nodes which may modify or change messages between the UE and the RAN and (ii) DoS attacks related to network entities (e.g., RAN and Core network).

6.Y.2	Solution details

The cyber attack detection solution is described below by using NWDAF as the analytics function, but the solution does not prevent to use any equivalent analytics function if required to be considered.

Cyber attack detection over the network:

A service consumer (i.e., OAM/AF/NF) can subscribe to the NWDAF and get notified about the malicious behaviour related analytics associated to one or more UEs using the procedure shown in Figure 6.Y.2-2.

Figure 6.Y.2-1 Network Data Collection and analytics procedure for cyber-attack detection

The steps shown in figure 6.Y.2-1 is described as follows:

1.	The analytics service consumer subscribes to the network attack detection related analytics information by invoking the Nnwdaf_AnalyticsSubscription_Subscribe service operation message, with the analytics ID (set to the malicious behaviour), list of event ID(s) (related to UE malicious behaviour, attacks/threats (e.g., cyber-attack, DDoS, etc.,), received messages (e.g., malformed messages, malicious data) violating predefined service operation input or output formats, message requests exceeding configured limits, unintended or unrecognized operations, repeated authentication failure), target of analytics (indicates UE ID(s) i.e., such as SUPI or a group of UEs (an Internal Group ID), analytics filter can be optionally included (e.g., as area of interest, S-NSSAIs, DNN, and Application ID) and may include other information for the analytics according to TS 23.288 Clause 6.1.3.

NOTE:	In the case of untrusted AF the Target of Analytics Reporting can be a GPSI or an External Group Identifier that is mapped in the 5GC to a SUPI or an Internal Group Identifier as described in TS 23.288.

If the service consumer is already subscribed to the network attack detection related analytics information, the service consumer requests analytics information by invoking Nnwdaf_AnalyticsInfo_Request service operation with the respective analytics ID and the target of analytics.

2.	When a request for analytics information is received, the NWDAF determines whether triggering new data collection is needed.

3. If the NWDAF determine to perform data collection, it need to subscribe to 5GC NFs (i.e., serving AMF, serving SMF, AFs (i.e., via NEF if located externally)) and OAM according to the target of analytics (i.e., UE IDs) for related event exposure services to be notified for data on a set of events (as described in TS 23.288 Clause 6.2.2.1) as requested by the service consumer in step 1.

The NWDAF subscribes to set of event IDs (related to UE malicious behaviour, attacks/threats (e.g., cyber-attack, DDoS, etc.,), received messages (e.g., malformed messages, malicious data) violating predefined service operation input or output formats, message requests exceeding configured limits, unintended or unrecognized operations, repeated authentication failure) by invoking the Nnf_EventExposure_Subscribe service operation (in case of AF being the data producer, the NWDAF can use Naf_EventExposure_Subscribe service operation) along with the UE IDs.

4. If the NWDAF subscribes to the set of event IDs (as in step 3), the NF manages the inference data which includes all event related information.

5. The NF notifies the NWDAF (e.g., event specific inference data) by invoking the Nnf_EventExposure_Notify service operation.

NOTE: If the data need to be collected from an AF, then in steps 3-5, the target AF will be involved instead of NF and if the AF is not trusted, then the event exposure is invoked via the NEF.

6. Alternatively, the NWDAF can also collect data from OAM (e.g., for the target of analytics (UE IDs) related to the NFs/AFs to collect event specific information mentionted in step 3 in the form of inference data/logs) using the procedure specified in TS 23.288 Clause 6.2.3.

7. The NWDAF peforms the requested analytics using the data collected from the NFs, AFs, and/or OAM.

NOTE: The UE malicious behaviour analytics related implementation logic can be upto Operator’s implementation and it is out of 3GPP scope.

8. If the service consumer is subscribed to analytics information, the NWDAF notifies the service consumer (i.e., based on the request from the service consumer using Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response service operation), the Analytics Reporting Parameters includes the event specific network attack detection analytics information with the corresponding attack detection and threat alerts respectively as shown in the table 6.Y.2-1 and 6.Y.2-2 respectively.

Table 6.Y.2-1: Network attack detection Statistics

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of observed exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Scalar value indicating the severity of the abnormal behaviour |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > Cause | Indicates the cause for the exception and alerts such as related to UE malicious behaviour, configuration issues, type of attack (e.g., cyber attack/DoS/DDoS or anyother threat), overload/flooding, software issues accordingly. |
| > List of malicious UEs | UE Identification information (i.e., SUPIs/ Internal Group IDs) related to UEs that are identified as behaving maliciously. |
| > List of impacted network functions | Identification information of all the impacted NFs, and AFs due to the UE malicious behaviour. |
| > Amount | Estimated number of NFs, and AFs affected by the Exception |
| > Exception category | Indication if the UE behaviour is an attack or genuine error |

Table 6.Y.2-2: Network attack detection Predictions

| Information | Description |
| --- | --- |
| Exceptions (1..max) | List of predicted exceptions |
| > Exception ID | The risk detected by NWDAF |
| > Exception Level | Scalar value indicating the severity of the abnormal behaviour |
| > Exception trend | Measured trend (up/down/unknown/stable) |
| > Cause | Indicates the cause for the exception and alerts such as related to UE malicious behaviour, configuration issues, type of attack (e.g., cyber attack/DoS/DDoS or anyother threat), overload/flooding, software issues accordingly. |
| > List of malicious UEs | UE Identification information (i.e., SUPIs/ Internal Group IDs) related to UEs that are identified as behaving maliciously. |
| > List of impacted network functions | Identification information of all the impacted NFs, and AFs due to the UE malicious behaviour. |
| > Amount | Estimated number of NFs, and AFs affected by the Exception |
| > Exception category | Indication if the UE behaviour is an attack or genuine error |
| > Confidence | Confidence of the prediction |

### 6.Y.3	Evaluation

TBD

*****End of Change 1*****