3GPP TSG-SA3 Meeting #110Ad-Hoc-e 	S3-231763

# Electronic meeting, Online, 17 - 21 April 2023									(revision of S3-yyxxxx)

# Source:	Ericsson

# Title:	Resolve EN for conclusion for key issue #2

# Document for:	Approval

Agenda Item:	5.15

# 1	Decision/action requested

It is proposed that the following pCR is included in the TR [1].

# 2	References

[1]	3GPP TR 33.890: "Study on security support for Next Generation Real Time Communication services"

# 3	Rationale

This contribution proposes to resolve this Editor's Note:

Editor’s Note: Conclusion on further P2P cases are FFS.

# 4	Detailed proposal

It is proposed that the following changes are included in the TR [1].

**** START OF CHANGES ****

## 7.2	Conclusion on Key Issue #2

The following conclusions are agreed for KI#2:

A)	The DCMF/enhanced MRF will terminate DTLS for the following cases:

1)	Bootstrap data channel;

2)	P2A/A2P application data channel if the external Data Server supports only HTTP;

3)	P2P application data channel with the involvement of DCMF/enhanced MRF;

NOTE:	Conclusions on further P2P cases should be considered in normative work.

C)	If the external Data Server supports both HTTP and IETF RFC 8831 WebRTC data channel protocols, the end-to-end (e2e) encryption for the data channel should be supported as specified in IETF RFC 8831 [9] and IETF RFC 8827 [13] for the P2A/A2P Application data channel use cases.

NOTE:	LI considerations are FFS.

**** END OF CHANGES ****