3GPP TSG-SA3 Meeting #115AdHoc-e	S3-241111

Electronic meeting, online, 15 - 19 April 2024

- Source:	Charter, Nokia, CableLabs, Verizon

- Title:	pCR on KI for 5G NR Femto connecting securely with 5G Core

- Document for:	Approval, Information, Discussion

Agenda Item:

# 1	Decision/action requested

It is requested to approve this tdoc to include the key issue in TR 33.745.

# 2	References

[1] 33.745 Study on security aspects of 5G Next Radio (NR) Femto

[2] 3GPP TS 33.320 Security of Home Node B (HNB)/Home evolved Node B

# 3	Rationale

SP-231675 was approved in SA during December 2023. In line with this study, SA3 should also study the security impacts of supporting Femtos in 5G network in 33.745.

The rationale for this study is.

- Security aspects of a 5G NR Femto is currently not explicitly specified in 3GPP standards.

- Femto deployments are likely to be diverse and outside direct operators’ control, i.e. in residential homes, enterprises, and small business. The connection from these diversely deployed femtos to the operator network need to be secure.

3GPP TS 33.320 Security of Home Node B (HNB)/Home evolved Node B defines security requirements for LT HeNBs, these could be leveraged to define security requirements for 5G NR Femtos.

# 4	Detailed proposal

**** START OF NEXT CHANGE ****

## 5.x	Key issue #x: Security aspect of 5G NR Femto connecting to the operator network.

### 5.x.1	Key issue details

When a 5G NR Femto  connects to the operators’ core network, based on a deployment scenario the 5G NR Femto cell may not be in operators’ control. The 5G NR Femto may be using unsecure public and/or 3rd party network to connect with the operator core. Unelss adequate security measures are in place, this may make both, the 5G NR Femto as well as operator’s network vulnerable to security threats and compromise its integrity and functionality.

### 5.x.2	Threats

Possibile loss of confidentiality, integrity and threats on network availability are likely due to lack of security of the services offered by 5G NR Femtos deployed in non-trusted environments.

### 5.x.3	Potential security requirements

The 5GS should shall support a mechanism to establish mutual authentication between 5GS and 5G NR Femto connected over unsecure public/3rd part networks.

The 5GS should support a mechanism to establish secure connection between 5GS and Femto connected over unsecure public/3rd part networks.

Editor’s Note: clauses 5.2, 5.3 and clause 7 from TS 33.320 can be reused as much as possible to meet the above requirements.

**** END OF CHANGE ****