3GPP TSG-SA3 Meeting #115AdHoc-e	draft_S3-241158-r1

Electronic meeting, online, 15 - 19 April 2024

- Source:	ZTE

- Title:	New solution for SUPI privacy issue

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

This contribution proposes to add a new solution for SUPI privacy issue

# 2	References

# 3	Rationale

The contribution proposes to add a new solution for SUPI privacy issue.

# 4	Detailed proposal

It is suggested to approve the following change.

*************** Start of the change ****************

## 7.Y	Solution #Y:Use a new PLMNNPN UE ID to resolve the SUPI privacy issue

### 7.Y.1	Introduction

This solution address the key issue#3 SUPI privacy issue in PLMN hosting NPN scenario.A Security for PLMNNPN Network Function(SPNF) is designed to conceal/de-conceal PLMNNPN UE ID

### 7.Y.2	Solution details

1.The AMF/SEAF may initiate an authentication with the UE during any procedure establishing a signalling connection with the UE, according to the AMF/SEAF's policy. The UE shall use SUCI or 5G-GUTI in the Registration Request. If the 5G-GUTI point to the AMF which is not located in the current premises, such as the AMF is in the customer premises while the 5G-GUTI points to an AMF which located in the operator premises, AMF/SEAF may send a identity request to get the SUCI of UE, according to the AMF/SEAF's policy.

2.The AMF/SEAF shall invoke the Nausf_UEAuthentication service by sending a Nausf_UEAuthentication_Authenticate Request message to the AUSF whenever the AMF/SEAF wishes to initiate an authentication.The Nausf_UEAuthentication_Authenticate Request message shall contain either SUCI or UE ID which is used in a customer premises(PLMNNPN UE ID). Nausf_UEAuthentication_Authenticate Request message may further contain the serving network name. Nausf_UEAuthentication_Authenticate Request message may further contain a PLMNNPN service indication. The PLMNNPN service indication indicate the service request comes from the customer premises.

3.Upon receiving the Nausf_UEAuthentication_Authenticate Request message, the AUSF sends a Nudm_UEAuthentication_Get Request to UDM.The Nudm_UEAuthentication_Get Request sent from AUSF to UDM includes the following information:SUCI or UE ID which is used in a customer premises; the serving network name; if received from SEAF, PLMNNPN service indication;

4.Upon reception of the Nudm_UEAuthentication_Get Request, the UDM shall invoke SIDF if a SUCI is received. SIDF shall de-conceal SUCI to gain SUPI before UDM can process the request.

UDM shall invoke Security for PLMNNPN Network Function(SPNF) if a PLMNNPN UE ID is received. SPNF shall return SUPI. For each Nudm_Authenticate_Get Request, the UDM/ARPF shall create an authentication vector. If a SUCI is received, Nudm_Authenticate_Get Response may further contain a new PLMNNPN UE ID.

5.The AUSF sends a AV in Nausf_UEAuthentication_UEAuthentication Response message.

6.The AMF/SEAF sends the Authentication request message to UE and UE response an authentication response message with RES*.

7.The AMF/SEAF sends a Nausf_UEAuthentication_Authenticate Request message with RES* to AUSF.

8.The AUSF shall indicate to the AMF/SEAF in the Nausf_UEAuthentication_Authenticate Response whether the authentication was successful or not from the home network point of view. If the authentication was successful, the KSEAF shall be sent to the AMF/SEAF in the Nausf_UEAuthentication_Authenticate Response. In case the AUSF received a SUCI from the AMF/SEAF in the authentication request, and if the authentication was successful, then the AUSF shall also include the PLMNNPN UE ID in the Nausf_UEAuthentication_Authenticate Response message.

9.The AMF/SEAF sends UE a registration accept message to UE if the authentication is passed.The AMF/SEAF assign a new 5G-GUTI for the UE

### 7.Y.3	Evaluation

TBD

Editor’s Note:When network slicing is used, the need for scenarios which support an AMF located in the PNI-NPN 	customer domain is FFS.

Editor’s Note:This solution has UE impact, how to avoid UE impact is FFS.

*************** End of the change ****************