3GPP TSG-SA3 Meeting #115AdHoc-e	S3-241218-r1

- Electronic meeting, online, 15 - 19 April 2024					          (revision of S3-yyxxxx)

- Source:	Huawei, HiSilicon, NOKIA, Nokia Shanghai Bell, Ericsson, Samsung?

- Title:	EEC provided information verification

- Document for:	Approval

Agenda Item:	5.14

# 1	Decision/action requested

Approve the changes in clause 4 for inclusion in the document: draftCR to TR 33.749

# 2	References

# 3	Rationale

Since no conclusion of EEC provided information verification was reached in R18, it was agreed in the Edge SID that this topic will be studied again in R19. The contribution proposes to shift the KI of EEC provided information verification in TR 33.739 to TR 33.749.

# 4	Detailed proposal

It is suggested to approve the following changes.

*************** Start of the 1st change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x1]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[x2]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

*************** End of the 1st change ****************

*************** Start of the 2nd change ****************

# 5	Key issues

### 5.Y	Key issue #Y: EEC provided information verification

#### 5.Y.1	Key issue details

Clause 8.6.5 of 3GPP TS 23.558 [x1] defines UE identifier API which is used by an EAS or EEC to obtain the identifier of the UE if the EAS or EEC does not have it (e.g. has not already cached). This identifier, called UE ID (could be the GPSI or the EEL-generated Edge UE ID, defined in clause 7.2.6 of 3GPP TS 23.558 [x1]), is used by the EAS to invoke capability APIs specific to UEs over EDGE-3 and/or EDGE-7 depending on the UE ID type.

The EES uses user information (e.g. IP address) received in the UE Identifier API invocation and obtains the UE identifier by interacting with NEF as specified in clause 4.15.10 of 3GPP TS 23.502 [x2]. The EES may utilize the Nnef_UEId_Get service (clause 4.15.10 of 3GPP TS 23.502 [x2]) providing the user information provided by the EEC, i.e. private UE IP address.

Since the user information (i.e. private UE IP address) may be used to determine the UE ID, it is needed to ensure that the user information is trusted, and the EEC and the application servers (e.g. EAS) is authorized to use this user information as a parameter in the API invocation.

This key issue is to study the security and privacy aspects of usage of user information provided by the EEC in the UE ID API invocation.

#### 5.Y.2	Threats

If user information (i.e. the private UE IP address) provided by the EEC is not verified and the EEC is not authorized to use this information, a malicious or compromised EEC or a malicious API consumer can try to execute IP address spoofing attacks to learn identifiers of other UEs, including those they are not authorized to access. In this case, the EAS obtains an incorrect UE identifier, when the EAS use this incorrect UE identifier to invoke capability APIs specific to UE over EDGE-7, unauthorized information (e.g. UE location) belonging to another UE is exposed to the EAS.

- The usage of UE IP address by the application server without any authorization by the UE my compromise the privacy of the UE. For instance, a compromised or malicious EAS could use the IP address to track the UE, or to aggregate similar IPs, which would correspond to similar position, and make additional analysis over groups of UEs.

#### 5.Y.3	Potential security requirements

The user information (i.e. private UE IP address) provided by the EEC should be verified and the EEC should be authorized to use this information.

The 5G system should support a mechanism to authorize the application servers (e.g. EAS) for the usage of the 5G system UE Id and privacy related information retrieval.

The 5G system should support a mechanism for verification of the IP address provided by the EEC.

The procedure and corresponding services intended to retrieve the 5G system UE Id and privacy related information should be secured to prevent security and privacy breaches caused by compromised or malicious application servers.

The user information (i.e. private UE IP address) provided by the EEC should be verified and the EEC should be authorized to use this information.

*************** End of the 2nd change ****************