3GPP TSG-SA3 Meeting #115AdHoc-e	S3-241220r2

- Electronic meeting, online, 15 - 19 April 2024					                   (revision of S3-yyxxxx)

- Source:	Huawei, HiSilicon, InterDigital, China Telecom

- Title:	New key issue on security of Authentication and Authorization of one or more non-3GPP devices behind one gateway UE or 5G-RG

- Document for:	Approval

Agenda Item:	5.10

# 1	Decision/action requested

It is proposed to approve the new key issue of TR 33.700-32.

# 3	Rationale

It is proposed to include the key issue in the security study of usage of user identifiers in the 5G system.

# 4	Detailed proposal

***  BEGINNING OF THE 1st CHANGE  ***

## X.Y	Key issue #Y: Authentication and Authorization of one or more non-3GPP devices behind one gateway UE or 5G-RG

### X.Y.1	Key issue details

This key issue is going to address Authentication and Authorization of one or more non-3GPP devices behind one gateway UE or 5G-RG. It is to address the security issues related to the key issue #4 in the TR 23.700-32 [2], i.e. Identifying non-3GPP Devices Connecting behind a UE or 5G-RG. The key issue is for the following use case (see TR 23.700-32 [2]):

- Use case #2: one or more non-3GPP devices behind one gateway UE or 5G-RG.

### X.Y.2	Threats

If the non-3GPP devices behind one gateway UE or 5G-RG are not authenticated and authorized by the network, the attacker can access the network as a non-3GPP device via one gateway UE or 5G-RG without any authorization and restriction.

If the number of simultaneously active non-3GPP devices behind the UE or 5G-RG is not restricted, an excess number of non-3GPP devices may abuse the UE or 5G-RG subscription simultaneously.

### X.Y.3	Potential security requirements

The 3GPP system shall support the identification of a non-3GPP device behind UE or 5G-RG.

The non-3GPP devices behind one gateway UE or 5G-RG shall be authenticated and authorized.

The 3GPP system shall provide means to support authentication and authorization of a non-3GPP device behind UE or 5G-RG.

NOTE: The authentication of the non-3GPP device is not done by the 5GC.

The 3GPP system shall support a mechanism to restrict the number of simultaneously active non-3GPP devices behind the UE or 5G-RG.

***  END OF THE CHANGES  ***