3GPP TSG-SA3 Meeting #115AdHoc-e	draft_S3-241228-r1

Electronic meeting, online, 15 - 19 April 2024

- Source:	China Mobile

- Title:	New solution of onboard UDM

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

It is proposed to approve this new solution in TR 33.700-29.

# 2	References

[1]	3GPP TR 23.700-29: "Study on integration of satellite components in the 5G architecture; Phase 3".

# 3	Rationale

To ensure authentication procedure completion, the UDM could be onboard to enable S&F operation.

# 4	Detailed proposal

*** Start of 1st Change****

## 6.Y	Solution #Y: Onboard UDM

### 6.Y.1	Introduction

This solution addresses key issue #1.

During the feeder link’s intermittent unavailability, in order for the completion of authentication and NAS security procedures, UDM/AUSF could be onboard

6.Y.2	Solution details

The following assumptions and principles apply:

-	The gNB, AMF and UDM/AUSF are placed on board the same satellite.

-	The UE has a subscription and credentials in the onboard UDM.

-     The onboard UDM is synchronized with the UDM on the ground when feeder link is available.

UE

SMF

UPF

AMF

RAN

UPF

DN

UDM/AUSF

When the gNB, AMF and UDM/AUSF are all on board, the authentication and NAS security procedures follow what defined in TS 33.501[4]. The differences are:

-	After the on-board registration completion, the on-board UDM may send the authentication result to the ground UDM for backup.

Editor’s Note: It is FFS whether the solution can support roaming scenarios

### 6.Y.3	Evaluation

TBD

**** End of 1st Change****