3GPP TSG-SA3 Meeting #115e 	S3-241240

- E-Meeting, 15 - 19 April 2024                     	      				(revision of S3-yyxxxx)

- Source:	China Mobile

- Title:	Add new key issue on UE access control to TR 33.745

- Document for:	Approval

Agenda Item:	5.17

# 1	Decision/action requested

This contribution proposes to add new key issue to TR 33.745.

# 2	References

[1]	3GPP TR 33.745: "Study on Security aspects of 5G NR Femto."

# 3	Rationale

TS 33.320 specified Access Control Mechanisms for H(e)NB with CSG/non-CSG method. However, based on architecture assumption in SA2, the CAG concept defined for PNI-NPN will be re-used for Femto access control. The contribution proposes to investigate any needed enhancements of UE access control to support the UE moving between CAG cell of 5G Femto and CSG cell.

# 4	Detailed proposal

It is suggested to approve the following change.

*************** Start of the change ***************

## 5.X	Key Issue #X: UE access control

### 5.X.1	Key issue details

SA2’s architecture assumes that the existing CAG concept defined for PNI-NPN is re-used for Femto access control. This key issue investigates any needed enhancements of UE access control to support the UE moving between CAG cell of 5G Femto and CSG cell. The core network shall be able to check that only mobile users in the relevant CAG/CSG can access services via a specific Femto NB.

### 5.X.2	Security threats

If a rogue UE accesses to an 5G Femto gNB with a given CAG ID, to which it does not belong to, the following types of attacks could potentially occur:

-  The Femto owner might end-up paying the charges for the rogue user

### 5.X.3	Potential security requirements

UE access control or membership verification mechanism shall be supported, access control should be performed when the UE moves between CAG cell of 5G Femto and CSG cell with a given CSG ID/CAG ID.

*************** End of the change ****************