3GPP TSG-SA3 Meeting #115e 	S3-241246-r1

- e-meeting, 15 - 19 April 2024                                                          revision of S3-2xxxx

- Source:	CMCC

- Title:	New solution for key issue 3

- Document for:	Approval

Agenda Item:	5.3

# 1	Decision/action requested

It is proposed to approve the proposed changes for inclusion in TR 33.757 [1].

# 2	References

[1]	3GPP TR 33.757: Study of security for PLMN hosting a NPN

# 3	Rationale

This contribution proposes a solution for key issue 3. It’s proposed to use trusted environment mechanism to secure the sensitive data stored in the NF in dedicated network.

# 4	Detailed proposal

It is proposed to approve the following changes for inclusion in TR 33.757 [1].

**** Start of Changes****

## 6.X	Solution #X: Secure sensitive data with trusted environment

### 6.X.1	Introduction

The solution addresses key issue #3. It is proposed that an Trusted Environment is used for the execution of sensitive functions and the storage of sensitive data in the NFs deployed in customer premise.

### X.2	Solution details

A Trusted Environment (TrE) is a logical entity which provides a trustworthy environment for the execution of sensitive functions and the storage of sensitive data. All data produced through execution of functions within the TrE is unknowable to unauthorized external entities, which protects data it holds from unauthorized access and tampering.

The TrE is built from an irremovable, HW-based root of trust by way of a secure boot process, which occurs whenever an NF in customer premises is turned on or goes through a hard reset. The root of trust is physically bound to the NF. The secure boot process includes checks of the integrity of the TrE performed by the root of trust. Only successfully verified components shall be loaded or started. The TrE, after having been successfully started, shall proceed to verify other components of the hosting NF (e.g. operating system and further programs) that are necessary for trusted operation of  the NF.

The TrE is used to provide the following protections to secure the sensitive data in customer premise:

-  Sensitive data such as SUPI and security context in UE context should be stored in the TrE of the NF in customer premise.

-  Sensitive functions such as key derivation functions should be performed within TrE.

-  All signaling messages should be confidentiality, integrity and replay protected while being transmitted in the customer premise.

-  The following attributes should additionally be confidentiality protected when being sent between the NF in dedicated network and 5GC:

- SUPI.

In summary, this solution proposes that the customer has appropriate physical and other security measures in place to protect the SUPI.

### 6.X.3	Evaluation

Editor's note: evaluation is ffs

**** End of Changes****