3GPP TSG-SA3 Meeting #115AdHoc-e	draft_S3-241405-r3

Electronic meeting, online, 15 - 19 April 2024

- Source:	Samsung

- Title:	Solution to prevent GERAN/UTRAN bidding down attack

- Document for:	Approval

Agenda Item:	5.6

# 1	Decision/action requested

It is proposed to approve this solution for addressing key issue#1 on GERAN/UTRAN bidding down attack.

# 2	References

NA

# 3	Rationale

This contribution proposes two methods to notify UE about the decommissioned GERAN/UTRAN RAT types.

- The network notifies the UE during attach/registration procedure.

- UE is configured by the network as part of pre-configuration in the UICC.

# 4	Detailed proposal

*** Start of Changes ***

## 6.Y	Solution #Y: Solution to prevent GERAN/UTRAN bidding down attack using UICC Configuration

### 6.Y.1	Introduction

This solution addresses the security requirement of key issue#1.

### 6.Y.2	Solution details

If the HPLMN completed the decommissioning or network supports LTE and/or NR only, then

the UICC is pre-configured with the information on restricted RAT types (i.e., GERAN/UTRAN) for the entire HPLMN. The UE excludes the combination HPLMN and restricted RAT types of the HPLMN for any PLMN/Cell (re)selection procedures.

### 6.Y.3	Evaluation

This solution addresses key issue#1 to prevent UE from bidding down attack from fake GERAN/UTRAN.

Impacts:

- The solution has impact on the UE to store and use the preconfigured information on restricted RAT types in the HPLMN.

- This solution does not address the problem for legacy devices

- The UICC configuration based solution is only limited to the non-roaming scenarios.

*** Next Change ***

## 6.Z	Solution #Z: Provisioning of information on restricted RAT types using NAS message

### 6.Z.1	Introduction

This solution addresses the security requirement of key issue#1. As the decommissioning can be performed in a phased manner by the MNO, this solution details a mechanism to inform the UE whether in that particular region (Tracking Area (TA)) network supports GERAN and/or UTRAN.

### 6.Z.2	Solution details

In this solution,

the network notifies the UE during Attach/Registration procedure (for the TAI/list of TAIs included in the Attach/Registration accept message or for the entire PLMN).

Figure 6.Z.2-1: UE is notified about the restricted RAT types (GERAN/UTRAN) in Attach/Registration accept message

During attach or registration procedure the network indicates to the UE about the information on restricted RAT types (i.e., GERAN/UTRAN) in attach or registration accept message for the TAI or included list of TAIs or for the entire PLMN, as shown in Figure 6.Z.2-1. The restricted RAT types are excluded in any follow-up procedures by the UE.

### 6.Z.3	Evaluation

This solution addresses key issue#1 to prevent UE from bidding down attack from fake GERAN/UTRAN.

Impacts:

- The solution has impact on the UE and the AMF/MME

- .

- This solution does not address the problem for legacy devices

- This solution works even in roaming scenarios.

- This solution uses TA for information on decommissioned RATs, when the network performs decommissioning in a phased manner.

*** End of Change ***