3GPP TSG-SA3 Meeting #115AdHoc-e	S3-241407-r2

Electronic meeting, online, 15 - 19 April 2024

- Source:	Samsung

- Title:	New solution on preventing DoS attacks in S&F operation

- Document for:	Approval

Agenda Item:	5.7

# 1	Decision/action requested

This pCR proposes a new solution on KI#1 in TR 33.700-29.

# 2	References

NA

# 3	Rationale

This contribution proposes a new solution on KI#1.

# 4	Detailed proposal

*************** Start of 1st Change ****************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.700-29: "Study on integration of satellite components in the 5G architecture; Phase 3".

[3]	3GPP TS 33.401: "3GPP System Architecture Evolution (SAE); Security Architecture".

[4]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[5]	3GPP TS 33.328: "IP Multimedia Subsystem (IMS) media plane security".

[6]	3GPP TS 33.210: "Network Domain Security: IP network layer security".

[xx]	IETF RFC 6507: "Elliptic Curve-Based Certificateless Signatures for Identity-Based Encryption (ECCSI)".

[yy]	3GPP TS 33.102: "3G security; Security architecture".

*************** Start of 2nd Change ****************

## 6.X	Solution #X: Solution on preventing DoS attacks in S&F operation

### 6.X.1	Introduction

This solution addresses key issue #1: Security protection in Store and Forward Satellite Operation.

When only service link is available, a satellite may receive a number of registration request messages from UEs which are not authenticated yet. Unless there is a means for the satellite to verify whether the messages are from valid UEs or not, the satellite would store and forward all of the received messages. It may lead to a DoS (denial of service) attack on the satellite.

In addition, after a valid UE sends registration request message, the UE will wait till the service link is available again for authentication. However, if the satellite is a false satellite, the UE may not be able to connect to core network for a long time. It may lead to a DoS attack on the UE. The DoS attacks described above may happen more easily than a ground base station case, due to the Store and Forward property.

In this solution, before the primary authentication is performed between the UE and core network, ECCSI (elliptic curve-based certificateless signatures for identity-based encryption) as defined in IETF RFC 6507 [xx] is used for mutual authentication between UE and satellite, in order to prevent the DoS attacks. In this solution, it is assumed that a satellite includes eNB and the functionality of MME related to the authentication called MME (NT).

### 6.X.2	Solution details

Figure 6.X.2.1-1 Authentication process of S&F mode

To prevent DoS attacks, the following procedure is preceded:

- The network provisions UEs and Satellite with a set of credentials for ECCSI. The credentials include Public Validation Token (PVT), Secret Signing Key (SSK) associated with UE's or Satellite's identity, and KMS Public Authentication Key (KPAK).

NOTE:	KMS can be a standalone entity or collocared in an existing NF.

After that, the authentication process of S&F mode is as follows.

PHASE 1. (Service link is available, Feeder link is unavailable)

- The satellite provides a random number generated by the satellite (SAT.RN) and S&F indicator indicating that the satellite is operating in S&F mode. These may be included in the SIB (System Information Broadcast) message

- The UE initiates the attach procedure by transmitting Attach Request. This message includes UE.ID, UE.Sig, UE.RN, and S&F indicator in addition to existing parameters. UE.ID is different from SUPI and it is used for generating a digital signature (UE.Sig), which is generated through ECCSI algorithm using UE.ID and Attach request message. S&F indicator indicates that the UE will operate in S&F mode.

The satellite checks the validity of the UE by verifying the UE.Sig.

- If the verification is successful, the satellite stores the received attach request message and transmits SAT.ID, SAT.Sig, and Re-attach Info to the UE. SAT.ID is the identity of the satellite and it is used to generate a digital signature (SAT.Sig), which is generated through ECCSI algorithm using SAT.ID and UE.RN. The Re-attach Info is information necessary for the UE to attempt the reconnection in step 7 (e.g., information about when the UE should retry to reconnect or list of satellite(s) that the UE should retry to reconnect).

The UE checks the validity of the satellite by verifying the SAT.Sig. If verified, UE waits for step 7 based on the guideline received.

PHASE 2. (Service link is unavailable, Feeder link is available)

- The satellite requests authentication data for the UE by sending the Authentication Data Request message to the ground network (GND). The request includes IMSI, SN identity, and Network type.

- Upon the receipt of the Authentication Data Request from the satellite, HSS in the GND generates Authentication Vector(s) as defined in clause 6.3.2 in TS 33.102 [yy]. Authentication Vector consists of KASME, RAND, AUTN, and XRES.

- The GND sends an authentication response back to the satellite that contains Authentication Vector(s).

PHASE 3. (Service link is available, Feeder link is unavailable)

The satellite in this PHASE may be different from the satellite in PHASE 1.

- The UE retries the network connection by transmitting the Attach Request.

- The satellite sends Authentication Request including AUTN and RAND.

- At the receipt of the RAND and AUTN, the UE verifies the freshness of the received values by checking whether AUTN can be accepted as described in TS 33.102 [yy]. If so, the UE computes a response RES.

- UE responds with Authentication Response message including RES in case of successful AUTN verification. In this case, the UE computes KASME from CK, IK, and serving network's identity.

- The satellite checks that the RES equals XRES. If so, the authentication is successful. As a result of the authentication and key agreement, an intermediate key KASME is shared between the UE and the satellite.

- NAS SMC procedure is performed between the UE and the satellite.

### 6.X.3	Evaluation

Editor's Note:	Regarding generation and verification of signatures, the expected impact on satellite system resources is FFS.

*************** End of Change ****************