3GPP TSG-SA3 Meeting #115AdHoc-e	draft_S3-241424-r1

Electronic meeting, online, 15 - 19 April 2024

- Source:	Ericsson, Lenovo, MITRE Corporation, OTD_US, Johns Hopkins APL, Motorola Mobility

- Title:	Adding Key Issue on WT2 "Security mechanism for dynamic policy enforcement"

- Document for:	Approval

Agenda Item:	5.1

# 1	Decision/action requested

Approve the pCR to TR 33.794 [1] below.

# 2	References

[1]	3GPP TR 33.794 "Study on enablers for Zero Trust Security"

# 3	Rationale

WT2 "Security mechanism for dynamic policy enforcement" does not have a key issue yet. This contribution proposes to add a key issue that allows SA3 to address the work task.

# 4	Detailed proposal

*** BEGIN CHANGES ***

## 6.X	Key Issue #X: Security mechanisms for policy enforcement at the 5G SBA

### 6.X.1	Key issue details

Security evaluation and monitoring can lead to the identification of a potential attack in a 5G network. After the immediate actions on the infrastructure layers such as shutting down relevant virtual machines or containers, long-term actions on the 5G SBA are necessary such as updating the NF profiles related to NFs that were affected by the attack.

### 6.X.2	Security threats

If the NRF is not updated with information about an NF that has been subject to an attack and mitigations are only performed at infrastructure layers, an attacker could reuse information gained during the attack for extending or re-newing the attack.

### 6.X.3	Potential security requirements

The 5GS should provide the means to update the NRF with information about an NF that has been subject to an attack.

### NOTE z:	Solutions to this key issue describe updates of the NRF configuration that are necessary after the identification of an attack, including the following use case:

### Update of NF profiles at the NRF

NOTE x: 	The policy decision point (PDP) needs to take into account information from layers outside the 3GPP scope and is subject to operators' overall operational security policies, and is hence outside of 3GPP scope.

NOTE y: 	Existing mechanisms in TS 33.501 [4] and other 3GPP specifications for updating NF profiles need to be taken into account.

*** END CHANGES ***