- 3GPP TSG-SA3 (Security)	S3-100786

- SA3#60, 28 June – 02 July 2010, Montreal, Canada	revision of S3-xxxxxx

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.320 | CR | 0037 |  rev | - |  Current version: | 9.2.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Corrections to the reference to OMA WAP OCSP profile |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia Corporation, Nokia Siemens Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | EHNBSec |  |  |  |  |  |  | Date:  |  | 13/06/2010 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | Currently the specification references the specification “Open Mobile Alliance OMA-WAP-OCSP V1.0”. This spec gives a profiling of OCSP usage. It was written in 2004, and thus represents the status of that time with respect to cryptographic strength of algorithms. As other 3GPP specifications are updated to this respect, also the reference to OMA has to be profiled to take care of these updates. Also the OMA spec requires support of WAP-specific protocols, which is not needed in the H(e)NB environment. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | On all occurrences of references to OMA OCSP, profiling according to TS33.310 is added, and support of WAP-specific protocols is not required. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Inconsistency of the specification, as OMA OCSP profile specifies other algorithms than used in e.g. the referenced 3GPP specification TS 33.310. In addition, support of WAP-specific protocols is mandated in OMA spec, which is not necessary for H(e)NB deployment. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 3.2, 7.2.2, 8.2.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

********************** start of 1st change **************************

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AAA	Authentication, Authorization, and Accounting

ACS	Auto-Configuration Server

AKA	Authentication and Key Agreement

CA	Certification Authority

CPE	Customer Premises Equipment

CSG	Closed Subscriber Group

DPD	Dead Peer Detection

eNB	Evolved Node-B

EAP	Extensible Authentication Protocol

ESP	Encapsulating Security Payload

E-UTRAN	Evolved UTRAN

FQDN	Fully Qualified Domain Name

GNSS	Global Navigation Satellite System

H(e)NB	Home NodeB or Home eNodeB

H(e)NB-GW	Home (e)NodeB Gateway

H(e)MS	Home NodeB Management or Home eNodeB Management System

HeMS	Home eNodeB Management System

HeNB	Home eNodeB

HMS	Home NodeB Management System

HNB	Home NodeB

HP	Hosting Party

HPM	HP Module

IKE	Internet Key Exchange

IMSI	International Mobile Subscriber Identity

LTE	Long Term Evolution

MME	Mobility Management Entity

MSK	Master Session Key

NAT	Network Address Translation

PKI	Public Key Infrastructure

SA	Security Association

SeGW	Security Gateway

TLS	Transport Layer Security

TrE	Trusted Environment

UICC	Universal Integrated Circuit Card

URI	Uniform Resource Identifier

URL	Uniform Resource Locator

USIM 	Universal Subscriber Identity Module

UMTS	Universal Mobile Telecommunications System

UTRAN	Universal Terrestrial Radio Access Network

WAP	Wireless Application Protocol

********************** start of next change **************************

### 7.2.2	SeGW and Device Mutual Authentication Procedure

Device authentication shall be performed using IKEv2 with public key signature based authentication with certificates, as specified in RFC 4306 [4]. The H(e)NB device shall authenticate itself to the SeGW with a certificate based on the globally unique and permanent H(e)NB identity, signed by an operator authorized entity. The SeGW shall authenticate itself to the H(e)NB using a certificate signed by an operator trusted CA. The H(e)NB shall verify the SeGW identity by checking the subjectAltName field of the SeGW certificate against the name of the SeGW used by the H(e)NB to connect to the SeGW.

NOTE 1:	 If DNS is available, the SeGW’s name is the FQDN used to resolve its IP address; otherwise it is the IP address of the SeGW.

The H(e)NB may check the revocation status of the SeGW certificate using OCSP as specified in [22] and [23] with the exception that the SHA-1 and SHA-256 hash functions shall be mandatory to support. For security reasons, the use of SHA-1 is not recommended for newly created OCSP responses. If the profile in [23] is used, support for WAP specific protocols is not required, and the provisions on certificates and signatures given in TS 33.310 [7], clause 6.1, shall be followed.

NOTE 1a:	It is likely that in a future 3GPP release, OCSP responses which use SHA-1 as the hash algorithm will be prohibited.

Support for OCSP is optional for the operator network. The H(e)NB should support OCSP.

NOTE 2:	It is strongly recommended to support OCSP in the H(e)NB, as this feature may become mandatory for H(e)NB in future releases.

The OCSP communication between H(e)NB and OCSP server may use the in-band signaling of certificate revocation status in IKEv2 according to RFC 4806 [24], through which the SeGW can include an OCSP response within IKEv2. Support for this extension to IKEv2 in H(e)NB and SeGW is optional.

The SeGW may check the revocation status of the H(e)NB certificate using CRLs according to TS 33.310 [7] or OCSP as specified in [22] and [23] with the exception that the SHA-1 and SHA-256 hash functions shall be mandatory to support. For security reasons, the use of SHA-1 is not recommended for newly created CRLs and OCSP responses. If the profile in [23] is used, support for WAP specific protocols is not required, and the provisions on certificates and signatures given in TS 33.310 [7], clause 6.1, shall be followed.

NOTE 2a:	It is likely that in a future 3GPP release, CRLs and OCSP responses which use SHA-1 as the hash algorithm will be prohibited.

The SeGW shall implement support for either CRL checking or OCSP or both.  The locations of the CRL Server and OCSP Responder may be in the operator's network or provided by the manufacturer/vendor.  Neither the operator nor the manufacturer is required to provide a CRL Server or an OCSP Responder.  For the case when the operator provides a CRL Server or OCSP Responder, the manufacturer shall forward revocation data to the operator.  The interface to forward revocation data is out of scope of the present document.

If the H(e)NB certificate contains CRL or OCSP server information (cf. sub-clause 7.2.5.2), then the SeGW may contact this server for revocation information.

NOTE 3:	A CRL or OCSP server located at manufacturer of H(e)NB allows distribution of revocation information by the manufacturer directly. To use such revocation information, normally the SeGW needs a CRL or OCSP client capable to reach the public Internet to contact these servers.

Validity check of H(e)NB certificates in SeGW shall be configurable by the operator, i.e. whether to use CRLs, OCSP or both and whether to use operator CRL or OCSP server, manufacturer CRL or OCSP server, or more than one of them.

The H(e)NB’s TrE shall be used to provide the following critical security functions supporting the IKEv2 and certificate processes:.

-	The H(e)NB’s identity shall be stored in the TrE and shall not be modifiable.

-	The H(e)NB’s private key shall be stored in the TrE and shall not be exposed outside of the TrE.

-	The root certificate used to verify the signatures on the SeGW certificate shall be stored in the H(e)NB’s TrE and shall be writable by authorized access only. The verification process for signatures shall be performed by the H(e)NB’s TrE.

-	The H(e)NB’s TrE shall be used to compute the AUTH payload used during the IKE_AUTH request message exchanges.

NOTE 4: Autonomous validation is performed during secure start-up and performs validation of the H(e)NB. As IKEv2 allows the inclusion of information data into Notify Payload, information regarding the trustworthy state of the H(e)NB may be carried in the Notify Payload (see Annex A.1) during IKEv2 procedures from the H(e)NB to the SeGW.Notify Payload within IKEv2's IKE_AUTH message is protected by IKEv2 SK and AUTH.  In addition, the Notify Payload, as constructed by the TrE, should include a nonce and should be cryptographically signed by the TrE.

********************** start of next change **************************

#### 8.3.2.1	General

In case that the H(e)MS is accessible on the public Internet, the H(e)MS is exposed to attackers located in insecure network. H(e)MS traffic shall be protected by TLS tunnel established between H(e)NB and H(e)MS. In this case, mutual authentication between H(e)NB and H(e)MS shall be based on device certificate for the H(e)NB and network certificate for the H(e)MS. The H(e)NB shall verify the H(e)MS identity by checking the subjectAltName field of the H(e)MS certificate against the name of the H(e)MS.

NOTE 1:	If DNS is available, the H(e)MS’s name is the FQDN used to resolve its IP address; otherwise it is the IP address of the H(e)MS.

The H(e)NB may check the revocation status of the H(e)MS certificate using OCSP as specified in [22] and [23] with the exception that the SHA-1 and SHA-256 hash functions shall be mandatory to support. For security reasons, the use of SHA-1 is not recommended for newly created OCSP responses. If the profile in [23] is used, support for WAP specific protocols is not required, and the provisions on certificates and signatures given in TS 33.310 [7], clause 6.1, shall be followed.

NOTE 1a:	It is likely that in a future 3GPP release, OCSP responses which use SHA-1 as the hash algorithm will be prohibited.

Support for OCSP is optional for the operator network. The H(e)NB should support OCSP.

NOTE 2:	It is strongly recommended to support OCSP in the H(e)NB, as this feature may become mandatory for H(e)NB in future releases.

The OCSP communication between H(e)NB and OCSP server may use the in-band signaling of certificate revocation status in TLS according to RFC 4366 [25]. Support for this extension to TLS in H(e)NB and H(e)MS is optional.

The H(e)MS may check the revocation status of the H(e)NB certificate using CRLs according to TS 33.310 [7] or OCSP as specified in [22] and [23] with the exception that the SHA-1 and SHA-256 hash functions shall be mandatory to support. For security reasons, the use of SHA-1 is not recommended for newly created CRLs and OCSP responses. If the profile in [23] is used, support for WAP specific protocols is not required, and the provisions on certificates and signatures given in TS 33.310 [7], clause 6.1, shall be followed.

NOTE 2a:	It is likely that in a future 3GPP release, CRLs and OCSP responses which use SHA-1 as the hash algorithm will be prohibited.

The H(e)MS shall implement support for either CRL checking or OCSP or both.  The locations of the CRL Server and OCSP Responder may be in the operator's network or provided by the manufacturer/vendor.  Neither the operator nor the manufacturer is required to provide a CRL Server or an OCSP Responder.  For the case when the operator provides a CRL Server or OCSP Responder, the manufacturer shall forward revocation data to the operator.  The interface to forward revocation data is out of scope of the present document.

If the H(e)NB certificate contains CRL or OCSP server information (cf. sub-clause 8.3.3.1), then the H(e)MS may contact this server for revocation information.

NOTE 3:	A CRL or OCSP server located at manufacturer of H(e)NB allows distribution of revocation information by the manufacturer directly. To use such revocation information, normally the H(e)MS needs a CRL or OCSP client capable to reach the public Internet to contact these servers.

Validity check of H(e)NB certificates in H(e)MS shall be configurable by the operator, i.e. whether to use CRLs, OCSP or both and whether to use operator CRL or OCSP server, manufacturer CRL or OCSP server, or more than one of them.

*********************** end of changes ***************************