3GPP TSG SA WG3 Security - S3#60	 S3-100905

- Montreal, Canada 28 June– 2 July 2010 				   revision of S3-10xxxx

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.320 | CR | 0042 |  rev | - |  Current version: | 9.2.1 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Security aspects in H(e)NB mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | NEC Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | SA3 |  |  |  |  |  |  |  |  |  |
| Work item code:  | EHNB-Sec |  |  |  |  |  |  | Date:  |  | 20/06/2010 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | The current security aspects do not cover security in mobility from and to H(e)NB. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | It is shown in S3-100733 that mobility work is already done in RAN groups therefore no further work is needed in SA3. Therefore we add references of RAN documents and add a new section on security aspects of H(e)NB mobility. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Security for mobility is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

* * * * 1st change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: “Vocabulary for 3GPP Specifications”.

[2]	3GPP TS 32.583: “Telecommunications management; Home Node B (HNB) Operations, Administration, Maintenance and Provisioning (OAM&P); Procedure flows for Type 1 interface HNB to HNB Management System (HMS)”.

[3]	IETF RFC 4187: “Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA)”.

[4]	IETF RFC 4306: “Internet Key Exchange (IKEv2) Protocol”.

[5]	IETF RFC 4301: “Security Architecture for the Internet Protocol”.

[6]  	IETF RFC 4739: “Multiple Authentication Exchanges in the Internet  Key Exchange (IKEv2 Protocol”, Nov 2006.

[7]	3GPP TS 33.310: “Network Domain Security (NDS); Authentication Framework (AF)”.

[8]  	3GPP TS 23.003: “Numbering, addressing and identification”.

[9]	3GPP TS 33.210: “3G security; Network Domain Security (NDS); IP network layer security”.

[10] 	3GPP TS 33.234: “3G security; Wireless Local Area Network (WLAN) interworking security”.

[11]	3GPP TS 32.593: “Telecommunication management; Procedure flows for Type 1 interface H(e)NB to H(e)NB Management System (H(e)MS)”.

[12] 	3GPP TS 25.467: “UTRAN architecture for 3G Home Node B (HNB); Stage 2”.

[13]	3GPP TS 24.008: “Mobile radio interface Layer 3 specification; Core network protocols; Stage 3”.

[14]	3GPP TS 24.301: “Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3”.

[15]	The Broadband Forum TR-069: “CPE WAN Management Protocol v1.1”, Issue 1 Amendment 2, December 2007.

[16]	IETF RFC 4346:  “The Transport Layer Security (TLS) Protocol Version 1.1”.

[17]	IETF RFC 5246:  “The Transport Layer Security (TLS) Protocol Version 1.2”.

[18]	ETSI ES 282 004 (V1.1.1): Telecommunications and Internet Converged Services and Protocols for Advanced Networking (TISPAN); NGN functional architecture; Network Attachment Sub-System (NASS), 2006.

[19]	ETSI ES 283 035 (V1.1.1): "Telecommunications and Internet Converged Services and Protocols for Advanced Networking (TISPAN); Network Attachment Sub-System (NASS); e2 interface based on the DIAMETER protocol",,2006.

[20]	3GPP TS 33.102: “3G security; Security architecture”

[21]	3GPP TS 33.401: “3GPP System Architecture Evolution (SAE): Security architecture”

[22]	IETF RFC 2560: “X.509 Internet Public Key Infrastructure Online Certificate Status Protocol – OCSP”

[23]	Open Mobile Alliance OMA-WAP-OCSP V1.0: "Online Certificate Status Protocol Mobile Profile". URL: http://www.openmobilealliance.org/

[24]	IETF RFC 4806: “Online Certificate Status Protocol (OCSP) Extensions to IKEv2”

[25]	IETF RFC 4366: “Transport Layer Security (TLS) Extensions”

[26]	IETF RFC 5280:” Internet X.509 Public Key Infrastructure Certificate and Certificate Revocation List (CRL) Profile”.

[27]	3GPP TS 36.300: “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description”.

[yy]	3GPP TS 25.367: “Mobility procedures for Home Node B (HNB); Overall description; Stage 2.”.

* * * * 2nd change * * * *

# 10	Security Aspects for Mobility

## 10.1	Inbound mobility

In mobility from macro NB or HNB towards HNB CSG/hybrid, access control or membership verification for CSG capable UE and non CSG capable UE shall be performed as described in [12].

.Key management is done in core network as described in [20].

Access control or membership verification for UE in mobility from macro eNB or HeNB towards HeNB CSG/hybrid is handled in mobility scenario in [27].

Key handling shall follow that in [21].

## 10.2	Outbound mobility

For mobility from HNB, the normal procedure can be applied as stated in [yy]. Key handling shall follow the macro mobility procedure.

For mobility from HeNB, the normal procedure can be applied as stated in [27]. Key handling shall follow the macro mobility procedure.

* * * * End of change * * * *