# 3GPP TSG SA WG3 (Security) Meeting #86Bis	S3-170714

# 27 – 31 March, 2017, Busan, Korea	revision of S3-17xabc

# Source:	Huawei, Hisilicon

# Title:	Questions related to Key Issue #8.3: Security on UEs’ access to slices

# Document for:	Discussion and Approval

Agenda Item:	5.1.8 Network Slicing Security

# 1	Decision/action requested

This contribution adds questions and interim agreements for Issue #8.3: Security on UEs’ access to slices

# 2	References

[1] 3GPP TR 33.899v0.7.0 (2017-2) Study on the security aspects of the next generation system

# 3	Rationale

This contribution proposes a list of questions that may lead to interim agreements, addressing key issue #8.3: Security on UEs’ access to slices, for TR33.899.

# 4	Detailed proposal

It is proposed to approve below pCR. Since all texts are new, revision mark is not used.

**********************Begin of changes********************************

### E.8.2	Questions and Interim Agreements for Security on UEs’ access to slices

#### E.8.2.3   UE accesss to slices

##### E.8.2.3.1 Description of Questions

As observed by 3GPP TR 22.864, services offered by the network slice could either belong to the MNO or a 3rd party service provider. When the slice is allocated to a 3rd party service provider, the ability of the provider to verify and ascertain the identifier of the subscription, trying to gain access to the slice, becomes a critical security component to be addressed.

For different application scenarios, the access to the slices may be different. Hopefully, the answers to the following questions are helpful for reaching agreement on this key issue.

Question 1: Is it allowed to have an optional slice authentication (in addition to MNO authentication before UE can access the slice?

Question 2: Is it allowed to use different authentication methods for MNO authentication and slice authentication?

Question 3: Is it allowed to have mandatory MNO authorization   and mandatory slice authentication in order for the UE to access the slice?

##### E.8.1.1.2 Interim Agreement

(The following agreement is related to the above three questions in order)

1. UE may need to have additional authentication with slice before UE can access the slice, in additional to authentication between UE and Network.

2. The authentication method used between UE and Network and the authentication method between UE and slices may be different.

3. A UE shall be authenticated by slice before accessing the slice. In addition, the UE shall be authorized by MNO

*********************************End of changes*******************************